| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0120-v1 |
| --- | --- |

| Title: | AHG17: On Access Unit Delimiter and Picture Header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Ye-Kui Wang Jianle Chen San Diego, CA, USA | Tel: Email: | - fhendry@futurewei.com ye-kui.wang@futurewei.com jianle.chen@futurewei.com |
| Source: | Futurewei Technologies Inc. |  |  |

_____________________________

# Abstract

In this contribution, the following items are proposed:

- Remove access unit delimiter (AUD) and define picture header which is mandated to be present in the beginning of each picture / layer access unit.

- Move signalling of picture level syntax elements in slice header to picture header. Picture level syntax elements in slice header refer to syntax elements that are signalled in slice header that are required to have the same values for all slices that belong to the same picture. These syntax elements include: slice_pic_parameter_set_id, non_reference_picture_flag, colour_plane_id, slice_pic_order_cnt_lsb, recovery_poc_cnt, no_output_of_prior_pics_flag, pic_output_flag, slice_temporal_mvp_enabled_flag.

- Signal non-picture level syntax elements in slice header that are typically the same for all slices of the same picture in picture header. Non-picture level syntax elements in slice header refer to syntax elements that are signalled in slice header and are not required to have the same values for all slices that belong to the same picture. When those syntax elements are not present in picture header, they may be signalled in slice header.

# Introduction

In the 15th JVET meeting in Gothenburg, it was agreed to mandate the presence of access unit delimiter (AUD) for each access unit (AU). By mandating the presence of AUD, it would be easier for decoder to detect picture change (i.e., knowing if a VCL NAL unit contains the first slice of a picture) without having to parse information from slice header, or any parameter set.

Having a AUD present in each AU may allow us to signal picture level information in AUD. Currently there are syntax elements that are signalled in slice header with constraint such that their values shall be the same for all slices of the same picture as pasted below from general slice header semantics. We refer these syntax elements as picture level syntax elements.

When present, the value of each of the slice header syntax elements slice_pic_parameter_set_id, non_reference_picture_flag, colour_plane_id, slice_pic_order_cnt_lsb, recovery_poc_cnt, no_output_of_prior_pics_flag, pic_output_flag, and slice_temporal_mvp_enabled_flag shall be the same in all slice headers of a coded picture.

It would be more efficient if picture level syntax elements are signalled in AUD or other non-VCL NAL unit that is present for each picture, rather than repeating them in each slice even when their values are the same for all slices of the same picture.

Furthermore, there are some syntax elements signalled in slice header that are not picture level syntax elements (i.e., their values are not required to be the same for all slices of the same picture) but are typically set the same by encoders in common use-cases. Such non-picture level syntax element may include the following (not an exhaustive list):

- Syntax elements related to signalling of reference picture lists.

- Syntax element related to signalling of joing cb/cr sign flag

- Syntax elements related to signalling of SAO enabling.

- Syntax elements related to signalling of ALF enabling.

- Syntax elements related to signalling of LMCS enabling.

- Syntax elements related to signalling of scaling list enabling.

It would be beneficial if the above non-picture level syntax element may be signalled in AUD or other non-VCL NAL unit that is present for each picture when their values are the same for all slices of the same picture. Only when the values of those syntax elements are different for slices of the same picture they need to be signalled in slice header.

# Problem description

Signalling picture level syntax elements and some non-picture level syntax elements in AUD has at least two problems:

- Only one AUD is supposed to be present in an AU. In the multi layer bitstream, an AU may contain more than one picture. Although it is possible to signal picture level syntax elements of all pictures of an AU in the associated AUD, it may not be a good idea because in the case of bitstream extraction it will require extra work for removing syntax elements of pictures that are removed during the process.

- AUD NAL unit is specified to be the first NAL unit in an AU. Consequently, syntax elements signalled in an AUD cannot have dependency to other parameter sets since they may not be available when those syntax elements are parsed. This limitation make it not possible to signal even the picture level syntax elements in AUD. For example, if POC LSB information is signalled in AUD (i.e., slice_pic_order_cnt_lsb), parsing of the syntax element has dependency to SPS to determine how many bits are used for the signalling.

# Proposals

To overcome the problems above, we propose the following:

- Remove access unit delimiter (AUD) and define picture header contained in a new non-VCL NAL unit (e.g., NAL unit with type PH_NUT) and mandate the presence of a picture header in the beginning of each picture / layer access unit. Picture header is designed to have the following properties:

  - Temporal Id and layer Id of picture header NAL unit are the same as temporal Id and layer Id of layer access unit that contains the picture header.

  - Picture header NAL unit shall precede NAL unit containing the first slice of its associated picture. This established the association between a picture header and slices of picture associated with the picture header without the need of having picture header Id signalled in picture header and referred to from slice header.

  - Picture header NAL unit shall follow picture level parameter sets or higher level such as DPS, VPS, SPS, PPS, etc. This consequently requires those parameter sets to be not repeated / present within a picture or within an access unit.

  - Picture header contains information about picture type of its associated picture. The picture type may be used to define the following (not an exhaustive list)

    - The picture is an IDR picture

    - The picture is a CRA picture

    - The picture is a GDR picture

    - The picture is a non-IRAP, non-GDR picture and contains I-slices only

    - The picture is a non-IRAP, non-GDR picture and may contain P- and I-slices only

    - The picture is a non-IRAP, non-GDR picture and contains any of B-, P-, and/or I-slices

- Move signalling of picture level syntax elements in slice header to picture header.

- Signal non-picture level syntax elements in slice header that are typically the same for all slices of the same picture in picture header. When those syntax elements are not present in picture header, they may be signalled in slice header.

Text for proposed picture header is given in the attached spec text.

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).