| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0122-v1 |
| --- | --- |

| Title: | AHG17: On Adaptation Parameter Set |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Ye-Kui Wang Jianle Chen San Diego, CA, USA | Tel: Email: | - fhendry@futurewei.com ye-kui.wang@futurewei.com jianle.chen@futurewei.com |
| Source: | Futurewei Technologies Inc. |  |  |

_____________________________

# Abstract

It was asserted that the following constraints that are specified for ALF APS referencing from slice headers are problematic and do not work for supporting random access:

For intra slices and slices in an IRAP picture, slice_alf_aps_id_luma[ i ] shall not refer to an ALF APS associated with other pictures rather than the picture containing the intra slices or the IRAP picture.

For intra slices and slices in an IRAP picture, slice_alf_aps_id_chroma shall not refer to an ALF APS associated with other pictures rather than the picture containing the intra slices or the IRAP picture.

If the intent of the above constraints is indeed for ensuring support for random access feature, two options are proposed:

- Option 1: Remove the constraints from VVC Draft. It is asserted that APS can also be transmitted out-of-band which is similar to other parameter sets such as PPSs and there is no problem with PPS for enabling random access.

- Option 2: Modify the constraints as follows and apply similar constraints to other APS types as well:

For slices in an IRAP picture or a GDR picture and slices in pictures that are not RASL pictures and follow the IRAP picture or the GDR picture in decoding order, slice_alf_aps_id_luma[ i ] shall not refer to an ALF APS referred by slices of pictures that precede the IRAP picture or the GDR picture in decoding order.

For slices in an IRAP picture or a GDR picture and slices in pictures that are not RASL pictures and follow the IRAP picture or the GDR picture in decoding order, slice_alf_aps_id_chroma shall not refer to an ALF APS referred by slices of pictures that precede the IRAP picture or the GDR picture in decoding order.

# Problem description

In VVC Draft 6 specification, there are constraints specified for ALF as follows:

For intra slices and slices in an IRAP picture, slice_alf_aps_id_luma[ i ] shall not refer to an ALF APS associated with other pictures rather than the picture containing the intra slices or the IRAP picture.

For intra slices and slices in an IRAP picture, slice_alf_aps_id_chroma shall not refer to an ALF APS associated with other pictures rather than the picture containing the intra slices or the IRAP picture.

If the intent of the above constraints is to ensure support for random access, it is asserted that the constraints have at least the following problems:

- APS can be transmitted in-band or out-of-band like other parameter sets such as SPSs, PPSs, etc. When parameter sets are transmitted out-of-band (e.g., contained in sample entries or in separated parameter set track in ISO-based media file format), random access feature is not affected by reference to earlier parameter set. Due to this fact, the need of those constraints above is questionable.

- The constraints apply to any pictures that contain intra slices. This is redundant since it does not serve / help the purpose of supporting random access by refreshing ALF parameters in APS.

- The constraints apply to IRAP picture but not pictures that follow the IRAP in decoding order. By mandating ALF parameters to be refreshed for an IRAP picture but not necessary for pictures that following the IRAP picture in decoding order, it cannot be guaranteed that when random access from the IRAP picture, ALF parameters for pictures that follow the IRAP picture are available.

- The constraints do not consider RASL pictures. RASL pictures are picture that may not be decoded correctly when random access from an IRAP picture thus it should be fine for RASL pictures to use ALF parameters that may be not available when random access happens. By forcing ALF parameters to be refreshed for RASL pictures, it may degrade compression efficiency for RASL pictures.

- The constraints do not consider other type of pictures that may also be used to random access from. GDR picture, for example, can be used by decoder to random access from.

# Proposal

We believe that there is no need to disallow reference to parameter set that is present before picture that may be used as random access point, therefore, we propose that the above constraints are removed from VVC Draft 6.

However, if the JVET group decides that it is desirable to disallow reference to ALF APS that is present before picture that may be used as random access point, we propose that the constraints are modified as follows:

For slices in an IRAP picture or a GDR picture and slices in pictures that are not RASL pictures and follow the IRAP picture or the GDR picture in decoding order, slice_alf_aps_id_luma[ i ] shall not refer to an ALF APS referred by slices of pictures that precede the IRAP picture or the GDR picture in decoding order.

For slices in an IRAP picture or a GDR picture and slices in pictures that are not RASL pictures and follow the IRAP picture or the GDR picture in decoding order, slice_alf_aps_id_chroma shall not refer to an ALF APS referred by slices of pictures that precede the IRAP picture or the GDR picture in decoding order.

Furthermore, similar constraints should also be applied to other type of APSs as follows:

For slices in an IRAP picture or a GDR picture and slices in pictures that are not RASL pictures and follow the IRAP picture or the GDR picture in decoding order, slice_lmcs_aps_id shall not refer to an LMCS APS referred by slices of pictures that precede the IRAP picture or the GDR picture in decoding order.

For slices in an IRAP picture or a GDR picture and slices in pictures that are not RASL pictures and follow the IRAP picture or the GDR picture in decoding order, slice_scaling_list_aps_id shall not refer to an scaling list APS referred by slices of pictures that precede the IRAP picture or the GDR picture in decoding order.

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).