| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0124-v1 |
| --- | --- |

| Title: | AHG17: On mixed NAL unit types within a picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Hendry San Diego, CA, USA | Email: | ye-kui.wang@futurewei.com fhendry@futurewei.com |
| Source: | Futurewei Technologies Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to allow mixed IRAP and non-IRAP NAL units within a picture. It is asserted that such capability is useful for use-cases involving sub-bitstream merging from two bitstream with different IRAP periods. To enable such functionality, the following aspects are proposed:

- Add a flag in access unit delimiter (AUD) or in picture header, if one is specified, that specifies whether all VCL NAL units of a picture have the same NAL unit type.

- Add a constraint to require that, for any particular picture, either all VCL NAL units have the same NAL unit type, or some VCL NAL units have a particular IRAP NAL unit type and the rest have a particular non-IRAP VCL NAL unit type.

- Add another constraint to require that slices in pictures that follow a picture containing both IRAP and non-IRAP slices in decoding order and that cover the same picture regions of the IRAP slices in the picture do not refer to pictures earlier than the picture in decoding order for inter prediction reference.

# Introduction

At the 14th and the 15th JVET meetings, the idea of allowing mix of IRAP and non-IRAP NAL unit types within a picture had been proposed and discussed in which it was concluded from the discussion at the 15th JVET meeting that further study is needed.

The following is snippet of meeting notes from the 15th JVET meeting.

-----begin note snippet-----

Consider mixed NAL unit types within a picture based on JVET-O0140.

Further discussed Friday 12 July 0035 (GJS).

A motivation is region-wise random access using subpictures. An example use case is viewport dependent streaming of 360° video. Basically it would be from a BEAMer scenario.

It was commented that using layers would be an alternative way to provide this functionality while constraining to have only one NAL unit type per picture per layer.

There would be a level of indirection involved in identifying the random access properties of the picture from the NAL unit header.

It was commented that this indication could be put into the AUD.

Mandating AUD could potentially simplify a lot of basic needs.

It was also commented that something like an SEI message instead of NAL unit type can be used to indicate the predictive properties of the bitstream.

The encoding of the different regions would need to be very carefully coordinated.

Further study of this was needed.

------end note snippet------

Taking guides from the discussion points above and considering the other decisions made during the 15th JVET meeting, we have the following observations:

- Subpicture based independent coded region was agreed. With the availability of the subpicture feature, for sub-bitstream merging, it would be simpler to use subpictures than using layers. When merging sub-bitstreams using subpictures, the result would still be a single layer bitstream rather than a multi-layer bitstream which may add unnecessary complexity.

- Access unit delimiter (AUD) is mandated to be present for each access unit. With the presence of AUD for every access unit, as stated in the meeting note above, the indication of mixed IRAP and non-IRAP NAL unit within a picture can be signalled there.

- Regarding the meeting notes "It was also commented that something like an SEI message instead of NAL unit type can be used to indicate the predictive properties of the bitstream.",

Therefore, we propose to allow mixed IRAP and non-IRAP NAL unit types within a picture, by simply having a picture-level indication of whether there are mixed IRAP and non-IRAP NAL unit types within a picture.

# Proposal

We propose to add a flag in AUD, which when equal to 0 indicates that all VCL NAL units of the picture associated with the AUD have the same NAL unit type. Also, instead of adding the condition of the flag to lots of places in the specification text that are dependent on a picture being an IRAP picture, as an editorial improvement, it is proposed to update the definition of IRAP picture to additionally have the condition of this flag equal to 0. The flag may also be signalled in other picture level header such as picture header, for example.

We further propose that the support of mixed NAL unit types within a picture is specified in a limited fashion. To be more accurate, it is proposed that, for any particular picture, either all VCL NAL units have the same NAL unit type, or some VCL NAL units have a particular IRAP NAL unit type and the rest have a particular non-IRAP VCL NAL unit type. In other words, VCL NAL units of any particular picture cannot have more than one IRAP NAL unit type and cannot have more than one non-IRAP NAL unit type. The justification is that with this constraint, the use cases used as the basis for the adoption of allowing mixed NAL unit types at the previous JVET meetings are satisfied, while additional flexibility does not seem add any value while impose additional decoder implementation costs, at least for conformance testing.

Finally, to address a comment received from the initial discussion of JVET-O0140 at the HLS BoG at the previous JVET meeting, the meaning of an IRAP slice in a picture containing both IRAP and non-IRAP slices is clarified, by adding a constraint. This constraint requires that slices in pictures that follow such a picture in decoding order and that cover the same picture regions of the IRAP slices in the picture do not refer to pictures earlier than the picture in decoding order for inter prediction reference.

The proposed text changes are as follows:

### Definitions

intra random access point (IRAP) picture: A coded picture for which mixed_nalu_types_flag is equal to 0 and each VCL NAL unit has NalUnitType in the range of IDR_W_RADL to CRA_NUT, inclusive.

### Access unit delimiter RBSP syntax

| access_unit_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| pic_type | u(3) |
| mixed_nalu_types_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

### NAL unit header semantics

...

NOTE 4 – An instantaneous decoding refresh (IDR) picture having NalUnitType equal to IDR_N_LP does not have associated leading pictures present in the bitstream. An IDR picture having NalUnitType equal to IDR_W_RADL does not have associated RASL pictures present in the bitstream, but may have associated RADL pictures in the bitstream.

For VCL NAL units of any particular picture, the following applies:

–	If mixed_nalu_types_ flag is equal to 0, all the VCL NAL units shall have the same value of nal_unit_type.

–	Otherwise, some of the VCL NAL units shall have a particular IRAP NAL unit type value (i.e., a value of nal_unit_type in the range of IDR_W_RADL to CRA_NUT, inclusive), while all the other VCL NAL units shall have a particular non-IRAP VCL NAL unit type (i.e., a value of nal_unit_type in the range of TRAIL_NUT to RSV_VCL_15, inclusive, or equal to GRA_NUT).

...

### Access unit delimiter RBSP semantics

...

mixed_nalu_types_flag equal to 1 specifies that each picture associated with the access unit delimiter has multiple VCL NAL units and these NAL units do not have the same value of nal_unit_type. mixed_nalu_types_flag equal to 0 specifies that the VCL NAL units of each picture associated with the access unit delimiter has the same value of nal_unit_type. When sps_idr_rpl_present_flag is equal to 0, the value of mixed_nalu_types_flag shall be equal to 0.

For each IRAP slice in a picture picA that also has at least one non-IRAP slice, the following applies:

- The IRAP slice shall belong to a subpicture subpicA for which the value of the corresponding sub_pic_treated_as_pic_flag[ i ] is equal to 1.

- The IRAP slice shall not belong to a subpicture in the same picture containing one or more non-IRAP slices.

- For all the following layer AUs in decoding order, neither RefPicList[ 0 ] nor RefPicList[ 1 ] of a slice in subpicA shall include any picture preceding picA in decoding order in an active entry.

...

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).