| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0127 |
| --- | --- |

| Title: | On subpicture wraparound motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Yuwen He, Ahmed Hamza 9276 Scranton Road Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to signal wraparound motion compensation for subpicture when subpicture is treated as a picture in the decoding process.

# Introduction

In conventional motion compensation, repetitive padding is applied to derive the values of the out-of-bounds samples by copying from those nearest neighbours on the corresponding picture boundary. While for ERP projection format, the spherical neighbors outside of the left picture boundaries can be obtained from samples inside the right picture boundary, and vice versa. Given the wide usage of the ERP projection format, and the relative ease of implementation, the horizontal wrap around motion compensation was adopted in the VVC to improve the visual quality of 360-video coded in the ERP projection format.

In VVC, a subpicture may be treated as a picture in the decoding process excluding in-loop filtering operations. However, the horizontal wraparound motion compensation may not apply to the sub-picture. It is possible that the spherical neighbors outside of the left subpicture boundaries can be obtained from samples inside  the right sub-picture boundary, and vice versa. Figure 1 shows an example a 360-degree video is divided horizontally into 2 sub-pictures, and horizontal wrap-around motion compensation may apply to each sub-picture.

Figure 1 subpicture-based horizontal wraparound motion compensation

Figure 2 shows example of viewport dependent streaming where subpictures are extracted and repositioned to form a viewport dependent picture. Each subpicture may be from different source with different resolution and wraparound offset settings. As a result, subpicture based wraparound parameters would be required for the decoding process to support various region-wise packing formats.

Figure 2 subpicture extraction and reposition example

# Proposal

## Subpicture wraparound syntax

This contribution proposes to signal wraparound motion compensation for each sub-picture as shown in Table 1. In order to refer to SPS wraparound parameters, the SPS wraparound parameters are proposed to signal before subpicture parameters.

Table 1 Proposed subpicture syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) |  |
| sps_ref_wraparound_offset_minus1 | ue(v) |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| if ( subpic_treated_as_pic_flag[ i ]) |  |
| subpic_ref_wraparound_enable_flag[ i ] | u(1) |
| if (subpic_ref_wraparound_enable_flag[ i ]) |  |
| subpic_ref_wraparound_offset_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

subpic_ref_wraparound_enabled_flag[ i ] equal to 1 specifies that horizontal wrap-around motion compensation is applied on i-th sub-picture inter prediction. subpic_ref_wraparound_enabled_flag[ i ] equal to 0 specifies that horizontal wrap-around motion compensation is not applied on i-th sub-picture.

subpic_ref_wraparound_offset_minus1 plus 1 specifies the offset used for computing the horizontal wrap-around position of i-th sub-picture in units of MinCbSizeY luma samples. The value of ref_wraparound_offset_minus1 shall be in the range of ( CtbSizeY / MinCbSizeY ) + 1 to ( subpic_width_in_luma_samples[ i ] / MinCbSizeY ) − 1, inclusive, where subpic_width_in_luma_samples[ i ] is the value of i-th sub-picture width in luma samples.

## Subpicture decoding process

The corresponding decoding process is proposed as follows.

8.5.3.2.2 Luma sample bilinear interpolation process

…

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..1:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1 and subpic_wraparound_enable_flag[ SubPicIdxis equal to 0, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, subPic_ref_wraparound_enabled_flag ?
		ClipH( ( subpic_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, SubPicWidth, ( xIntL + i ) ) : 
		xIntL + i )

yInti = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL + i )

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, ( xIntL + i ) ) : 
		xIntL + i )

yInti = Clip3( 0, picH − 1, yIntL + i )

8.5.6.3.2 Luma sample interpolation filtering process

…

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..7:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, subPic_ref_wraparound_enabled_flag ?
		ClipH( ( subpic_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, SubPicWidth, xIntL + i − 3 ) : xIntL + i − 3 )

yInti = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL + i − 3 )

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL + i − 3 ) : 
		xIntL + i − 3 )

yInti = Clip3( 0, picH − 1, yIntL + i − 3 )

8.5.6.3.4 Chroma sample interpolation process

…

The variable xOffset is set equal to ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY ) / SubWidthC.

The chroma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..3:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos / SubWidthC, SubPicRightBoundaryPos / SubWidthC, subpic_ref_wraparound_enabled_flag ? ClipH( xOffset, SubPicWidthC, xIntC + i ) : xIntL + i )

yInti = Clip3( SubPicTopBoundaryPos / SubHeightC, SubPicBotBoundaryPos / SubHeightC, yIntL + i )

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picWC − 1, sps_ref_wraparound_enabled_flag ? ClipH( xOffset, picWC, xIntC + i − 1 ) :	
		xIntC + i − 1 )

yInti = Clip3( 0, picHC − 1, yIntC + i − 1 )

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).