| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0132 |
| --- | --- |

| Title: | AHG8: On inter-layer motion prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Yuwen He, Ahmed Hamza 9276 Scranton Road Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes inter-layer motion prediction for SNR scalability. The design can be summarized as follows:

- Propose to signal an VPS flag to enable/disable inter-layer motion prediction

- Proposes constraint on SbTMVP for inter-layer motion prediction

- Proposes to simplify inter-layer motion prediction by skipping TMVP when SbTMVP is available for inter-layer motion prediction

# Introduction

In 15th JVET meeting, it was agreed to allow TMVP but the collocated picture shall have the same resolution as the current picture. It is noted in VVC Specification D6vE that “It is a requirement of bitstream conformance that the resolutions of the reference picture referred to by collocated_ref_idx and the current picture shall be the same.”

For VVC scalability, TMVP and SbTMVP may be allowed for SNR scalability where the resolutions of the inter-layer collocated reference picture and the current picture are the same.

This contribution proposes to support inter-layer motion prediction for SNR scalability with further constraints and simplification.

# Proposal

## Inter-layer motion prediction flag

An inter-layer motion prediction flag is proposed in VPS as shown in Table 1. It assumes inter-layer sample prediction is always enabled between current picture and its reference picture from the direct dependent layer.

Table 1 Proposed Video Parameter Set Syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(6) |
| if( vps_max_layers_minus1 > 0 ) |  |
| vps_all_independent_layers_flag | u(1) |
| if( !vps_all_independent_layers_flag) |  |
| vps_all_inter_layer_motion_prediction_enable_flag | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_layer_id[ i ] | u(6) |
| if( i > 0 && !vps_all_independent_layers_flag ) { |  |
| vps_independent_layer_flag[ i ] | u(1) |
| if( !vps_independent_layer_flag[ i ] ) |  |
| for( j = 0; j < i; j++ ) |  |
| vps_direct_dependency_flag[ i ][ j ] | u(1) |
| if (vps_direct_dependency_flag[ i ][ j ] && ! vps_all_inter_layer_motion_prediction_enable_flag) |  |
| vps_inter_layer_motion_prediction_enable_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |
| if( vps_max_layers_minus1 > 0 ) { |  |

vps_all_inter_layer_motion_prediction_enable_flag equal to 1 specifies the inferred value of vps_inter_layer_motion_prediction_enable_flag[ i ][ j ] for all combinations of i-th and j-th layers. vps_all_inter_layer_motion_prediction_enable_flag equal to  0 specifies that the syntax element vps_inter_layer_motion_prediction_enable_flag[ i ][ j ] may or may not be present. When not present, the value of vps_all_inter_layer_motion_prediction_enable_flag is inferred to be equal to 0.

vps_inter_layer_motion_prediction_enable_flag[ i ][ j ] equal to 1 indicates the j-th direct dependent layer of i-th layer may be used for inter-layer motion prediction of the i-th layer. vps_inter_layer_motion_prediction_enable_flag[ i ][ j ] equal to 0 indicates the j-th direct dependent layer of i-th layer is not used for inter-layer motion prediction of the i-th layer. When not present, the value of vps_inter_layer_motion_prediction_enable_flag[ i ][ j ] is inferred to be equal to the value of vps_all_inter_layer_motion_prediction_enable_flag.

## Constraint of SbTMVP for inter-layer motion prediction

When TMVP fetches the temporal motion vectors from the collocated block in the collocated picture, SbTMVP applies a motion shift before fetching the temporal motion information from the collocated picture, where the motion shift is obtained from the motion vector from one of the spatial neighboring blocks of the current CU.

When SbTMVP applies to inter-layer motion prediction, the motion shift may be constrained to zero since the collocated position for MV fetching is exactly the position of the current block in the inter-layer reference picture. The correlation between current block motion and its zero offset corresponding block in the collocated reference picture would be higher than other locations in collocated reference picture. The constraint may further simplify SbTMVP process.

The corresponding update of VVC spec. is as follows.

collocated_ref_idx specifies the reference index of the collocated picture used for temporal motion vector prediction.

When slice_type is equal to P or when slice_type is equal to B and collocated_from_l0_flag is equal to 1, collocated_ref_idx refers to a picture in list 0, and the value of collocated_ref_idx shall be in the range of 0 to NumRefIdxActive[ 0 ] − 1, inclusive.

When slice_type is equal to B and collocated_from_l0_flag is equal to 0, collocated_ref_idx refers to a picture in list 1, and the value of collocated_ref_idx shall be in the range of 0 to NumRefIdxActive[ 1 ] − 1, inclusive.

When the layer IDs of the reference picture referred to by collocated_ref_idx and the current picture is not the same,  availableFlagInterLayerColPic is set equal to 1; otherwise, availableFlagLayerColPic is set equal to 0.

8.5.5.2 Derivation process for motion vector and reference indices in subblock merge mode

The variables numSbX, numSbY and the subblock merging candidate list, subblockMergeCandList are derived by the following ordered steps:

- When sps_sbtmvp_enabled_flag is equal to 1, the following applies:

- If availableFlagInterLayerColPic is equal to 1, availableFlagA1 is set equal to 0

- Otherwise, for the derivation of availableFlagA1, refIdxLXA1, predFlagLXA1 and mvLXA1 the following applies:

    - The luma location ( xNbA1, yNbA1 ) inside the neighbouring luma coding block is set equal to ( xCb − 1,  yCb + cbHeight − 1 ).

## TMVP and SbTMVP for inter-layer motion prediction

For inter-layer motion prediction, SbTMVP may provide more fine granularity motion prediction than TMVP as the collocated reference picture and the current picture are at the same time instance. TMVP is proposed to be disabled for inter-layer motion prediction when SbTMVP is available. This may also beneficial for the inter prediction as the MV predictor candidate position originally occupied by TMVP can be replaced by other MV predictors, such as spatial predictor.

The corresponding update of VVC spec. clause 8.5.2.12 is as follows.

The variables mvLXCol and availableFlagLXCol are derived as follows:

- If availableFlagSbCol of colCb is equal to 1 and sbFlag is equal to 0, both components of mvLXCol are set equal to 0 and the availableFlagXCol is set equal to 0.

- Otherwise, if colCb is coded in an intra or IBC prediction mode, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise, the motion vector mvCol, the reference index refIdxCol and the reference list identifier listCol are derived as follows:

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).