| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0139-v1 |
| --- | --- |

| Title: | AHG17/AHG12: General questions on subpicture designs for discussion |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | For discussions |  |  |
| Author(s) or Contact(s): | Lulin Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This document asks some general questions on the subpicture design for discussion. The related links to our contribution are also provided.

# Introduction

In the VVC Draft 6 [1], the subpicture information is signalled with the flags for subpicture processing in the SPS, see below as highlighted.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

The rectangular slice information is in the PPS, and the raster scan slice information is implied in the slice header. The current draft defines a subpicture is an rectangular region of one or more slices within a picture.

# General questions for discussion

Question: a subpicture may also consist of one or more rectangular slices within a picture?

Our concerns: yes, only rectangular slices may form a subpicrue region. Although technically muliple raster scan slices may produce a subpicture region but it is not desireable in practise, for example, the picture width such as 360 video may be large enough that practically no subpicture structures may make them useful for the use cases. Separating the raster scan slices from the subpicture structures makes sense to the proponents and it would be more straightforward from the syntax point of view. (also see JVET-P0140 [AHG17/AHG12: Some comments on the VVC text]).

Question: subpicture IDs, if signalled, do not change in the CVS?

Our concerns: it may or may not change. A remapable ID to the subpicure index in the SPS may be flexible in this aspect. (also see JVET-P0141 [AHG17/AHG12: On signalling the subpicture IDs] and JVET-P0144 [AHG17/AHG12: On associating slices with a subpicture])

Question: a sparated subpicture grid in the SPS or signalling in terms of slices?

Our concerns: it may be better to signal the subpicture structures from the slice structures. (also see JVET-P0144 [AHG17/AHG12: On associating slices with a subpicture))

Question: the current subpicture grid, if desired to have, may be improved?

Our concerns: yes, it may be improved using the grid in units of CTBs with the signalling of the bottom-right delta grid index. (also see JVET-P0142 [AHG17/AHG12: Comments on the subpicture grid in the SPS] and JVET-P0143 [AHG17/AHG12: On signalling of subpicture structure in the SPS])

Question: the twp flags of subpic_treated_as_pic_flag[ i ] and loop_filter_across_subpic_enabled_flag[ i ] are orthogonal?

Our concerns: it should not be orthogonal. The flag of subpic_treated_as_pic_flag[ i ] should have a higher priority than that of loop_filter_across_subpic_enabled_flag[ i ]. Practically when a subpicture is treated as a picture including the boundaries, it should be enabling the subpicture extraction process as a meaningful tool for the purpose of subpicture. (also see JVET-P0145 [AHG17/AHG12: On the flags for processing with subpictures and other discussions relating to the ALF process])

Question: can subpictures, if extractable, have a differenct random access properties from the whole picture?

Our concerns: yes, the feature is useful, typically in viewport dependent streming applications and bitstream editing. The AUD may be used to specify that. (also see JVET-P0146 [AHG17/AHG12: On enabling subpicture with random access properties])

# References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team, JVET-O2001

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).