| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0143-v1 |
| --- | --- |

| Title: | AHG17/AHG12: On signalling of subpicture structure in the SPS |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen, Chih-Wei Hsu, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, cw.hsu, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

It is alleged that the proposed approach for the subpicture structures is more efficient in bit savings.

- The subpicture grid element in units of CTBs

- The bottom-right grid index delta specifies the subpicture

- For the first subpicture, subpic_grid_idx_delta_sign_flag[ i ] is not needed

# Introduction

In the current VVC draft [1], the subpicture structure with a grid approach is added in the SPS.

      - 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Proposal

Given a grid in units of 4 samples in the current draft, all grid elements are scanned for labelling with a subpicture index, it seems not efficient as each grid element needs some bits of u(v) and decoding process needs memory for that labelling. This document proposes a way similar to the signalling of the slice structures with the bottom-right brick index delta and uses the grid in units of CTBs, as in a separate document, JVET-P0142 (AHG17/AHG12: Comments on the subpicture grid in the SPS), to specify the subpicture structure in the SPS. max_subpics_minus1 is changed to num_subpics_minus1 and num_subpics_minus1 is moved to the VPS. For the first subpicture, subpic_grid_idx_delta_sign_flag[ i ] is not needed.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| log2_ctu_size_minus5 | u(2) |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| num_subpics_minus1 | u(8) |
| signalled_subpic_grid_flag | u(1) |
| if( signalled_subpic_grid_flag ) { |  |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| } |  |
| else { |  |
| subpic_grid_col_width_minus1 = 0 |  |
| subpic_grid_row_height_minus1 = 0 |  |
| } |  |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| bottom_right_subpic_gid_idx_length_minus1 | u(v) |
| for( i = 0; i < num_subpics_minus1; i++ ) { |  |
| bottom_right_subpic_grid_idx_delta[ i ] | u(v) |
| if( i > 0 ) |  |
| subpic_grid_idx_delta_sign_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_subpics_minus1; i++ ) { |  |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| … |  |
| log2_ctu_size_minus5 | u(2) |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

num_subpics_minus1 plus 1 specifies the number of subpictures that may be present in the CVS. num_subpics_minus1 shall be in the range of 0 to 254. The value of 255 is reserved for future use by ITU-T | ISO/IEC.

subpic_grid_col_width_minus1 plus 1 specifies the width of each element of the subpicture identifier grid, excluding the right-most grid column of the picture, in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / CtbSizeY) ) bits. When not present, subpic_grid_row_width_minus1 is inferred to be 0.

The variable NumSubPicGridCols is derived as follows:

NumSubPicGridCols = (pic_width_max_in_luma_samples + subpic_grid_col_width_minus1 * CtbSizeY + CtbSizeY – 1) /

( subpic_grid_col_width_minus1 * CtbSizeY + CtbSizeY )                          (7-5)

subpic_grid_row_height_minus1 plus 1 specifies the height of each element of the subpicture identifier grid, excluding the bottom grid row of the picture, in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / CtbSizeY) ) bits. When not present, subpic_grid_row_height_minus1 is inferred to be 0.

The variable NumSubPicGridRows is derived as follows:

NumSubPicGridRows = (pic_height_max_in_luma_samples + subpic_grid_row_height_minus1 * CtbSizeY + CtbSizeY – 1 ) /
		( subpic_grid_row_height_minus1 * CtbSizeY + CtbSizeY)                         (7-6)

The specified elements of the subpicture identifier grid are indexed in the order of ranster scan.

bottom_right_subpic_gid_idx_length_minus1 plus 1 specifies the number of bits used to represent the syntax element bottom_right_subpic_grid_idx_delta[ i ]. The value of bottom_right_subpic_grid_idx_length_minus1 shall be in the range of 0 to Ceil( Log2( NumSubPicGridRows * NumSubPicGridCols ) ) − 1, inclusive.

bottom_right_subpic_grid_idx_delta[ i ] when i is greater than 0 specifies the difference between the subpicture grid index of the subpicture located at the bottom-right corner of the i-th subpicture and and the subpicture grid index of the bottom-right corner of the ( i – 1 )-th subpicture. bottom_right_subpic_grid_idx_delta[ 0 ] specifies the subpic grid index of the bottom right corner of the 0-th subpicture.

subpic_grid_idx_delta_sign_flag[ i ] equal to 1 indicates a positive sign for bottom_right_subpic_grid_idx_delta[ i ]. sign_bottom_right_subpic_grid_idx_delta[ i ] equal to 0 indicates a negative sign for bottom_right_subpic_grid_idx_delta[ i ].

# Conclusions

The proposed approach for the subpicture structures is more efficient in bit savings.

- The subpicture grid element in units of CTBs

- The bottom-right grid index delta specifies the subpicture

- For the first subpicture, subpic_grid_idx_delta_sign_flag[ i ] is not needed

# References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team, JVET-O2001

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).