| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0144-v1 |
| --- | --- |

| Title: | AHG17/AHG12: On associating slices with a subpicture |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

Two methods are proposed to associate the slices with a subpicture. It is alleged that Method 2 is more efficient and simpler than Method 1.

- Method 1: In the PPS, for each slice, assign a subpicture ID. In the slice header, signal the subpicure ID for the raster scan slice to which the current slice belongs.

- Method 2: Remove the subpicture grid based signalling in the SPS and specify the subpicture structures based on the slice structures in the PPS with the subpicture size derivations. In the slice header, signal the subpicure ID for the raster scan slice to which the current slice belongs.

- A bug fix: When single_tile_in_pic_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to 0.

# Introduction

In the VVC Draft 6 [1], the subpicture information is signalled in the SPS and the rectangular slice information remains in the PPS as highlighted below.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| output_flag_present_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| … |  |
| single_brick_per_slice_flag | u(1) |
| if( !single_brick_per_slice_flag ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag && !single_brick_per_slice_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| bottom_right_brick_idx_length_minus1 | ue(v) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| bottom_right_brick_idx_delta[ i ] | u(v) |
| brick_idx_delta_sign_flag[ i ] | u(1) |
| } |  |
| } |  |
| loop_filter_across_bricks_enabled_flag | u(1) |
| if( loop_filter_across_bricks_enabled_flag ) |  |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( rect_slice_flag ) { |  |
| signalled_slice_id_flag | u(1) |
| if( signalled_slice_id_flag ) { |  |
| signalled_slice_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| slice_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| slice_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Proposal

A subpicture, as defined in the current VVC Draft 6, is a rectangular region of one or more slices within a picture. Currently the subpicture information is in the SPS, the rectangular slice information is in the PPS, and the raster scan slice information is implied in the slice header. There needs an association in general between the subpictures and the slices. Two methods are propopsed in this contribution for the group discussion.

Method 1

In the PPS, the slice association with subpicture may be added (see the highlighted in yellow) when specifying the rectangular slice infromation with the subpicture IDs. In the slice header, signal the subpicure ID for the raster scan slice to which the current slice belongs. In both cases, the subpicture ID is mapped to the subpicture index specified in the SPS.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| … |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| single_tile_in_pic_flag | u(1) |
| … |  |
| if( rect_slice_flag ) { |  |
| signalled_slice_id_flag | u(1) |
| if( signalled_slice_id_flag ) { |  |
| signalled_slice_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| slice_id[ i ] | u(v) |
| } |  |
| } |  |
| if( subpics_present_flag ) { |  |
| if( rect_slice_flag ) |  |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| subpic_id[ i ] | u(v) |
| } |  |
| entropy_coding_sync_enabled_flag | u(1) |
| … |  |
| slice_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

num_slices_in_pic_minus1 plus 1 specifies the number of slices in each picture referring to the PPS. The value of num_slices_in_pic_minus1 shall be in the range of 0 to NumBricksInPic − 1, inclusive. When not present and single_brick_per_slice_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to NumBricksInPic − 1. When single_tile_in_pic_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to 0.

subpic_id[ i ] specifies the subpicture ID to which the i-th slice belongs. The length of the subpic_id[ i ] syntax element is Ceil( Log2( max_subpics_minus1 + 1 ) ) bits. The value of subpic_id[ i ] is a mapping to the subpicture index specified in the SPS. The slices with the same subpicture ID collectively form the subpicture region.

| slice_header( ) { |  | Descriptor |
| --- | --- | --- |
| slice_pic_parameter_set_id |  | ue(v) |
| if( subpics_present_flag && !rect_slice_flag) |  |  |
| subpicture_id | u(v) |  |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |  |
| slice_address |  | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |  |
| num_bricks_in_slice_minus1 |  | ue(v) |
| non_reference_picture_flag |  | u(1) |
| slice_type |  | ue(v) |
| if( separate_colour_plane_flag = = 1 ) |  |  |
| colour_plane_id |  | u(2) |
| slice_pic_order_cnt_lsb |  | u(v) |

subpicture_id specifies the subpicture ID to which the the current slice belongs. The length of the subpic_id syntax element is Ceil( Log2( max_subpics_minus1 + 1 ) ) bits. The value of subpicture_id is a mapping to the subpicture index specified in the SPS. The slices with the same subpicture ID collectively form the subpicture region.

Method 2

As the slice information is specified in the PPS and a subpicture may consist of one or more slices, the subpicture information may be derived from the slice information instead of specifying in the SPS. The syntax may be modified as highlighted as below.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics max_subpics_minus1; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| … |  |
| if( rect_slice_flag ) { |  |
| signalled_slice_id_flag | u(1) |
| if( signalled_slice_id_flag ) { |  |
| signalled_slice_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| slice_id[ i ] | u(v) |
| } |  |
| } |  |
| if( subpics_present_flag ) { |  |
| if( rect_slice_flag ) |  |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| subpic_id[ i ] | u(v) |
| } |  |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

num_slices_in_pic_minus1 plus 1 specifies the number of slices in each picture referring to the PPS. The value of num_slices_in_pic_minus1 shall be in the range of 0 to NumBricksInPic − 1, inclusive. When not present and single_brick_per_slice_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to NumBricksInPic − 1. When single_tile_in_pic_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to 0.

subpic_id[ i ] specifies the subpicture ID to which the i-th slice belongs. The length of the syntax element is Ceil( Log2( max_subpics_minus1 + 1 ) ) bits. The value of subpic_id[ i ] is a mappting to the subpicture index in the range of 0 to max_subpics+minus1, inclusive, specified in the SPS. The slices with the same subpicture ID collectively form the subpicture region.

The variable SubPicTopLeftBrickIdx[ subpic_id[ i ] ], SubPicBottomRightBrickIdx[ subpic_id[ i ] ], which specify the brick index of the brick located at the top left corner of the subpicture with subpic_id[ i ] and the brick index of the brick located at the bottom right corner of the subpicture with subpic_id[ i ], and NumSubPics, the number of subpictures, are derived as follows:

- rect_slice_flag is equal to 1

NumSubPics = 0
for( i = 0; i <= num_slices_in_pic_minus1; i++ ) {

if ( i = = 0)

SubPicTopLeftBrickIdx[ subpic_id[ i ] ] = TopLeftBrickIdx[ i ]
	else if( subpic_id[ i ]  !=  subpic_id[ i – 1 ] ) {
			SubPicTopLeftBrickIdx[ subpic_id[ i ] ] = TopLeftBrickIdx[ i ]

SubPicBottomRightBrickIdx[ subpic_id[ i – 1 ] ] = BottomRightBrickIdx[ i - 1 ]

NumSubPics++

}
}

- otherwise ( rect_slice_flag is equal to 0 )

SubPicTopLeftBrickIdx[ subpic_id] = SliceBrickIdx[ 0 ]

SubPicBottomRightBrickIdx[ subpic_id ] = SliceBrickIdx[ 0 ] + num_bricks_in_slice_minus1

The (7-93) derivation in 7.4.7.1 may be modified accordingly as below.

The variables SubPicIdx, SubPicLeftBoundaryPos, SubPicTopBoundaryPos, SubPicRightBoundaryPos, and SubPicBotBoundaryPos are derived as follows:

SubPicIdx = CtbToSubPicIdx[ CtbAddrBsToRs[ FirstCtbAddrBs[ SliceBrickIdx[ 0 ] ] ] ]
if( subpic_treated_as_pic_flag[ SubPicIdx ] ) {

brickIdx = SubPicTopLeftBrickIdx[ SubPicIdx ]
	   SubPicLeftBoundaryPos = ( BrickColBd[ brickIdx ] - BrickWidth[ brickIdx ] ) * CtbSizeY

SubPicTopBoundaryPos = ( BrickRowBd[ brickIdx ] - BrickHeight[ brickIdx ] ) * CtbSizeY

brickIdx = SubPicBottomRightBrickIdx[ SubPicIdx ]	 (7-93)

SubPicRightBoundaryPos = BrickColBd[ brickIdx ] * CtbSizeY
	   SubPicBotBoundaryPos = BrickRowBd[ brickIdx ] * CtbSizeY

}

| slice_header( ) { |  | Descriptor |
| --- | --- | --- |
| slice_pic_parameter_set_id |  | ue(v) |
| if( subpics_present_flag && !rect_slice_flag) |  |  |
| subpicture_id | u(v) |  |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |  |
| slice_address |  | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |  |
| num_bricks_in_slice_minus1 |  | ue(v) |
| non_reference_picture_flag |  | u(1) |
| slice_type |  | ue(v) |
| if( separate_colour_plane_flag = = 1 ) |  |  |
| colour_plane_id |  | u(2) |
| slice_pic_order_cnt_lsb |  | u(v) |

subpicture_id specifies the subpicture ID to which the the current slice belongs. The length of the subpic_id syntax element is Ceil( Log2( max_subpics_minus1 + 1 ) ) bits. The value of subpicture_id is a mapping to the subpicture index in the range of 0 to max_subpics_minus1, inclusive, specified in the SPS. The slices with the same subpicture ID collectively form the subpicture region.

# Conclusions

Two methods are proposed to associate the slices with a subpicture. Method 2 is more efficient and simpler than Method 1.

- Method 1: In the PPS, for each slice, assign a subpicture ID. In the slice header, signal the subpicure ID for the raster scan slice to which the current slice belongs.

- Method 2: Remove the subpicture grid based signalling in the SPS and specify the subpicture structures based on the slice structures in the PPS with the subpicture size derivations. In the slice header, signal the subpicure ID for the raster scan slice to which the current slice belongs.

- A bug fix: When single_tile_in_pic_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to 0.

# References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team, JVET-O2001

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).