| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0149-v1 |
| --- | --- |

| Title: | AHG18: Disabling LMCS for lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two methods to achieve lossless coding when luma mapping with chroma residual scaling (LMCS) is enabled at sequence level and transquant bypass mode is enabled at picture level. In Method 1, if the transquant bypass mode is enabled at picture level, it is proposed to force disabling LMCS at slice level. In Method 2, it is proposed to force disabling LMCS for lossless coded blocks and to allow LMCS for lossy coded blocks. Both methods can support lossless coding for an entire picture and lossless coding for a partial picture.

# Introduction

In VTM6.0, when LMCS is enabled at both sequence level and slice level, all of the luma reconstruction samples in the slice are in the forward mapped domain. For inter block, since the luma reconstruction samples in reference pictures are in the original domain, luma prediction samples are converted to the forward mapped domain before adding the residuals. The inverse mapping is applied to all the luma reconstructed samples in one slice before loop filtering. As for the chroma components, the chroma residuals are scaled by a scaling factor before added to the chroma prediction samples. Since the luma mapping and chroma residual scaling are not perfectly invertible, lossless coding cannot be achieved with LMCS. In this contribution, two methods are proposed to achieve lossless coding when LMCS is enabled at sequence level and transquant bypass mode is enabled at picture level.

# Proposed methods

## Method 1: Disable LMCS at slice level

It is proposed to disable LMCS at slice level if lossless coding is made possible at picture level. The changes to VVC Draft 6 [1] are shown as follows.

| slice_header( ) { |  |
| --- | --- |
| … |  |
| if( sps_lmcs_enabled_flag && ! transquant_bypass_enabled_flag) { |  |
| slice_lmcs_enabled_flag | u(1) |
| if( slice_lmcs_enabled_flag ) { |  |
| slice_lmcs_aps_id | u(2) |
| if( ChromaArrayType != 0 ) |  |
| slice_chroma_residual_scale_flag | u(1) |
| } |  |
| } |  |
| … |  |

When transquant_bypass_enabled_flag is set to 1 in picture parameter set (PPS), slice_lmcs_enabled_flag is not signalled and inferred to 0.

## Method 2: Disable LMCS at block level

In cases when some blocks in a picture can be lossless coded and the rest blocks in the picture can be lossy coded, LMCS can still be applied to lossy coded blocks as a coding efficiency tool. Therefore, in Method 2, LMCS is disabled only for lossless blocks. If a block is lossless coded, the forward mapping of inter prediction luma samples, the inverse mapping of luma reconstructed samples before loop filtering, the chroma scaling are skipped for the block. The changes to VVC Draft 6 is as follows.

### Picture reconstruction process #### General Inputs to this process are: - a location ( xCurr, yCurr ) specifying the top-left sample of the current block relative to the top-left sample of the current picture component, - the variables nCurrSw and nCurrSh specifying the width and height, respectively, of the current block, - a variable cIdx specifying the colour component of the current block, - an (nCurrSw) x (nCurrSh) array predSamples specifying the predicted samples of the current block, - an (nCurrSw) x (nCurrSh) array resSamples specifying the residual samples of the current block. Output of this process is a reconstructed picture sample array recSamples. Depending on the value of the colour component cIdx, the following assignments are made: - If cIdx is equal to 0, recSamples corresponds to the reconstructed picture sample array SL and the function clipCidx1 corresponds to Clip1Y. - Otherwise, if cIdx is equal to 1, tuCbfChroma is set equal to tu_cbf_cb[ xCurr ][ yCurr ], recSamples corresponds to the reconstructed chroma sample array SCb and the function clipCidx1 corresponds to Clip1C. - Otherwise (cIdx is equal to 2), tuCbfChroma is set equal to tu_cbf_cr[ xCurr ][ yCurr ], recSamples corresponds to the reconstructed chroma sample array SCr and the function clipCidx1 corresponds to Clip1C. Depending on the value of slice_lmcs_enabled_flag, the following applies: - If cu_transquant_bypass_flag[ xCurr ][ yCurr ] is equal to 1 or slice_lmcs_enabled_flag is equal to 0, the (nCurrSw)x(nCurrSh) block of the reconstructed samples recSamples at location ( xCurr, yCurr ) is derived as follows for i = 0..nCurrSw − 1, j = 0..nCurrSh − 1: recSamples[ xCurr + i ][ yCurr + j ] = clipCidx1( predSamples[ i ][ j ] + resSamples[ i ][ j ] ) (8-1005) - Otherwise (slice_lmcs_enabled_flag is equal to 1), the following applies: - If cIdx is equal to 0, the following applies: - The picture reconstruction with mapping process for luma samples as specified in clause 8.7.5.2 is invoked with the luma location ( xCurr, yCurr ), the block width nCurrSw and height nCurrSh, the predicted luma sample array predSamples, and the residual luma sample array resSamples as inputs, and the output is the reconstructed luma sample array recSamples. - Otherwise (cIdx is greater than 0), the picture reconstruction with luma dependent chroma residual scaling process for chroma samples as specified in clause 8.7.5.3 is invoked with the chroma location ( xCurr, yCurr ), the transform block width nCurrSw and height nCurrSh, the coded block flag of the current chroma transform block tuCbfChroma, the predicted chroma sample array predSamples, and the residual chroma sample array resSamples as inputs, and the output is the reconstructed chroma sample array recSamples. The following assignments are made for i = 0..nCurrSw − 1, j = 0..nCurrSh − 1: xVb = ( xCurr + i ) % ( ( cIdx = = 0 ) ? IbcBufWidthY : IbcBufWidthC ) (8-1006) yVb = ( yCurr + j ) % ( ( cIdx = = 0 ) ? CtbSizeY : CtbSizeC ) (8-1007) IbcVirBuf[ cIdx ][ xVb ][ yVb ] = recSamples[ xCurr + i ][ yCurr + j ] (8-1008) IsAvailable[ cIdx ][ xCurr + i ][ yCurr + j ] = TRUE (8-1009) ### … ### Picture inverse mapping process for luma samples #### General Input to this process is a reconstructed picture luma sample array SL. The output to this process is a modified reconstructed picture luma sample array SL. The inverse mapping process for a luma sample SL[ x ][ y ] with x = 0..pic_width_in_luma_samples − 1, y = 0..pic_height_in_luma_samples − 1 is invoked as specified in clause 8.8.2.2 with the variable lumaSample set equal to SL[ x ][ y ] and the variable isTransQuantByPass set equal to cu_transquant_bypass_flag [ x ][ y ] as the input and the output is assigned to the luma sample SL[ x ][ y ]. #### Inverse mapping process for a luma sample Input to this process is a luma sample lumaSample and a variable isTransQuantByPass. Output of this process is a modified luma sample invLumaSample . The value of invLumaSample is derived as follows: - If slice_lmcs_enabled_flag of the slice that contains the luma sample lumaSample is equal to 1 and isTransQuantByPass is equal to 0, the following ordered steps apply: - The variable idxYInv is derived by invoking the identification of piece-wise function index process for a luma sample as specified in clause 8.8.2.3 with lumaSample as the input and idxYInv as the output. - The variable invSample is derived as follows: invSample = InputPivot[ idxYInv ] + ( InvScaleCoeff[ idxYInv ] * (8-1019) ( lumaSample − LmcsPivot[ idxYInv ] ) + ( 1 << 10 ) ) >> 11 - The inverse mapped luma sample invLumaSample is derived as follows: invLumaSample = Clip1Y( invSample ) (8-1020) - Otherwise, invLumaSample is set equal to lumaSample.

# Conclusions

This contribution presents two methods to achieve lossless coding when LMCS is enabled at sequence level. In Method 1, LMCS is disabled at slice level if transquant bypass mode is enabled at picture level. In Method 2, LMCS is disabled at block level. In lossless coded blocks, the forward mapping, inverse mapping, and chroma scaling are skipped.

# References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team, JVET-O2001.

- AHG18 VTM-6.0-lossless, https://vcgit.hhi.fraunhofer.de/jvet-o-ahg-18/VVCSoftware_VTM

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).