| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0154-v2 |
| --- | --- |

| Title: | CE4-related: PROF prediction sample range reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Zhi-Yi Lin, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {peter.chuang, zhi-yi.lin, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VVC Draft 6, prediction refinement with optical flow (PROF) for affine prediction was adopted to refine the affine prediction samples. The bit-depth of each refined prediction sample is 17-bit for 10-bit video. The refined prediction samples can be further processed by weighted prediction, which leads to using multipliers beyond 16-bit in the weighted prediction and is not software implementation friendly. In this proposal, three methods are proposed. In Method 1, it is proposed to clip the prediction offset within 14 bits for 10-bit video. In Method 2, it is proposed to clip the refined prediction samples to 16 bits for 10-bit video. Both methods allow using 16-bit multipliers in the weighted prediction. Simulation results reportedly show that the luma BD-rate of Method 1 and Method 2 are 0.00% and 0.00% under RA, respectively. In Method 3, PROF is disabled when weighted prediction or bi-prediction coding unit level weights (BCW) is aplied. Simulation results reportedly show that the luma BD-rate of Method 3 is 0.xx% under RA

# Introduction

In the PROF of VVC Draft 6 [1], affine prediction samples are refined. For a prediction sample I(i, j), a prediction offset ΔI(i, j) is derived and added to I(i, j). The ΔI(i, j) is calculated by the optical flow equation gx(i,j)*Δvx (i,j)+gy(i,j)*Δvy(i,j), where the gx(i,j) and gy(i,j) are the horizontal and vertical gradients of the prediction samples using a 3-tap filter [-1, 0, 1], and the the Δvx(i,j) and Δvy(i,j) are the horizontal and vertical motion vector (MV) differences between sample-based affine MV and the subblock MV computed for the sample at location (i,j). In VTM6.0, the bitdepth of I(i, j) and ΔI(i, j) are 16-bit and 17-bit, respectively. The bit-depth of refined prediction sample I’(I, j), which is equal to I(i, j) + ΔI(i, j), is 17-bit. The refined prediction sample can be further processed by weighted prediction, which leads to using multipliers beyond 16-bit in the weighted prediction. Using multipliers beyond 16-bit is not implementation friendly, especially for software implementations that require 32-bit multipliers. The 32-bit single instruction multiple data (SIMD) implementation is required instead of 16-bit SIMD implementation, which decreases the processing throughput. In this proposal, three methods are proposed to solve this issue.

# Proposed methods

## Method 1: Prediction offset clipping

It is proposed to clip the prediction offset, ΔI(i, j), to K bits with range as [0, 2K – 1]. K is equal to sample bit-depth + max(2, 14 – sample bit-depth). In case of 10-bit video, K is equal to 14. The specification changes are as follows.

#### 8.5.6.4 Prediction refinement with optical flow process

Inputs to this process are:

- two variables sbWidth and sbHeight specifying the width and the height of the current subblock,

- one (sbWidth + borderExtension) * (sbHeight + borderExtension) prediction sample array predSamples,

- one (sbWidth * sbHeight) motion vector difference array diffMv.

Output of this process is the (sbWidth)x(sbHeight) array pbSamples of prediction sample values.

Variable shift1 is set equal to Max( 6, BitDepthY − 6 ).

For x =0..sbWidth − 1, y =0..sbHeight − 1, the following ordered steps apply:

- The variables gradientH[ x ][ y ] and gradientV[ x ][ y ] are derived as follows:

gradientH[ x ][ y ]  =  ( predSamples[ x + 2 ][ y ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 )	(8-796)

gradientV[ x ][ y ]  =  ( predSamples[ x ][ y + 2 ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 )	(8-797)

- The variable dI is derived as follows:

dI = gradientH[ x ][ y ] * diffMv[ x ][ y ][ 0 ] + gradientV[ x ][ y ] * diffMv[ x ][ y ][ 1 ]	(8-798)

- Prediction sample value at location ( x, y ) in the subblock is derived as follows:

dILimit = (1 << (BitDepthY + max(2, 14 – BitDepthY))) – 1

pbSamples[ x ][ y ] = predSamples[ x + 1 ][ y + 1 ] + Clip3(0, dILimit, ( dI + 1 ) >> 1 )	(8-799)

## Method 2: Refined prediction sample clipping

It is proposed to clip the refined prediction sample, I’(i, j), to K bits with range as [0, 2K – 1]. The K is equal to max(16, sample bit-depth + max(2, 14 – sample bit-depth)). In case of 10-bit video, K is equal to 16. The specification changes are as follows.

#### 8.5.6.4 Prediction refinement with optical flow process

Inputs to this process are:

- two variables sbWidth and sbHeight specifying the width and the height of the current subblock,

- one (sbWidth + borderExtension) * (sbHeight + borderExtension) prediction sample array predSamples,

- one (sbWidth * sbHeight) motion vector difference array diffMv.

Output of this process is the (sbWidth)x(sbHeight) array pbSamples of prediction sample values.

Variable shift1 is set equal to Max( 6, BitDepthY − 6 ).

For x =0..sbWidth − 1, y =0..sbHeight − 1, the following ordered steps apply:

- The variables gradientH[ x ][ y ] and gradientV[ x ][ y ] are derived as follows:

gradientH[ x ][ y ]  =  ( predSamples[ x + 2 ][ y ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 )	(8-796)

gradientV[ x ][ y ]  =  ( predSamples[ x ][ y + 2 ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 )	(8-797)

- The variable dI is derived as follows:

dI = gradientH[ x ][ y ] * diffMv[ x ][ y ][ 0 ] + gradientV[ x ][ y ] * diffMv[ x ][ y ][ 1 ]	(8-798)

- Prediction sample value at location ( x, y ) in the subblock is derived as follows:

pbSamplesLimit = (1 << max(16, (BitDepthY + max(2, 14 – BitDepthY)))) – 1

pbSamples[ x ][ y ] = Clip3(0, pbSamplesLimit, predSamples[ x + 1 ][ y + 1 ] + ( ( dI + 1 ) >> 1 ))	(8-799)

## Method 3: Disabling PROF when weighted prediction or BCW is applied

It is proposed to disable the PROF when the weighted prediction or BCW is applied. The specification changes are as follows.

The variable cbProfFlagLX is derived as follows:

–	If one or more of the following conditions are true, cbProfFlagLX is set equal to FALSE.

–	sps_affine_prof_enabled_flag is equal to 0.

–	fallbackModeTriggered is equal to 1.

–	numCpMv is equal to 2 and cpMvLX[ 1 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 1 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ].

–	numCpMv is equal to 3 and cpMvLX[ 1 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 1 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ] and cpMvLX[ 2 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 2 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ].

–	luma_weight_lX_flag[ refIdxLX ] is not equal to 0.

–	BcwIdx[ xCb ][ yCb ] is not equal to 0

–	Otherwise, cbProfFlagLX set equal to TRUE.

# Experiment results

The proposed methods are implemented on top of VTM6.0 [2], and the simulation is performed under the common test conditions (CTCs) as described in JVET-O1010 [3]. Table 1, Table 2, and Table 3 show the results of Method 1, Method 2, and Method 3, respectively. Simulation results reportedly show that the BD-rate changes are negligible.

Table 1.  Results of Method 1 (to be updated)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% |  |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 104% |
| Class F | 0.01% | 0.01% | 0.01% | 100% | 104% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 104% |
| Class F |  |  |  |  |  |

Table 2.  Results of Method 2 (to be updated)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% |  |
| Class C | 0.00% | 0.00% | 0.00% | 100% |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 100% |  |
| Class F | 0.00% | 0.00% | 0.00% | 101% |  |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F |  |  |  |  |  |

Table 1.  Results of Method 3 (to be updated)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

# Conclusions

This contribution presents two methods to allow using 16-bit multipliers in the weighted prediction for PROF. Simulation results reportedly show that the luma BD-rates of Method 1 and Method 2 are 0.00% and 0.00% under RA, respectively. In Method 3, PROF is disabled when the weighted prediction or BCW is aplied. Simulation results reportedly show that the luma BD-rate of Method 3 is 0.xx% under RA

# References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team, JVET-O2001.

- VTM6.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).