| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0156-v2 |
| --- | --- |

| Title: | CE5-related: ALF padding process when raster scan slices are used |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai, Ching-Yeh Chen, Tzu-Der Chuang, Chih-Wei Hsu Yu-Wen Huang, Shaw-Min Lei Anand Meher Kotra, Semih Esenlik, Han Gao, Biao Wang, Elena Alshina | Email: | {jenny.lai, chingyeh.chen, peter.chuang, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com Anand.meher.kotra @huawei.com |
| Source: | MediaTek Inc., Huawei Technologies. Co., Ltd. |  |  |

_____________________________

- Abstract

In VVC Draft 6, when raster scan slices are used in one picture and the loop filtering process cannot cross the slice boundaries (i.e. loop_filter_across_slices_enabled_flag is equal to 0), the adaptive loop filter (ALF) padding process is undefined in two corner cases. In the current design, only the left, top, right, and bottom boundaries of one coding tree unit (CTU) are considered to decide the ALF padding process is enabled or not. This is good enough for rectangular slices in one picture but not enough for raster scan slices. In raster scan slices, the top-left (bottom-right) neighbouring samples may be unavailable, even if both top and left (bottom and right) boundaries of one CTU are not slice boundaries. In order to fix this, a method is proposed as follows. If top-left (bottom-right) neighbouring samples are unavailable, top (bottom) neighbouring samples are treated as unavailable. Then, the current ALF padding process can be simply applied to handle the unavailable neighbouring samples.

Introduction

In VVC Draft 6 [1], when the loop filtering process cannot cross the slice boundaries (i.e., loop_filter_across_slices_enabled_flag is equal to 0), the condition of enabling the ALF padding process only considers checking whether the left, top, right, and bottom boundaries of one CTU are slice boundaries separately. If one picture is partitioned into rectangular slices, this condition is good enough, as shown in Fig. 1, where the red block is a to-be-processed 4x4 block. In this case, the availability of top-left (bottom-right) neighbouring samples is the same as that of top (bottom) neighbouring samples or left (right) neighbouring samples. Hence checking four boundaries of one CTU is good enough. However, the current condition of enabling the ALF padding process is improper when raster scan slices are used in one picture. As shown in Fig. 2, two red blocks are two to-be-processed 4x4 blocks. The unavailable neighbouring samples of these two blocks are located in the corresponding top-left region and the corresponding bottom-right region, respectively. These cases cannot be detected correctly by the current condition of enabling the ALF padding process, so the corresponding ALF padding process is undefined to handle these unavailable neighbouring samples.

Figure 1. One picture with rectangular slices.

Figure 2. One picture with raster scan slices.

Proposed method

In order to solve this issue, two additional checks are added into the condition of enabling ALF padding process, when loop filter cannot cross slice boundaries (loop_filter_across_slices_enabled_flag is equal to 0). If top-left neighboring samples are unavailable and left neighbouring samples are available, top neighbouring samples are also treated as unavailable. If bottom-right neighboring samples are unavailable and right neighbouring samples are available, bottom neighbouring samples are treated as unavailable. Next, the current ALF padding process is applied to handle the unavailable neighbouring samples.

Corresponding text changes

The corresponding text changes are provided and highlighted in yellow color in the following.

8.8.5.5	ALF boundary position derivation process

Inputs of this process are:

  - a luma location ( xCtb, yCtb ) specifying the top-left sample of the current luma coding tree block relative to the top left sample of the current picture,

  - a luma location ( x, y ) specifying the current sample relative to the top-left sample of the current luma coding tree block.

Output of this process are:

  - the left vertical boundary position clipLeftPos,

  - the right vertical boundary position clipRightPos,

  - the above horizontal boundary position clipTopPos,

  - the below horizontal boundary position clipBottomPos.

The variables clipLeftPos, clipRightPos, clipTopPos and clipBottomPos are set equal to −128.

The variable clipTopPos is modified as follows:

  - If the bottom boundary of the current coding tree block is not the bottom boundary of the picture and y − ( CtbSizeY − 4 ) is greater than or equal to 0, the variable clipTopPos is set equal to yCtb + CtbSizeY − 4.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is equal to 0, and yCtb + y − PpsVirtualBoundariesPosY[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

clipTopPos =  PpsVirtualBoundariesPosY[ n ]		(8-1292)

  - Otherwise, if y is less than 3, and the top boundary of the current coding tree block is not the top boundary of the picture, and one or more of the following conditions are true, the variable clipTopPos is set equal to yCtb:

    - If the top boundary of the current coding tree block is the top boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the top boundary of the current coding tree block is the top boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If  x is less than 2 and (xCtb -1, yCtb -1) location does not belong to the current slice, the left boundary of the current coding tree block is not the left boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the top boundary of the current coding tree block is the top boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

The variable clipBottomPos is modified as follows:

  - If the bottom boundary of the current coding tree block is not the bottom boundary of the picture and CtbSizeY − 4 − y is greater than 0 and is less than 4, the variable clipBottomPos is set equal to yCtb + CtbSizeY − 4.

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, PpsVirtualBoundariesPosY[ n ] % CtbSizeY is equal to 0, PpsVirtualBoundariesPosY[ n ] is not equal to pic_height_in_luma_samples − 1 or 0, and PpsVirtualBoundariesPosY[ n ] − yCtb − y is greater than 0 and less than 4 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

clipBottomPos =  PpsVirtualBoundariesPosY[ n ]		(8-1293)

  - Otherwise, if CtbSizeY − y is less than 4, and the bottom boundary of the current coding tree block is not the bottom boundary of the picture, and one or more of the following conditions are true, the variable clipBottomPos is set equal to yCtb + CtbSizeY:

    - If the bottom boundary of the current coding tree block is the bottom boundary of the brick, and loop_filter_across_bricks_enabled_flag is equal to 0.

    - If the bottom boundary of the current coding tree block is the bottom boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If  CtbSizeY - x is less than 3 and the location  (xCtb + CtbSizeY + 1, yCtb + CtbSizeY +1) location does not belong to the current slice, the right boundary of the current coding tree block is not the right boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - If the bottom boundary of the current coding tree block is the bottom boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team, JVET-O2001

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).