| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0159-v1 |
| --- | --- |

| Title: | CE5-related: DMVR deblocking by inheriting neighbouring boundary strength values |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to apply luma deblocking to the coding unit (CU) coded with decoder-side motion vector refinement (DMVR) mode. For a CU coded with DMVR, if two sides of the current edge that corresponds to internal prediction block (PB) or transform block (TB) boundaries have all-zero residual, the bS value is inherited from the left or top coding block (CB) boundaries. Two methods based on the bS value inheritance are tested in this contribution. From the subjective analysis results, the proposed methods improve the subjective quality of coded sequences when compared to the anchor (VTM6.0). Objective results of the proposed methods are reported as follows:

[Method 1] DMVR deblocking by inheriting the bS values from the left or top CB boundaries
Under ALF on:
VTM6.0-RA: {Y-BD-rate=0.0X%, EncT=10X%, DecT=10X%}
Under ALF off:
VTM6.0-RA: {Y-BD-rate=0.0X%, EncT=10X%, DecT=10X%}

[Method 2] Method 1 + set bS to 0 if unrefined motion vector (MV) is similar to previous merging candidates
Under ALF on:
VTM6.0-RA: {Y-BD-rate=0.0X %, EncT=10X%, DecT=10X%}
Under ALF off:
VTM6.0-RA: {Y-BD-rate=0.0X%, EncT=10X%, DecT=10X%}

# Introduction

In VTM6.0 [1], since refined MVs cannot be used for the deblocking processes, once a CU is coded with DMVR, no deblocking is applied to internal PB boundaries. However, block artifact of DMVR coded CU is visible especially at high motion activity regions, as shown in Figure 1, where the DMVR coded CUs are marked in green and block artifacts are visible at 16x16 internal PB boundaries that do not coincide with CB or TB boundaires.

Figure 1. The 9th decoded picture of BaseketballDrill test sequence, VTM6.0 with RA configuration, QP=37, DMVR coded CBs are marked in green.

# Proposed method

To apply luma deblocking to DMVR coded CUs, the following two methods are proposed. Method 1 includes the following.

- The deblocking grid for internal PB boundaries is changed to 16x16 basis.

- If both sides of the edge coincided with internal PB or TB boundaries have all-zero residual, the bS for luma is inherited from the top or left CB boundaries, as shown in Figure 2. Otherwise (i.e., at least one side has nonzero residual), bS for luma is set to 1.

- The bS value is set to 1 if the inherited bS value is greater than 1.

- If the edge coincides with TB boundaries, the maximum filter length could be up to 7. Otherwise, the maximum filter length is set to 1 (i.e., only weak filtering decisions and operations are performed, and the maximum number of samples to-be-modified is one).

Based on Method 1, Method 2 adds one more constraint as follows.

- When both sides of the edge coincided with internal PB or TB boundaries have all-zero residual, the bS is set to 0 if the unrefined MV difference between the current selected merging candidate and any of the previous merging candidates in the same merging candidate list is smaller than 16 (in unit of 1/16 luma sample).

| (a) | (b) |
| --- | --- |

Figure 2. (a) An example of inheriting bS values to the internal horizontal edges from the immediately above edges that coincide with CB boundary. (b) Another example of inheriting bS values to the internal vertical edges from the immediately left edges that coincide with CB boundary. Note, bSxi~j is denoted as the bS values of the i-th above edge to the j-th above edge that coincides with CB boundary, and bSyi~j is denoted as the bS values of the i-th left edge to the j-th above edge that coincides with CB boundary.

# Experimental results

The proposed methods are implemented on top of VTM6.0 [2], and the simulation is performed under common test condition (CTC) as described in JVET-N1010 [3]. Table 1 and Table 2 show the results of Method 1 and Method 2, respectively. Both cases under ALF on and ALF off are shown.

Table 1.  Objective results of Method 1 on top of VTM6.0 (to be updated)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  | Over VTM6.0 with ALF switch off |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.11% | 0.05% | 0.02% | 101% | 0.11% |  |  |  |  |  |
| Class C | 0.09% | 0.00% | 0.02% | 101% | 0.09% |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.06% | 0.00% | 0.00% | 101% | 0.06% |  |  |  |  |  |
| Class F | 0.07% | 0.00% | -0.02% | 100% | 0.07% |  |  |  |  |  |

Table 2.  Objective results of Method 2 on top of VTM6.0 (to be updated)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  | Over VTM6.0 with ALF switch off |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | 0.05% | -0.04% | 0.03% | 101% | 0.05% |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.02% | 0.01% | 0.04% | 101% | 0.02% |  |  |  |  |  |
| Class F | 0.07% | 0.01% | -0.01% | 100% | 0.07% |  |  |  |  |  |

# Subjective evaluation

To illustrate the benefit of proposed methods, two examples are shown in the following.

| (a) | (b) |
| --- | --- |
| (c) |  |

Figure 3. BaskeballDrill test sequence, RA, QP 37, POC #9, (a) VTM6.0; (b) VTM6.0 + Method 2; (c) blocks marked in green are coded with DMVR.

| (a) | (b) |
| --- | --- |
| (c) |  |

Figure 4. FoodMarket test sequence, RA, QP 37, POC #39, (a) VTM6.0; (b) VTM6.0 + Method 2; (c) blocks marked in green are coded with DMVR.

# Conclusion

This contribution presents the results for DMVR deblocking by inheriting bS values form left or top CB boundaries. Subjective analysis shows that when compared to the anchor (VTM6.0), the proposed methods improve the subjective quality of coded sequences. It is suggested to adopt this technique to the VVC draft specification and the VTM reference software.

# References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team, JVET-O1001.

- VTM6.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

- C.-Y. Chen and A. Norkin, “Description of Core Experiment 5 (CE5): Loop filtering,” Document of Joint Video Experts Team, JVET-O2025.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).