| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0160-v1 |
| --- | --- |

| Title: | CE5-related: Unification of deblocking processes for transform block and prediction block boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to unify the luma deblocking filter processes for transform block (TB) and prediction block (PB) boundaries. Two methods are proposed. In Method 1, the deblocking grid basis for TB and PB boundaries is unified to 8x8 grid basis. In Method 2, when performing boundary strength (bS) calculation, the motion-related checks are performed only for the edges coincided with 8x8 grids inside the coding unit (CU) while the coefficient-related checks are kept unchanged. Objective results of the proposed methods are reported as follows:

[Method 1] Deblocking grid basis for TB and PB boundaries is unified to 8x8 grid basis
Under ALF on:
VTM6.0-RA: {Y-BD-rate = -0.01%, EncT = 100%, DecT = 100%}
VTM6.0-LB: {Y-BD-rate = -0.01%, EncT = 100%, DecT = 99%}
VTM6.0-LP: {Y-BD-rate = 0.03%, EncT = 100%, DecT = 99%}
Under ALF off:
VTM6.0-RA: {Y-BD-rate = 0.01%, EncT = 99%, DecT = 100%}
VTM6.0-LB: {Y-BD-rate = -0.04%, EncT = 99%, DecT = 98%}
VTM6.0-LP: {Y-BD-rate = -0.05%, EncT = 99%, DecT = 96%}

[Method 2] Motion-related checks in bS calculation are performed only for the edges coincided with 8x8 grids inside the CU
Under ALF on:
VTM6.0-RA: {Y-BD-rate = 0.0X%, EncT = 10X%, DecT = 10X%}
VTM6.0-LB: {Y-BD-rate = 0.0X%, EncT = 10X%, DecT = 10X%}
VTM6.0-LP: {Y-BD-rate = 0.0X%, EncT = 10X%, DecT = 10X%}
Under ALF off:
VTM6.0-RA: {Y-BD-rate = 0.0X%, EncT = 10X%, DecT = 10X%}
VTM6.0-LB: {Y-BD-rate = 0.0X%, EncT = 10X%, DecT = 10X%}
VTM6.0-LP: {Y-BD-rate = 0.0X%, EncT = 10X%, DecT = 10X%}

# Introduction

In VVC Draft 6 [1], the luma deblocking filter process is applied on 4x4 grids for coding block (CB) and TB boundaries, and on an 8x8 grids for PB boundaries. However, as mentioned in JVET-P0043 [2], different deblocking grid basis for TB and PB boundaries could cause inconsistency. To resolve the inconsistency issue, two methods are proposed. More details are described in the following sections.

# Proposed methods

To unify the luma deblocking filter processes for TB and PB boundaries, the following two methods are proposed.

## Method 1

The deblocking grid basis for TB and PB boundaries is unified to 8x8 grid basis. To be more specific, given an inter coded CU, luma deblocking processes of the internal TB or PB boundaries are performed only when the boundaries are coincided with 8x8 grids inside the CU.

## Method 2

The deblocking grid basis for TB and PB boundaries are not changed, but when performing bS calculation, the motion-related checks are performed only for the edges coincided with 8x8 grids inside the CU. The coefficient-related checks are kept unchanged.

# Experimental results

The proposed methods are implemented on top of VTM6.0 [3], and the simulation is performed under CTC as described in JVET-N1010 [4]. Table 1, and Table 2 show the results of Method 1 and Method 2, respectively. Both cases under ALF on and ALF off [5] are shown.

Table 1.  Objective results of Method 1 on top of VTM6.0

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  | Over VTM6.0 with ALF switch off |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.04% | 0.04% | 100% | 100% | 0.03% | 0.06% | -0.03% | 99% | 100% |
| Class A2 | 0.01% | 0.06% | 0.05% | 100% | 101% | 0.01% | 0.06% | 0.03% | 99% | 99% |
| Class B | -0.01% | 0.03% | 0.00% | 100% | 100% | 0.01% | -0.02% | -0.04% | 99% | 99% |
| Class C | -0.02% | 0.02% | 0.01% | 100% | 100% | 0.00% | 0.00% | -0.06% | 99% | 103% |
| Overall | -0.01% | 0.03% | 0.02% | 100% | 100% | 0.01% | 0.02% | -0.03% | 99% | 100% |
| Class D | -0.02% | -0.06% | 0.00% | 100% | 100% | -0.01% | 0.03% | -0.07% | 99% | 102% |
| Class F | -0.01% | 0.00% | -0.01% | 100% | 100% | 0.00% | -0.03% | 0.04% | 99% | 102% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  | Over VTM6.0 with ALF switch off |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | 0.00% | -0.02% | -0.03% | 100% | 99% | -0.02% | -0.08% | -0.23% | 100% | 100% |
| Class C | -0.03% | 0.10% | 0.05% | 100% | 100% | 0.00% | 0.06% | 0.09% | 99% | 95% |
| Class E | -0.01% | 0.42% | 0.38% | 100% | 98% | -0.11% | -0.12% | 0.01% | 99% | 98% |
| Overall | -0.01% | 0.13% | 0.10% | 100% | 99% | -0.04% | -0.05% | -0.06% | 99% | 98% |
| Class D | -0.04% | 0.23% | 0.21% | 100% | 99% | 0.01% | 0.27% | 0.22% | 100% | 92% |
| Class F | 0.07% | -0.11% | 0.43% | 100% | 100% | 0.02% | 0.45% | 0.07% | 98% | 97% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  | Over VTM6.0 with ALF switch off |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | -0.01% | -0.13% | 0.17% | 100% | 99% | -0.01% | -0.07% | 0.27% | 99% | 99% |
| Class C | 0.04% | 0.18% | 0.17% | 100% | 99% | -0.04% | -0.12% | 0.11% | 99% | 93% |
| Class E | 0.10% | 0.13% | 0.13% | 100% | 99% | -0.12% | -0.51% | -0.21% | 99% | 94% |
| Overall | 0.03% | 0.04% | 0.16% | 100% | 99% | -0.05% | -0.19% | 0.10% | 99% | 96% |
| Class D | 0.08% | -0.22% | 0.42% | 100% | 99% | 0.00% | 0.17% | 0.09% | 100% | 95% |
| Class F | -0.14% | -0.16% | 0.23% | 100% | 100% | -0.02% | 0.09% | -0.25% | 98% | 94% |

Table 2.  Objective results of Method 2 on top of VTM6.0 (to be updated)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  | Over VTM6.0 with ALF switch off |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  | Over VTM6.0 with ALF switch off |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  | Over VTM6.0 with ALF switch off |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

# Conclusion

In this contribution, two methods are proposed to unify the luma deblocking filter processes for TB and PB boundaries. It is suggested to adopt one of these methods into the VVC draft specification and the VTM reference software.

# References

- B. Bross, J. Chen, and S. Liu, "Versatile Video Coding (Draft 6)," Document of Joint Video Experts Team, JVET-O1001.

- B. Heng and M. Zhou, “AHG16/Non-CE5: A cleanup for de-blocking in the affine and TPM mode,” Document of Joint Video Experts Team, JVET-P0043.

- VTM6.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

- C.-Y. Chen and A. Norkin, "Description of Core Experiment 5 (CE5): Loop filtering," Document of Joint Video Experts Team, JVET-O2025.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).