| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0162-v1 |
| --- | --- |

| Title: | CE5-related: Simplified ALF syntax with removal of alf_ctb_use_first_aps_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Ling Hsiao, Chun-Chia Chen, Olena Chubach, Chih-Wei Hsu, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yuling.hsiao, chunchia.chen, olena.chubach, cw.hsu, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the signalling of adaptive loop filter (ALF) coding tree block (CTB) syntax for luma component. Compared with VTM6.0, alf_ctb_use_first_aps_flag, to indicate whether a luma CTB uses the filter set from the first adaptation parameter set (APS), is removed, and related parsing processes are simplified. Results are summarized as follows.

Proposed method
VTM6.0-AI: {Y-BD-rate=0.00%, EncT=100%, DecT=100%}
VTM6.0-RA: {Y-BD-rate=0.01%, EncT=100%, DecT=100%}
VTM6.0-LB: {Y-BD-rate=-0.03%, EncT=100%, DecT=99%}

# Introduction

In Versatile Video Coding (VVC) Draft 6 [1], the signalling of ALF CTB syntax for luma component is unnecessarily complicated. As shown in Table 1, the alf_ctb_use_first_aps_flag is signalled first to indicate whether the luma CTB uses the filter set from the first APS (i.e., the APS with adaptive_parameter_set_id equal to slice_alf_aps_id_luma[ 0 ]). If the alf_ctb_use_first_aps_flag is false, the alf_use_aps_flag is signalled to indicate whether the luma CTB selects a filter set from a non-first APS or a fixed filter set. When the alf_use_aps_flag is true and the current slice references more than two APSes, the alf_luma_prev_filter_idx_minus1 is signalled to indicate which non-first APS is selected for the filter set of the current luma CTB; otherwise, the alf_luma_fixed_filter_idx is signalled to indicate which fixed filter set is applied to the current luma CTB.

It is proposed to simplify the ALF CTB syntax by removing syntax element alf_ctb_use_first_aps_flag. Details are described in the following sections.

Table 1. ALF CTB syntax for luma component in VVC Draft 6

| alf_ctb_flag[ 0 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| --- | --- |
| if( alf_ctb_flag[ 0 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] ) { |  |
| if( slice_num_alf_aps_ids_luma > 0 ) |  |
| alf_ctb_use_first_aps_flag | ae(v) |
| if( !alf_ctb_use_first_aps_flag ) { |  |
| if( slice_num_alf_aps_ids_luma > 1 ) |  |
| alf_use_aps_flag | ae(v) |
| if( alf_use_aps_flag ) { |  |
| if( slice_num_alf_aps_ids_luma > 2 ) |  |
| alf_luma_prev_filter_idx_minus1 | ae(v) |
| } else |  |
| alf_luma_fixed_filter_idx | ae(v) |
| } |  |
| } |  |

# Proposed method

The alf_ctb_use_first_aps_flag is removed, as shown in Table 2. The alf_use_aps_flag is signalled first. If alf_use_aps_flag is true and the current slice references more than one APS, alf_luma_prev_filter_idx is signalled to indicate which APS is selected for the filter set of the current luma CTB; otherwise, alf_luma_fixed_filter_idx is signalled to indicate which fixed filter set is applied to the current luma CTB.

Table 2. Proposed ALF CTB syntax for luma component

| alf_ctb_flag[ 0 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| --- | --- |
| if( alf_ctb_flag[ 0 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] ) { |  |
| if( slice_num_alf_aps_ids_luma > 0 ) |  |
| alf_use_aps_flag | ae(v) |
| if( alf_use_aps_flag ) { |  |
| if( slice_num_alf_aps_ids_luma > 1 ) |  |
| alf_luma_prev_filter_idx | ae(v) |
| } else |  |
| alf_luma_fixed_filter_idx | ae(v) |
| } |  |

# Experiment results

The proposed method has been implemented on top of VTM6.0. The common test conditions [2] are used for evaluation. Table 3 summarizes the results of the proposed method.

Table 3. Results of the proposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.01% | 0.01% | 101% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.16% | -0.06% | 100% | 101% |
| Class A2 | 0.05% | 0.08% | 0.09% | 100% | 100% |
| Class B | 0.00% | 0.07% | 0.00% | 100% | 100% |
| Class C | -0.01% | -0.06% | -0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.01% | 0.00% | 100% | 100% |
| Class D | -0.02% | 0.02% | 0.01% | 101% | 100% |
| Class F | 0.00% | 0.00% | -0.03% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.04% | -0.02% | 100% | 99% |
| Class C | -0.04% | 0.10% | -0.14% | 100% | 99% |
| Class E | -0.09% | 0.34% | 0.52% | 100% | 98% |
| Overall | -0.03% | 0.14% | 0.07% | 100% | 99% |
| Class D | 0.00% | 0.50% | 0.10% | 101% | 100% |
| Class F | 0.00% | 0.01% | 0.40% | 100% | 99% |

# Conclusion

In this contribution, it is proposed to simplify the ALF CTB syntax for luma component by removing syntax element alf_ctb_use_first_aps_flag. Compared with VTM6.0, the BD rate change is minor.

# References

[1] B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team, JVET-O2001.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).