| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0166-v2 |
| --- | --- |

| Title: | CE6-related: Transform selection with intra prediction mode for implicit MTS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Man-Shu Chiang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {man-shu.chiang, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes that for an intra luma coding block, the intra prediction mode is used to select the primary transform when explicit multi-transform set (MTS) is disabled and implicit MTS is enabled. The current design is to use the transform block width and height to decide the transform types for implicit MTS. Two variations are proposed. Compared to VTM6.0 with explicit MTS disabled and implicit MTS enabled, the results are summarized as follows.

Variation 1: intra prediction mode with an even number uses DST-VII
AI: BD-rates = -0.06% (Y), -0.13% (Cb), -0.11% (Cr); EncT=99%; DecT=100%
RA: BD-rates = -0.05% (Y), -0.14% (Cb), -0.03% (Cr); EncT=100%; DecT=101%
LB: BD-rates = -0.05% (Y), -0.19% (Cb), -0.02% (Cr); EncT=99%; DecT=101%

Variation 2: intra prediction mode under specified conditions uses DST-VII
AI: BD-rates = -0.04% (Y), 0.11% (Cb), 0.13% (Cr)
RA: BD-rates = 0.00% (Y), 0.02% (Cb), 0.06% (Cr)
LB: BD-rates = 0.0X% (Y), 0.0X% (Cb), 0.0X% (Cr)

# Introduction

In VTM6.0, a unification scheme between intra subpartition (ISP) and non-ISP intra modes was adopted to select the transform types for implicit MTS. The scheme is stated as follows.

The default transform type for horizontal transform and vertical transform is DCT-II.

If the transform block width is within [4, 16], the horizontal transform is set to DST-VII.

If the transform block height is within [4, 16], the vertical transform is set to DST-VII.

# Proposed methods

The default primary transform type for horizontal transform and vertical transform is kept as DCT-II without change. When explicit MTS is disabled and implicit MTS is enabled, for an intra luma CB, it is proposed to set the primary transform type to DST-VII according to the intra prediction mode. Using intra mode for adjusting MTS was commented as an additional burden for the transform stage. However, in the VTM6.0 low frequency non-separable secondary transform (LFNST), intra prediction mode is already used to decide the transform set index. Therefore, using intra prediction mode at the transform stage has been agreed and regarded as an acceptable cost. Two variations are proposed as follows.

Variation 1:

If the transform block width (or height) is smaller than or equal to 32 and the intra prediction mode is an even number, the horizontal (or vertical) transform is set to DST-VII.

Variation 2:

If the transform block width (or height) is smaller than or equal to 16 and the followings are applied.

- If the intra prediction mode is equal to planar, the horizontal and vertical transform are set to DST-VII.

- Otherwise if the intra prediction mode is within [31, 37] and not equal to 33 or 35, the horizontal and vertical transform are set to DST-VII.

- Otherwise if the intra prediction mode is an even (or odd) number and the intra prediction mode is smaller than 31, the horizontal (or vertical) transform is set to DST-VII.

- Otherwise if the intra prediction mode is an odd (or even) number and the intra prediction mode is larger than 37, the horizontal (or vertical) transform is set to DST-VII.

# Experimental results

Table 1 shows the results of implicit MTS (VTM6.0 [1] with explicit MTS disabled and implicit MTS enabled) and the anchor is VTM6.0 with both explicit and implicit MTS disabled. The proposed two variations are implemented on top of VTM6.0. Table 2 and Table 3 show the results of variation 1 following the test condition 1 and test condition 2, respectively. Table 4 and Table 5 show the results of variation 2 following the test condition 1 and test condition 2, respectively. Table 6 shows the results of explicit MTS, i.e., VTM6.0 under common test condition (CTC) [2], and the anchor is VTM6.0 with both explicit and implicit MTS disabled. The two test conditions are described as follows.

[Test condition 1]
Anchor: VTM6.0 with both explicit and implicit MTS disabled
Test: The proposed variation with explicit MTS disabled and implicit MTS enabled

[Test condition 2]
Anchor: VTM6.0 under CTC
Test: The proposed variation under CTC

Table 1.  Results of implicit MTS (VTM6.0 with explicit MTS disabled and implicit MTS enabled), anchor: VTM6.0 with both explicit and implicit MTS disabled

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.95% | -0.67% | -0.63% | 104% | 103% |
| Class A2 | -1.11% | -1.14% | -1.19% | 104% | 107% |
| Class B | -0.99% | -1.32% | -1.30% | 104% | 100% |
| Class C | -0.65% | -0.59% | -0.81% | 104% | 102% |
| Class E | -1.36% | -1.37% | -1.45% | 103% | 103% |
| Overall | -0.99% | -1.03% | -1.08% | 104% | 103% |
| Class D | -0.53% | -0.55% | -0.56% | 102% | 98% |
| Class F | -0.30% | -0.43% | -0.44% | 103% | 99% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.43% | -0.13% | -0.43% | 101% | 102% |
| Class A2 | -0.59% | -0.34% | -0.16% | 101% | 102% |
| Class B | -0.74% | -0.84% | -0.94% | 102% | 101% |
| Class C | -0.53% | -0.48% | -0.42% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.59% | -0.50% | -0.54% | 101% | 101% |
| Class D | -0.49% | -0.23% | -0.14% | 101% | 102% |
| Class F | -0.32% | -0.22% | -0.28% | 101% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.62% | -0.41% | -0.23% | 100% | 102% |
| Class C | -0.40% | -0.18% | -0.13% | 102% | 101% |
| Class E | -0.22% | -0.07% | -0.47% | 102% | 101% |
| Overall | -0.45% | -0.25% | -0.26% | 101% | 101% |
| Class D | -0.44% | 0.00% | 0.12% | 101% | 100% |
| Class F | -0.12% | -0.12% | -0.81% | 99% | 99% |

Table 2.  Results of the proposed variation 1 under test condition 1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.08% | -1.08% | -1.09% | 103% | 107% |
| Class A2 | -1.11% | -1.29% | -1.34% | 103% | 105% |
| Class B | -1.10% | -1.48% | -1.40% | 102% | 99% |
| Class C | -0.71% | -0.60% | -0.75% | 104% | 103% |
| Class E | -1.29% | -1.29% | -1.43% | 105% | 101% |
| Overall | -1.04% | -1.16% | -1.20% | 103% | 103% |
| Class D | -0.59% | -0.61% | -0.50% | 101% | 97% |
| Class F | -0.27% | -0.44% | -0.45% | 101% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.53% | -0.56% | -0.71% | 101% | 105% |
| Class A2 | -0.65% | -0.47% | -0.26% | 100% | 102% |
| Class B | -0.78% | -0.96% | -0.87% | 101% | 102% |
| Class C | -0.53% | -0.45% | -0.32% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.64% | -0.65% | -0.57% | 101% | 102% |
| Class D | -0.45% | -0.05% | -0.21% | 101% | 100% |
| Class F | -0.35% | -0.26% | -0.31% | 100% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.67% | -0.85% | -0.79% | 100% | 104% |
| Class C | -0.44% | -0.14% | -0.16% | 101% | 101% |
| Class E | -0.31% | -0.18% | 0.38% | 100% | 104% |
| Overall | -0.50% | -0.44% | -0.29% | 100% | 103% |
| Class D | -0.41% | -0.07% | -0.51% | 101% | 103% |
| Class F | -0.18% | -0.21% | 0.02% | 100% | 101% |

Table 3.  Results of the proposed variation 1 under test condition 2

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | 0.04% | 100% | 102% |
| Class A2 | 0.04% | 0.08% | 0.03% | 99% | 96% |
| Class B | 0.03% | 0.07% | 0.06% | 99% | 98% |
| Class C | 0.02% | -0.08% | 0.08% | 98% | 98% |
| Class E | 0.06% | 0.11% | 0.15% | 99% | 101% |
| Overall | 0.03% | 0.04% | 0.07% | 99% | 99% |
| Class D | -0.01% | -0.08% | -0.06% | 98% | 102% |
| Class F | 0.06% | 0.10% | 0.00% | 98% | 96% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.16% | -0.03% | 101% | 110% |
| Class A2 | 0.03% | 0.03% | 0.07% | 100% | 105% |
| Class B | 0.00% | 0.05% | 0.02% | 100% | 104% |
| Class C | 0.01% | 0.17% | 0.10% | 100% | 105% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.10% | 0.04% | 100% | 106% |
| Class D | -0.01% | 0.03% | -0.04% | 100% | 105% |
| Class F | 0.02% | -0.09% | 0.06% | 100% | 103% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Table 4.  Results of the proposed variation 2 under test condition 1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.90% | -0.54% | -0.51% | 103% | 102% |
| Class A2 | -1.10% | -1.01% | -1.03% | 103% | 102% |
| Class B | -1.07% | -1.14% | -1.09% | 104% | 103% |
| Class C | -0.75% | -0.62% | -0.83% | 103% | 102% |
| Class E | -1.38% | -1.23% | -1.28% | 103% | 102% |
| Overall | -1.03% | -0.92% | -0.96% | 103% | 102% |
| Class D | -0.69% | -0.61% | -0.45% | 103% | 101% |
| Class F | -0.41% | -0.37% | -0.39% | 100% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.40% | -0.20% | -0.33% | 101% | 102% |
| Class A2 | -0.58% | -0.36% | -0.23% | 101% | 101% |
| Class B | -0.74% | -0.73% | -0.88% | 101% | 101% |
| Class C | -0.54% | -0.46% | -0.30% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.59% | -0.48% | -0.48% | 101% | 102% |
| Class D | -0.55% | -0.03% | -0.50% | 101% | 103% |
| Class F | -0.35% | -0.31% | -0.28% | 101% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Table 5.  Results of the proposed variation 2 under test condition 2

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.01% | 100% | 100% |
| Class A2 | 0.02% | 0.07% | 0.03% | 101% | 100% |
| Class B | 0.01% | 0.07% | 0.06% | 100% | 102% |
| Class C | 0.00% | -0.09% | 0.04% | 100% | 100% |
| Class E | 0.02% | 0.18% | 0.19% | 101% | 101% |
| Overall | 0.01% | 0.04% | 0.07% | 100% | 101% |
| Class D | -0.02% | 0.09% | -0.03% | 100% | 102% |
| Class F | -0.02% | 0.03% | -0.04% | 99% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.07% | 0.01% | 100% | 100% |
| Class A2 | 0.03% | -0.01% | -0.03% | 101% | 100% |
| Class B | 0.00% | 0.12% | 0.07% | 101% | 100% |
| Class C | 0.01% | -0.04% | -0.05% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.02% | 0.01% | 101% | 101% |
| Class D | -0.03% | -0.30% | 0.04% | 101% | 101% |
| Class F | -0.04% | -0.11% | 0.10% | 100% | 103% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Table 6.  Results of MTS (VTM6.0 under CTC), anchor: VTM6.0 with both explicit and implicit MTS disabled

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.24% | -0.73% | -0.74% | 115% | 101% |
| Class A2 | -1.36% | -1.00% | -0.99% | 118% | 109% |
| Class B | -1.41% | -0.96% | -0.83% | 119% | 99% |
| Class C | -0.79% | -0.06% | -0.35% | 121% | 103% |
| Class E | -1.42% | -0.90% | -0.98% | 118% | 101% |
| Overall | -1.24% | -0.72% | -0.76% | 119% | 102% |
| Class D | -0.70% | -0.04% | 0.05% | 122% | 98% |
| Class F | -0.43% | -0.15% | -0.08% | 109% | 103% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.60% | -0.63% | -0.89% | 106% | 101% |
| Class A2 | -0.72% | -0.46% | -0.16% | 107% | 100% |
| Class B | -0.85% | -0.87% | -0.81% | 108% | 101% |
| Class C | -0.56% | -0.40% | -0.29% | 110% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.70% | -0.61% | -0.56% | 108% | 100% |
| Class D | -0.48% | 0.12% | -0.13% | 110% | 101% |
| Class F | -0.48% | -0.15% | -0.23% | 108% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.72% | -0.80% | -0.41% | 106% | 101% |
| Class C | -0.41% | -0.03% | -0.03% | 105% | 100% |
| Class E | -0.50% | -0.43% | -0.81% | 102% | 100% |
| Overall | -0.56% | -0.45% | -0.38% | 105% | 100% |
| Class D | -0.41% | 0.01% | -0.26% | 104% | 100% |
| Class F | -0.26% | -0.35% | -0.93% | 102% | 97% |

# Conclusion

This contribution proposes to improve the implicit MTS by using intra prediction mode for transform selections. There are two variations with different transform selection methods by intra prediction modes. Compared to VTM6.0 with explicit MTS disabled and implicit MTS enabled, the proposed methods provide slight BD-rate savings. Compared to VTM6.0 CTC (using explicit MTS), the proposed implicit MTS can preserve most of the gains of explicit MTS and achieve 12% and 6% encoding time savings for AI and RA, respectively.

# References

[1]    VTM6.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

[2]    F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).