| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0169-v2 |
| --- | --- |

| Title: | CE7-related: Further context reduction for sig_coeff_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Tzu-Der Chuang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, peter.chuang, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two modified methods for entropy coding the syntax element sig_coeff_flag. In the proposed methods, the context reduction scheme in CE7-3.1 is applied to more subsets for further reducing the number of context variables. The proposed Method 1 reportedly provides 0.03%, 0.01%, and -0.02% luma BD-rates for AI, RA, and LB, respectively, versus the VTM6.0 anchor under the common test conditions (CTCs) while achieving reduction of context variables by 20. The proposed Method 2 reportedly provides -0.01%, 0.00%, and 0.00% luma BD-rates for AI, RA, and LB, respectively, versus the VTM6.0 anchor under the CTCs while achieving reduction of context variables by 24.

# Introduction

In CE7-3.1 [1], context reduction for entropy coding the syntax element sig_coeff_flag is achieved by reducing the numbers of the context variables associated with dependent quantization states 2 and 3. In the CE tests, the number of the context variables in a context subset corresponding to a transform block zone is reduced to 2 from 4 for some context subsets. In the subset, context selection is determined by the sum of absolute values of partialy reconstructed coefficient levels from a local template, indicated by variable locSumAbsPass1. The 1st context variable is selected when the reconstructed variable locSumAbsPass1 is equal to 0. The 2nd context variable is selected, otherwise. Context reduction is applied to 6 context subsets in CE7-3.1a and to 9 context subsets in CE7-3.1b, respectively.

# Proposed methods

In this contribution, context reduction is further applied to other subbsets for further reducing the number of context variables and simplifying the process for deriving the selected context index.

In Method 1, context reduction is applied to all subsets associated with dependent quantization states 2 and 3 and leads to context reduction by 20, as illustrated in Figure 1 (a). The context increment variable ctxInc for coding sig_coeff_flag at the current coefficient scan location (xC, yC) with colour component index cIdx and dependent quantization state QState is derived as follows:

- The variable d is set equal to xC + yC. The variable dIdx is derived as follows:

dIdx = ( d < 2 ? 1 : 0) + ( (cIdx == 0 && d < 5 ) ? 1 : 0 )

- If QState is less than or equal to 1, ctxInc is derived as follows:

ctxInc = (cIdx == 0 ? 0 : 24 ) + Min( ( locSumAbsPass1 + 1 ) >> 1, 3 ) + 4 * dIdx

- Otherwise, ctxInc is derived as follows:

ctxInc = (cIdx == 0 ? ( 12 + 6 * ( QState − 2 )) : ( 32 + 4 * ( QState − 2 ))) + ( locSumAbsPass1 == 0 ?  0 : 1 ) + 2 * dIdx

In Method 2, context reduction is applied to all subsets associated with dependent quantization states 2 and 3 or associated with chroma components and leads to context reduction by 24, as illustrated in Figure 1 (b). The context increment variable ctxInc for coding sig_coeff_flag at the current coefficient scan location (xC, yC ) with colour component index cIdx and dependent quantization state QState is derived as follows:

- The variable d is set equal to xC + yC. The variable dIdx is derived as follows:

dIdx = ( d < 2 ? 1 : 0) + ( (cIdx == 0 && d < 5 ) ? 1 : 0 )

- If QState is less than or equal to 1 and cIdx is equal to 0, ctxInc is derived as follows:

ctxInc = Min( ( locSumAbsPass1 + 1 ) >> 1, 3 ) + 4 * dIdx

- Otherwise, ctxInc is derived as follows:

ctxInc = ( cIdx == 0 ? ( 12 + 6 * ( QState − 2 )) : ( 24 + 4 * ( QState − 1 ))) +

( locSumAbsPass1 == 0 ?  0 : 1 ) + 2 * dIdx

2

2

2

2

2

Qstate = 3

2

2

2

2

2

Qstate = 2

4

4

4

4

4

Qstate = 0 or 1

2

2

2

2

2

Qstate = 3

2

2

2

2

2

Qstate = 2

4

4

4

2

2

Qstate = 0 or 1

- Method 1

(b) Method 2

Figure 1: Illustration of the tests for context reduction for coding the syntax element sig_coeff_flag in a transform block, where the number of the context variables is reduced from 4 to 2 for some context subsets.

# Experimental results

The proposed methods were integrated into the VTM6.0 reference software [2]. The coding results generated by the proposed methods were evaluated against the anchor results generated by the unmodified VTM6.0 reference software under the CE7 test conditions [3]. Tables 1 and 2 summarize the experimental results for Method 1 and Method 2, respectively, conducted under the CTCs over standard QPs 22, 27, 32, and 37. Tables 3 and 4 summarize the experimental results for Method 1 and Method 2, respectively, conducted over low QPs 2, 7, 12, and 17. Please note the run time results may not be accurate in the tables. The detailed simulation results can be found in the attached spreadsheets.

Table 1: Summary of the average BD bitrate results for Method 1 over standard QPs 22, 27, 32, and 37

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.03% | 0.06% | 101% | 102% |
| Class A2 | 0.04% | -0.06% | -0.07% | 102% | 103% |
| Class B | 0.02% | -0.02% | -0.05% | 101% | 102% |
| Class C | 0.02% | -0.10% | 0.01% | 100% | 99% |
| Class E | 0.01% | 0.04% | 0.10% | 97% | 99% |
| Overall | 0.03% | -0.04% | 0.00% | 100% | 101% |
| Class D | -0.01% | 0.04% | 0.06% | 98% | 96% |
| Class F | 0.01% | -0.03% | 0.01% | 96% | 96% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.07% | 0.00% | 104% | 102% |
| Class A2 | 0.01% | 0.19% | 0.06% | 105% | 101% |
| Class B | 0.00% | 0.04% | 0.09% | 102% | 98% |
| Class C | 0.01% | 0.01% | 0.00% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.04% | 0.04% | 102% | 99% |
| Class D | -0.06% | -0.31% | 0.21% | 99% | 103% |
| Class F | -0.03% | 0.02% | 0.09% | 100% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.10% | -0.15% | 101% | 96% |
| Class C | -0.01% | -0.15% | -0.05% | 106% | 105% |
| Class E | -0.07% | 0.22% | 0.59% | 104% | 104% |
| Overall | -0.02% | 0.05% | 0.07% | 103% | 101% |
| Class D | 0.05% | 0.00% | 0.63% | 105% | 108% |
| Class F | 0.10% | 0.12% | 0.35% | 100% | 100% |

Table 2: Summary of the average BD bitrate results for Method 2 over standard QPs 22, 27, 32, and 37)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.06% | 0.14% | 101% | 102% |
| Class A2 | 0.03% | 0.08% | 0.07% | 102% | 104% |
| Class B | 0.02% | 0.07% | 0.00% | 102% | 104% |
| Class C | 0.01% | 0.05% | 0.24% | 101% | 100% |
| Class E | 0.02% | 0.10% | 0.13% | 98% | 99% |
| Overall | 0.02% | 0.07% | 0.11% | 101% | 102% |
| Class D | 0.00% | 0.10% | 0.13% | 98% | 96% |
| Class F | 0.01% | 0.00% | 0.06% | 98% | 97% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.22% | 0.06% | 105% | 105% |
| Class A2 | 0.03% | 0.12% | 0.01% | 104% | 102% |
| Class B | -0.02% | 0.12% | -0.03% | 102% | 99% |
| Class C | 0.03% | 0.20% | -0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.16% | -0.01% | 102% | 101% |
| Class D | -0.06% | -0.31% | -0.05% | 99% | 105% |
| Class F | -0.04% | 0.10% | 0.15% | 99% | 108% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.12% | -0.24% | 103% | 101% |
| Class C | -0.01% | -0.03% | 0.08% | 101% | 101% |
| Class E | -0.13% | 0.02% | 0.90% | 101% | 100% |
| Overall | -0.03% | 0.05% | 0.15% | 102% | 101% |
| Class D | -0.02% | -0.25% | 0.48% | 98% | 102% |
| Class F | -0.05% | -0.22% | 0.40% | 97% | 100% |

Table 3: Summary of the average BD bitrate results for Method 1 over low QPs 2, 7, 12, and 17

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.08% | 0.07% | 95% | 91% |
| Class A2 | -0.02% | 0.07% | 0.08% | 97% | 95% |
| Class B | 0.02% | 0.02% | 0.07% | 100% | 100% |
| Class C | 0.01% | 0.04% | 0.08% | 100% | 100% |
| Class E | 0.02% | 0.01% | 0.06% | 100% | 101% |
| Overall | 0.01% | 0.04% | 0.07% | 99% | 98% |
| Class D | 0.02% | 0.00% | 0.05% | 100% | 100% |
| Class F | 0.03% | 0.02% | 0.01% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.02% | 99% | 100% |
| Class A2 | -0.01% | 0.02% | 0.04% | 101% | 100% |
| Class B | 0.02% | -0.02% | 0.02% | 99% | 102% |
| Class C | 0.01% | 0.00% | 0.06% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | 0.03% | 100% | 101% |
| Class D | 0.02% | -0.04% | 0.02% | 100% | 91% |
| Class F | 0.03% | -0.17% | -0.11% | 100% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.01% | 0.02% | 101% | 100% |
| Class C | 0.00% | 0.03% | 0.03% | 98% | 97% |
| Class E | 0.01% | 0.04% | 0.04% | 98% | 99% |
| Overall | 0.01% | 0.02% | 0.03% | 99% | 99% |
| Class D | 0.00% | 0.02% | -0.01% | 101% | 101% |
| Class F | -0.01% | -0.14% | -0.02% | 98% | 97% |

Table 4: Summary of the average BD bitrate results for Method 2 over low QPs 2, 7, 12, and 17

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.37% | 0.36% | 97% | 93% |
| Class A2 | 0.00% | 0.31% | 0.31% | 100% | 100% |
| Class B | 0.00% | 0.16% | 0.28% | 102% | 104% |
| Class C | 0.01% | 0.14% | 0.20% | 101% | 102% |
| Class E | -0.02% | 0.26% | 0.33% | 100% | 101% |
| Overall | -0.01% | 0.23% | 0.29% | 100% | 101% |
| Class D | 0.01% | 0.12% | 0.18% | 100% | 100% |
| Class F | 0.02% | 0.12% | 0.16% | 100% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.12% | 0.14% | 101% | 101% |
| Class A2 | 0.01% | 0.17% | 0.17% | 101% | 99% |
| Class B | 0.01% | 0.09% | 0.15% | 100% | 100% |
| Class C | -0.01% | 0.10% | 0.14% | 101% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.11% | 0.15% | 101% | 99% |
| Class D | 0.00% | 0.07% | 0.07% | 101% | 96% |
| Class F | -0.13% | -0.09% | 0.00% | 100% | 92% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.09% | 0.15% | 99% | 96% |
| Class C | -0.01% | 0.06% | 0.10% | 99% | 97% |
| Class E | 0.01% | 0.14% | 0.16% | 99% | 101% |
| Overall | 0.00% | 0.09% | 0.14% | 99% | 98% |
| Class D | 0.01% | 0.04% | 0.03% | 101% | 102% |
| Class F | -0.02% | -0.09% | 0.07% | 99% | 98% |

# Conclusions

This contribution proposes two modified methods to further reduce the number of context variables in the two additional context sets based on the context reduction scheme in CE7-3.1. The experimental results reportedly show that the proposed methods have negligible impact on BD rate performance while further reducing the number of context variables for entropy coding the syntax element sig_coeff_flag.

# References

- S.-T. Hsiang, T.-D. Chuang, Y.-W. Huang and S.-M. Lei, “CE7-3.1: Context reduction for sig_coeff_flag,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-P0047, 16th Meeting: Geneva, CH, 1–11 October 2019.

- VTM6.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM6.0

- H. Schwarz and M. Coban, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-O2027, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).