| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0175-v2 |
| --- | --- |

| Title: | AHG16/Non-CE4: A clean-up to merge list generation by removing shared merge list |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input document to JVET |  |  |
| Purpose: |  | Proposal |  |  |
| Author(s) or Contact(s): |  | Li Jingya Lim Chong Soon | Tel: Email: | +65 6552 5423 jingya.li@sg.panasonic.com |
| Source: |  | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to clean up merge list generation by removing shared merge list. The proposed method is to skip pruning process when current block is 4x4 IBC block. It noted that additional benefit is that hardware cost si reduced by avoiding deplicate memory and logic. Text is also simplified.

Coding impact of test a and b are as below (with VTM6.0 + IBC on as anchor):

CTC													Intra:	   -0.62%, -0.61%, -0.64%

RA:	   0.02%, -0.03%, -0.09%

LDB:     0.02%, 0.20%, 0.15%

ClassF													Intra:	   0.02%, -0.09%, 0.01%

RA:	   0.05%, 0.04%, 0.01%

LDB:     0.11%, 0.04%, 0.60%

TGM													Intra:	   0.14%, 0.13%, 0.10%

RA:	   0.10%, 0.06%, 0.03%

LDB:     0.01%, 0.17%, 0.12%

# Introduction

In current VTM6.0, shared merge list is used to solve the tight cycle budget issue for 4x4 sized IBC, wherein candidate list generation may take 12 cyles which is tight for a typical use case(400MHZ, 4K video). An illustration of virtual cycle counts for IBC candidate list generation is given in Figure 1 if shared merge list is not used.

Figure 1: illustration of virtual cycle counts for IBC candidate list generation of VTM6.0 (actual case may be different)

However, by using shared merge list only for IBC mode, CU location and size information need to be stored before CU split when the CU size will be less than limitation due to the split (64*64/32*(5+5) bits = 160B for a 64x64 VPDU). And this information is referred for merge list generation when IBC with small CU size.

Consequently, a decoder needs duplicated memory to store CU shape information (location and size) as illustrated in Figure 1.

(1) CU information after splitting for usual processing

(2) CU information before splitting for shared merge list

Figure 2: Illustration of addition memory and logic for shared merge list

It is desired to clean up current VTM6.0 design by remove the unnecessary cost by using an alternative method to reduce number of cycles of 4x4 IBC candidate list generation process as below:

- Directly add HMVP candidate to candidate list without pruning when current CU is 4x4 IBC block.

- Directly update HMVP table without pruning when current CU is 4x4 IBC block.

- Remove shared merge list.

An illustration of cycles saved by the proposal is given in Figure 2.

Figure 3: Illustration of virtual cycle counts for IBC candidate list generation of proposal(actual case may be different)

# Simulation results

Simulation results with IBC on (VTM6.0 as anchor is as below). It is observed that there is very little coding impact on classF and classTGM. Note that the encoding and decoding time is not very accurate:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.06% | 0.10% | 142% | 97% |
| Class A2 | -0.79% | -0.94% | -0.91% | 144% | 95% |
| Class B | -0.79% | -0.66% | -0.86% | 160% | 98% |
| Class C | -0.27% | -0.29% | -0.25% | 161% | 99% |
| Class E | -1.23% | -1.31% | -1.29% | 162% | 100% |
| Overall | -0.62% | -0.61% | -0.64% | 155% | 98% |
| Class D | -0.58% | -0.69% | -0.71% | 149% | 99% |
| Class F | 0.02% | -0.09% | 0.01% | 100% | 101% |
| TGM | 0.14% | 0.13% | 0.10% | 98% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | 0.02% | 0.24% | 104% | 98% |
| Class A2 | -0.06% | -0.12% | -0.07% | 105% | 99% |
| Class B | -0.14% | -0.12% | -0.34% | 108% | 100% |
| Class C | 0.21% | 0.09% | -0.03% | 111% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.03% | -0.09% | 108% | 99% |
| Class D | 0.07% | -0.29% | -0.34% | 111% | 99% |
| Class F | 0.05% | 0.04% | 0.01% | 144% | 147% |
| TGM | 0.10% | 0.06% | 0.03% | 96% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.09% | 0.10% | 113% | 98% |
| Class C | 0.03% | 0.14% | 0.03% | 117% | 100% |
| Class E | 0.04% | 0.46% | 0.38% | 111% | 99% |
| Overall | 0.02% | 0.20% | 0.15% | 114% | 99% |
| Class D | 0.06% | 0.30% | 0.10% | 120% | 101% |
| Class F | 0.11% | 0.04% | 0.60% | 96% | 97% |
| TGM | 0.01% | 0.17% | 0.12% | 94% | 98% |

# Conclusion

It is proposed to remove shared merge list for clean-up of IBC merge list generation. Doing so reduces hardware cost by avoiding deplicate memory and logic and also simplifies specification text.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).