| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0179-v1 |
| --- | --- |

| Title: | Non-CE5: ALF as a post-filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Boon Teo Hai Wei Sun Chong Soon Lim | Tel: Email: | +65-6550-5417 hanboon.teo@sg.panasonic.com haiwei.sun@sg.panasonic.com chongsoon.lim@sg.panasoni.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution proposes the use of ALF as a post-filter. A flag is signalled which switches ALF to function as either in-loop filter or post-filter. The operation and signalling of ALF itself is not changed.

# Introduction

In the VTM6, an Adaptive Loop Filter (ALF) with block-based filter adaption is applied. For the luma component, one among 25 filters is selected for each 4×4 block, based on the direction and activity of local gradients. The ALF filter parameters are signalled in Adaptation Parameter Set (APS). In one APS, up to 25 sets of luma filter coefficients and clipping value indexes, and up to eight sets of chroma filter coefficients and clipping value indexes could be signalled.

The ALF filter coefficients are calculated in the encoder and the filter sets which have the lowest rate distortion costs are used and signalled. Figure 1 shows VTM6 decoder block diagram and location of ALF.

Figure 1 – VTM6 decoder block diagram

# Proposal

An alternative use of ALF is proposed. The concept is to allow video creator/publisher (or artist) to create and deliver the same customized filtering effects to consumer using only VVC. Figure 2 shows the changes required to the decoder block diagram.

Figure 2 – Proposed change to ALF feedback loop

By adding a high-level switch to the ALF feedback loop, we can control the ALF to be used in either in-loop filter or as a post-filter. There is no change needed to existing ALF design. The signalling of APS for ALF and its filter coefficients remain the same as existing ALF design.

Instead of PSNR improvement, ALF post-filter can be designed to customize subjective picture quality. The ALF can be configured on/off for each CTU with the user customized filter (up to 25 sets of luma filters).

Example of a use-case in production studio, the content provider can customize the desired subjective quality and deliver to the consumer the same subjective quality by just using VVC without additional filter engines.

# Comparisons

Table 1 shows the comparison between different ways to apply the filter. In the example, sharpening filter is used to show the advantages (in green color) of using ALF as a post-filter.

Table 1 Comparing different ways to apply filter

| Pre-filter (before encoding) | HEVC post-filter | ALF as a post-filter |
| --- | --- | --- |
| Separate module from codec | Separate module from HEVC | Built-in VVC |
| Determined by content provider, but typically: Picture level on/off No classification 1 set of filter | Picture level on/off | CTU level on/off |
|  | No classification | 4x4 level luma block classification |
|  | 1 set of filter for luma, 1 set of filter for Cb, 1 set of filter for Cr. | Up to 25 sets of luma filters, 1 set of filter for Cb, 1 set of filter for Cr. |
| NA | NA | Virtual boundary filtering |
| NA | NA | Additional buffer to store unfiltered pixels (before ALF) |
| Sharpening original source | At same bitrate, smaller QP used compared to pre-filter. | At same bitrate, smaller QP used compared to pre-filter. |

# Summary

An alternative use of ALF was proposed. There are no changes to existing ALF engine or it’s signalling. With this proposed change, a new channel of picture quality customization can be realized. We propose to adopt using ALF as a post-filter into VVC specification.

# References

[1] J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 6 (VTM 6)”, JVET-O2002, JVET 15th Meeting: Gothenburg, SE, 3–12 July 2019.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft text method 1

Add a SPS flag.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_alf_enabled_flag | u(1) |
| if ( sps_alf_enabled_flag ) |  |
| sps_alf_post_flag | u(1) |
| … |  |

sps_alf_post_flag equal to 0 specifies that the adaptive loop filter is functioning as an in-loop filter. sps_alf_post_flag equal to 1 specifies that the adaptive loop filter is functioning as a post-filter. When not present, the value of sps_alf_post_flag is inferred to be equal to 0.

# Draft text method 2

Add SEI syntax.

D.2.xx	Adaptive post-filter hint SEI message syntax

| adaptive_post_filter_hint( payloadSize ) { | Descriptor |
| --- | --- |
| alf_post_flag | u(1) |
| if( alf_post_flag ) |  |
| alf_data( ) |  |
| } |  |
| … |  |

D.3.xx	Adaptive post-filter hint SEI message semantics

This SEI message provides the signal for adaptive loop filter to function as an in-loop filter or a post-filter. When post-filter function is selected, it can be used for customizing multiple filter to create different filtering effects.

alf_post_flag equal to 0 specifies that the adaptive loop filter is functioning as an in-loop filter. alf_post_flag equal to 1 specifies that the adaptive loop filter is functioning as a post-filter. alf_post_flag can only be equal to 1 when sps_alf_enabled_flag of the bitstream is 0. When not present, the value of alf_post_flag is inferred to be equal to 0.