| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0183-v1 |
| --- | --- |

| Title: | On Picture Timing Information Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Modifications are proposed related to Picture Timing information signaling including HRD decoding unit information signalling.

Following is proposed:

- Proposal 1: It is proposed to provide signalling to specify that each subpicture is a decoding unit.

- Proposal 2: It is proposed to conditionally signal cpb_removal_delay_delta_enabled_flag[ i ] for i in the range of TemporalId to pt_max_sub_layers_minus1−1, inclusive, based on cpb_removal_delay_deltas_present_flag.

- Proposal 3: It is asserted that the syntax element cpb_removal_delay_delta_idx[ i ] is coded as u(v) however the number of bits used for coding it are not specified. It is proposed to specify the number of bits used to code cpb_removal_delay_delta_idx[ i ].

# Introduction

VVC WD [1] includes HRD which supports decoding unit based HRD signalling and operation. This is achieved by signalling in picture timing SEI message, and decoding unit SEI message and SPS. Also VVC WD includes support for subpictures.

Modifications are proposed related to Picture Timing information signalling including HRD decoding unit information signalling.

Following is proposed:

- Proposal 1: It is proposed to provide signalling to specify that each subpicture is a decoding unit.

- Proposal 2: It is proposed to conditionally signal cpb_removal_delay_delta_enabled_flag[ i ] for i in the range of TemporalId to pt_max_sub_layers_minus1−1, inclusive, based on cpb_removal_delay_deltas_present_flag.

- Proposal 3: It is asserted that the syntax element cpb_removal_delay_delta_idx[ i ] is coded as u(v) however the number of bits used for coding it are not specified. It is proposed to specify the number of bits used to code cpb_removal_delay_delta_idx[ i ].

# Proposals

## Proposal 1

VVC WD [1] includes HRD which supports decoding unit based HRD signalling and operation. Also VVC WD includes support for subpictures. It is proposed to efficiently support signaling that each subpicture is a decoding unit.

It is asserted that for use cases such as viewing orientation change in mixed-resolution viewport-dependent 360° streaming, subpictures are suitable to be used. Also, in this case a decoding unit based operation, where subpictures are decoding units can result in lower latency which is important for low-latency high-quality immersive experience during viewing orientation change.

It is proposed to:

- Signal a flag in SPS to specify if each subpicture is a decoding unit.

- In picture timing SEI, the information about number of decoding units in the picture (num_decoding_units_minus1) and about number of NAL units in each decoding unit (num_nalus_in_du_minus1[ i ]) is not signalled when each subpicture is a decoding unit (and the above flag in SPS is equal to 1) and that information is derived.

- In the decoding unit information message, semantics of decoding unit index are modified when each subpicture is a decoding unit. It is asserted that this results in using shorter decoding unit indices when subpictures are used.

The proposed changes are shown highlighted compared with VVC WD.

7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| subpic_is_du_flag | u(1) |
| } |  |
| … |  |
| } |  |

subpic_is_du_flag equal to 1 specifies that each subpicture is a decoding unit. subpic_is_du_flag equal to 0 specifies that a subpicture may or may not be a decoding unit. When not present subpic_is_du_flag is inferred to be equal to 0.

D.2.3 Picture timing SEI message syntax

| pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| pt_max_sub_layers_minus1 | u(3) |
| cpb_removal_delay_minus1[ pt_max_sub_layers_minus1 ] | u(v) |
| for( i = TemporalId; i < pt_max_sub_layers_minus1; i++ ) { |  |
| sub_layer_delays_present_flag[ i ] | u(1) |
| if( sub_layer_delays_present_flag[ i ] ) { |  |
| cpb_removal_delay_delta_enabled_flag[ i ] | u(1) |
| if( cpb_removal_delay_delta_enabled_flag[ i ] ) |  |
| cpb_removal_delay_delta_idx[ i ] | u(v) |
| else |  |
| cpb_removal_delay_minus1[ i ] | u(v) |
| } |  |
| } |  |
| dpb_output_delay | u(v) |
| if( decoding_unit_hrd_params_present_flag ) |  |
| pic_dpb_output_du_delay | u(v) |
| if( decoding_unit_hrd_params_present_flag && decoding_unit_cpb_params_in_pic_timing_sei_flag ) { |  |
| if(!subpic_is_du_flag) |  |
| num_decoding_units_minus1 | ue(v) |
| du_common_cpb_removal_delay_flag | u(1) |
| if( du_common_cpb_removal_delay_flag ) |  |
| du_common_cpb_removal_delay_increment_minus1 | u(v) |
| for( i = 0; i <= num_decoding_units_minus1; i++ ) { |  |
| if(!subpic_is_du_flag) |  |
| num_nalus_in_du_minus1[ i ] | ue(v) |
| if( !du_common_cpb_removal_delay_flag && i < num_decoding_units_minus1 ) |  |
| du_cpb_removal_delay_increment_minus1[ i ] | u(v) |
| } |  |
| } |  |
| } |  |

D.2.4 Decoding unit information SEI message syntax

| decoding_unit_info( payloadSize ) { | Descriptor |
| --- | --- |
| decoding_unit_idx | ue(v) |
| if( !decoding_unit_cpb_params_in_pic_timing_sei_flag ) |  |
| du_spt_cpb_removal_delay_increment | u(v) |
| dpb_output_du_delay_present_flag | u(1) |
| if( dpb_output_du_delay_present_flag ) |  |
| pic_spt_dpb_output_du_delay | u(v) |
| } |  |

D3.3 Picture timing SEI message semantics

The picture timing SEI message provides CPB removal delay and DPB output delay information for the access unit associated with the SEI message.

If bp_nal_hrd_parameters_present_flag or bp_vcl_hrd_parameters_present_flag of the buffering period SEI mesage applicable for the current access unit is equal to 1, the variable CpbDpbDelaysPresentFlag is set equal to 1. Otherwise, CpbDpbDelaysPresentFlag is set equal to 0.

The presence of picture timing SEI messages is specified as follows:

–	If CpbDpbDelaysPresentFlag is equal to 1, a picture timing SEI message shall be associated with the current access unit.

–	Otherwise (CpbDpbDelaysPresentFlag is equal to 0), there shall not be a picture timing SEI message associated with the current access unit.

pt_max_sub_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in the bitstream. The value of pt_max_sub_layers_minus1 shall be in the range of 0 to 6, inclusive.

It is a requirement of bitstream conformance that the value of pt_max_sub_layers_minus1 in the picture timing SEI message is equal to the value of sps_max_sub_layers_minus1 in the SPS.

cpb_removal_delay_minus1[ i ] plus 1 is used to calculate the number of clock ticks between the nominal CPB removal times of the access unit associated with the picture timing SEI message and the preceding access unit in decoding order that contains a buffering period SEI message when HighestTid is equal to i. This value is also used to calculate an earliest possible time of arrival of access unit data into the CPB for the HSS. The length of cpb_removal_delay_minus1[ i ] is cpb_removal_delay_length_minus1 + 1 bits.

The variable BpResetFlag of the current picture is derived as follows:

- If the current picture is associated with a buffering period SEI message, BpResetFlag is set equal to 1.

- Otherwise, BpResetFlag is set equal to 0.

sub_layer_delays_present_flag[ i ] equal to 1 specifies that cpb_removal_delay_delta_idx[ i ] or cpb_removal_delay_minus1[ i ] is present for the the sub-layer with TemporalId equal to i. sub_layer_delays_present_flag[ i ] equal to 0 specifies that neither cpb_removal_delay_delta_idx[ i ] nor cpb_removal_delay_minus1[ i ] is present for the sub-layer with TemporalId equal to i. When not present, the value of sub_layer_delays_present_flag[ i ] is infered to be equal to 0.

cpb_removal_delay_delta_enabled[ i ] equal to 1 specifies that cpb_removal_delay_delta_idx[ i ] is present in the picture timing SEI message. cpb_removal_delay_delta_enabled[ i ] equal to 0 specifies that cpb_removal_delay_delta_idx[ i ] is not present in the picture timing SEI message. When not present, the value of cpb_removal_delay_delta_enabled[ i ] is infered to be equal to 0.

cpb_removal_delay_delta_idx[ i ] specifies the index of the CPB removal delta that applies to HighestTid equal to i in the list of cpb_removal_delay_delta[ j ] for j ranging from 0 to num_cpb_removal_delay_deltas_minus1, inclusive.

The variables CpbRemovalDelayMsb[ i ] and CpbRemovalDelayVal[ i ] of the current picture are derived as follows:

- If the current access unit is the access unit that initializes the HRD, CpbRemovalDelayMsb[ i ] and CpbRemovalDelayVal[ i ] are both set equal to 0, and the value of cpbRemovalDelayValTmp[ i ] is set equal to cpb_removal_delay_minus1[ i ] + 1.

- Otherwise, let the picture prevNonDiscardablePic be the previous picture in decoding order that has TemporalId equal to 0 that is not a RASL or RADL, let prevCpbRemovalDelayMinus1[ i ], prevCpbRemovalDelayMsb[ i ], and prevBpResetFlag be set equal to the values of cpbRemovalDelayValTmp[ i ] − 1, CpbRemovalDelayMsb[ i ], and BpResetFlag, respectively, for the picture prevNonDiscardablePic, and the following applies:

- CpbRemovalDelayMsb[ i ] is derived as follows:

cpbRemovalDelayValTmp[ i ] = cpb_removal_delay_delta_enabled_flag[ i ] ?
	cpb_removal_delay_minus1[ sps_max_sub_layers_minus1 ] + 1 +
	cpb_removal_delay_delta[ cpb_removal_delay_delta_idx[ i ] ] : cpb_removal_delay_minus1[ i ] + 1
if( prevBpResetFlag )
	CpbRemovalDelayMsb[ i ] = 0
else if( cpbRemovalDelayValTmp[ i ] < prevCpbRemovalDelayMinus1[ i ] )
	CpbRemovalDelayMsb[ i ] = prevCpbRemovalDelayMsb[ i ] + 2cpb_removal_delay_length_minus1 + 1	(D-1)
else
	CpbRemovalDelayMsb[ i ] = prevCpbRemovalDelayMsb[ i ]

- CpbRemovalDelayVal is derived as follows:

CpbRemovalDelayVal[ i ] = CpbRemovalDelayMsb[ i ] + cpbRemovalDelayValTmp[ i ]	(D-2)

The value of CpbRemovalDelayVal[ i ] shall be in the range of 1 to 232, inclusive.

The variable picDpbOutputDelta[ i ] is derived as follows:

- If sub_layer_delays_present_flag[ i ] is equal to 0, picDpbOutputDelta[ i ] is set equal to 0.

- Otherwise (sub_layer_delays_present_flag[ i ] is equal to 1), picDpbOutputDelta[ i ] is set equal to CpbRemovalDelayVal[ i ] – ( cpb_removal_delay_minus1[ sps_max_sub_layers_minus1 ] + 1 ).

dpb_output_delay is used to compute the DPB output time of the picture. It specifies how many clock ticks to wait after removal of an access unit from the CPB before the decoded picture is output from the DPB.

NOTE – A picture is not removed from the DPB at its output time when it is still marked as "used for short-term reference" or "used for long-term reference".

The length of dpb_output_delay is dpb_output_delay_length_minus1 + 1 bits. When sps_max_dec_pic_buffering_minus1 is equal to 0, the value of pic_dpb_output_delay shall be equal to 0.

The output time derived from the dpb_output_delay of any picture that is output from an output timing conforming decoder shall precede the output time derived from the dpb_output_delay of all pictures in any subsequent CVS in decoding order.

The picture output order established by the values of this syntax element shall be the same order as established by the values of PicOrderCntVal.

For pictures that are not output by the "bumping" process because they precede, in decoding order, a CLVSS picture that has no_output_of_prior_pics_flag equal to 1 or inferred to be equal to 1, the output times derived from dpb_output_delay shall be increasing with increasing value of PicOrderCntVal relative to all pictures within the same CVS.

pic_dpb_output_du_delay is used to compute the DPB output time of the picture when DecodingUnitHrdFlag is equal to 1. It specifies how many sub clock ticks to wait after removal of the last decoding unit in an access unit from the CPB before the decoded picture is output from the DPB.

The length of the syntax element pic_dpb_output_du_delay is given in bits by dpb_output_delay_du_length_minus1 + 1.

The output time derived from the pic_dpb_output_du_delay of any picture that is output from an output timing conforming decoder shall precede the output time derived from the pic_dpb_output_du_delay of all pictures in any subsequent CVS in decoding order.

The picture output order established by the values of this syntax element shall be the same order as established by the values of PicOrderCntVal.

For pictures that are not output by the "bumping" process because they precede, in decoding order, a CLVSS picture that has no_output_of_prior_pics_flag equal to 1 or inferred to be equal to 1, the output times derived from pic_dpb_output_du_delay shall be increasing with increasing value of PicOrderCntVal relative to all pictures within the same CVS.

For any two pictures in the CVS, the difference between the output times of the two pictures when DecodingUnitHrdFlag is equal to 1 shall be identical to the same difference when DecodingUnitHrdFlag is equal to 0.

num_decoding_units_minus1 plus 1 specifies the number of decoding units in the access unit the picture timing SEI message is associated with. When subpic_is_du flag is equal to 1  num_decoding_units_minus1  is inferred to be equal to NumSubPics-1. When subpic_is_du flag is equal to 0,  the value of num_decoding_units_minus1 shall be in the range of 0 to PicSizeInCtbsY − 1, inclusive. When subpic_is_du flag is equal to 1,  the value of num_decoding_units_minus1 shall be in the range of 0 to max_subpics_minus1, inclusive.

du_common_cpb_removal_delay_flag equal to 1 specifies that the syntax element du_common_cpb_removal_delay_increment_minus1 is present. du_common_cpb_removal_delay_flag equal to 0 specifies that the syntax element du_common_cpb_removal_delay_increment_minus1 is not present.

du_common_cpb_removal_delay_increment_minus1 plus 1 specifies the duration, in units of clock sub-ticks (see clause C.1), between the nominal CPB removal times of any two consecutive decoding units in decoding order in the access unit associated with the picture timing SEI message. This value is also used to calculate an earliest possible time of arrival of decoding unit data into the CPB for the HSS, as specified in Annex C. The lenght of this syntax element is du_cpb_removal_delay_increment_length_minus1 + 1.

num_nalus_in_du_minus1[ i ] plus 1 specifies the number of NAL units in the i-th decoding unit of the access unit the picture timing SEI message is associated with. When subpic_is_du_flag is equal to 1, num_nalus_in_du_minus1[ i ] is inferred to be equal to NumSlicesofSubPic[ SubPicIdx ] where  SubPicIdx  is the subpicture index of the i-th sub-picture. The value of num_nalus_in_du_minus1[ i ] shall be in the range of 0 to PicSizeInCtbsY − 1, inclusive.

The first decoding unit of the access unit consists of the first num_nalus_in_du_minus1[ 0 ] + 1 consecutive NAL units in decoding order in the access unit. The i-th (with i greater than 0) decoding unit of the access unit consists of the num_nalus_in_du_minus1[ i ] + 1 consecutive NAL units immediately following the last NAL unit in the previous decoding unit of the access unit, in decoding order. There shall be at least one VCL NAL unit in each decoding unit. All non-VCL NAL units associated with a VCL NAL unit shall be included in the same decoding unit as the VCL NAL unit.

du_cpb_removal_delay_increment_minus1[ i ] plus 1 specifies the duration, in units of clock sub-ticks, between the nominal CPB removal times of the ( i + 1 )-th decoding unit and the i-th decoding unit, in decoding order, in the access unit associated with the picture timing SEI message. This value is also used to calculate an earliest possible time of arrival of decoding unit data into the CPB for the HSS, as specified in Annex C. The lenght of this syntax element is du_cpb_removal_delay_increment_length_minus1 + 1.

D.3.4 Decoding unit information SEI message semantics

[Ed. (RS): The original proposal text in JVET-O0189 referes to layering and scalable nesting which is removed in the text below]

The decoding unit information SEI message provides CPB removal delay information for the decoding unit associated with the SEI message.

The following applies for the decoding unit information SEI message syntax and semantics:

–	The syntax elements decoding_unit_hrd_params_present_flag, decoding_unit_cpb_params_in_pic_timing_sei_flag and dpb_output_delay_du_length_minus1, and the variable CpbDpbDelaysPresentFlag are found in or derived from syntax elements in the general_hrd_parameters( ) syntax structure that is applicable to at least one of the operation points to which the decoding unit information SEI message applies.

–	The bitstream (or a part thereof) refers to the bitstream subset (or a part thereof) associated with any of the operation points to which the decoding unit information SEI message applies.

The presence of decoding unit information SEI messages for an operation point is specified as follows:

–	If CpbDpbDelaysPresentFlag is equal to 1, decoding_unit_hrd_params_present_flag is equal to 1 and decoding_unit_cpb_params_in_pic_timing_sei_flag is equal to 0, one or more decoding unit information SEI messages applicable to the operation point shall be associated with each decoding unit in the CVS.

–	Otherwise, if CpbDpbDelaysPresentFlag is equal to 1, decoding_unit_hrd_params_present_flag is equal to 1 and decoding_unit_cpb_params_in_pic_timing_sei_flag is equal to 1, one or more decoding unit information SEI messages applicable to the operation point may or may not be associated with each decoding unit in the CVS.

–	Otherwise (CpbDpbDelaysPresentFlag is equal to 0 or decoding_unit_hrd_params_present_flag is equal to 0), in the CVS there shall be no decoding unit that is associated with a decoding unit information SEI message applicable to the operation point.

The set of NAL units associated with a decoding unit information SEI message consists, in decoding order, of the SEI NAL unit containing the decoding unit information SEI message and all subsequent NAL units in the access unit up to but not including any subsequent SEI NAL unit containing a decoding unit information SEI message with a different value of decoding_unit_idx. Each decoding unit shall include at least one VCL NAL unit. All non-VCL NAL units associated with a VCL NAL unit shall be included in the decoding unit containing the VCL NAL unit.

decoding_unit_idx specifies the index, starting from 0, to the list of decoding units in the current access unit, of the decoding unit associated with the decoding unit information SEI message. When subpic_is_du_flag is equal to 1, the decoding_unit_idx specifies the subpicture index for the subpicture that corresponds to the decoding unit. When subpic_is_du_flag is equal to 0, the value of decoding_unit_idx shall be in the range of 0 to PicSizeInCtbsY − 1, inclusive. When subpic_is_du_flag is equal to 1, the value of decoding_unit_idx shall be in the range of 0 to max_subpics_minus1, inclusive (or shall be equal to one of the signalled value of sub-picture indices for the subpicture of the picture).

A decoding unit identified by a particular value of duIdx includes and only includes all NAL units associated with all decoding unit information SEI messages that have decoding_unit_idx equal to duIdx. Such a decoding unit is also referred to as associated with the decoding unit information SEI messages having decoding_unit_idx equal to duIdx.

For any two decoding units duA and duB in one access unit with decoding_unit_idx equal to duIdxA and duIdxB, respectively, where duIdxA is less than duIdxB, duA shall precede duB in decoding order.

A NAL unit of one decoding unit shall not be present, in decoding order, between any two NAL units of another decoding unit.

du_spt_cpb_removal_delay_increment specifies the duration, in units of clock sub-ticks, between the nominal CPB times of the last decoding unit in decoding order in the current access unit and the decoding unit associated with the decoding unit information SEI message. This value is also used to calculate an earliest possible time of arrival of decoding unit data into the CPB for the HSS, as specified in Annex C. The lenght of this syntax element is du_cpb_removal_delay_increment_length_minus1 + 1. When the decoding unit associated with the decoding unit information SEI message is the last decoding unit in the current access unit, the value of du_spt_cpb_removal_delay_increment shall be equal to 0.

dpb_output_du_delay_present_flag equal to 1 specifies the presence of the pic_spt_dpb_output_du_delay syntax element in the decoding unit information SEI message. dpb_output_du_delay_present_flag equal to 0 specifies the absence of the pic_spt_dpb_output_du_delay syntax element in the decoding unit information SEI message.

pic_spt_dpb_output_du_delay is used to compute the DPB output time of the picture when DecodingUnitHrdFlag is equal to 1. It specifies how many sub clock ticks to wait after removal of the last decoding unit in an access unit from the CPB before the decoded picture is output from the DPB. When not present, the value of pic_spt_dpb_output_du_delay is inferred to be equal to pic_dpb_output_du_delay. The length of the syntax element pic_spt_dpb_output_du_delay is given in bits by dpb_output_delay_du_‌length_minus1 + 1.

It is a requirement of bitstream conformance that all decoding unit information SEI messages that are associated with the same access unit, apply to the same operation point, and have dpb_output_du_delay_present_flag equal to 1 shall have the same value of pic_spt_dpb_output_du_delay.

The output time derived from the pic_spt_dpb_output_du_delay of any picture that is output from an output timing conforming decoder shall precede the output time derived from the pic_spt_dpb_output_du_delay of all pictures in any subsequent CVS in decoding order.

The picture output order established by the values of this syntax element shall be the same order as established by the values of PicOrderCntVal.

For pictures that are not output by the "bumping" process because they precede, in decoding order, a CLVSS picture that has no_output_of_prior_pics_flag equal to 1 or inferred to be equal to 1, the output times derived from pic_spt_dpb_output_du_delay shall be increasing with increasing value of PicOrderCntVal relative to all pictures within the same CVS.

For any two pictures in the CVS, the difference between the output times of the two pictures when DecodingUnitHrdFlag is equal to 1 shall be identical to the same difference when DecodingUnitHrdFlag is equal to 0.

## 7.4.7 Slice header semantics

## 7.4.7.1 General slice header semantics

…

num_bricks_in_slice_minus1, when present, specifies the number of bricks in the slice minus 1. The value of num_bricks_in_slice_minus1 shall be in the range of 0 to NumBricksInPic − 1, inclusive. When rect_slice_flag is equal to 0 and single_brick_per_slice_flag is equal to 1, the value of num_bricks_in_slice_minus1 is inferred to be equal to 0. When single_brick_per_slice_flag is equal to 1, the value of num_bricks_in_slice_minus1 is inferred to be equal to 0.

The variable NumBricksInCurrSlice, which specifies the number of bricks in the current slice, and SliceBrickIdx[ i ], which specifies the brick index of the i-th brick in the current slice, are derived as follows:

if( rect_slice_flag ) {
	sliceIdx = 0
	while( slice_address  !=  slice_id[ sliceIdx ] )
		sliceIdx++
	NumBricksInCurrSlice = NumBricksInSlice[ sliceIdx ]
	brickIdx = TopLeftBrickIdx[ sliceIdx ]
	for( bIdx = 0; brickIdx  <=  BottomRightBrickIdx[ sliceIdx ]; brickIdx++ )	(7-92)
		if( BricksToSliceMap[ brickIdx ]  = =  sliceIdx )
			SliceBrickIdx[ bIdx++ ] = brickIdx
} else {
	NumBricksInCurrSlice = num_bricks_in_slice_minus1 + 1
	SliceBrickIdx[ 0 ] = slice_address
	for( i = 1; i < NumBricksInCurrSlice; i++ )
		SliceBrickIdx[ i ] = SliceBrickIdx[ i − 1 ] + 1
}

The variables SubPicIdx, SubPicLeftBoundaryPos, SubPicTopBoundaryPos, SubPicRightBoundaryPos, and SubPicBotBoundaryPos are derived as follows:

SubPicIdx = CtbToSubPicIdx[ CtbAddrBsToRs[ FirstCtbAddrBs[ SliceBrickIdx[ 0 ] ] ] ]
if( subpic_treated_as_pic_flag[ SubPicIdx ] ) {
	SubPicLeftBoundaryPos = SubPicLeft[ SubPicIdx ] * ( subpic_grid_col_width_minus1 + 1 ) * 4
	SubPicRightBoundaryPos = ( SubPicLeft[ SubPicIdx ] + SubPicWidth[ SubPicIdx ] ) *
				( subpic_grid_col_width_minus1 + 1 ) * 4	(7-93)
	SubPicTopBoundaryPos = SubPicTop[ SubPicIdx ] * ( subpic_grid_row_height_minus1 + 1 )* 4
	SubPicBotBoundaryPos = ( SubPicTop[ SubPicIdx ] + SubPicHeight[ SubPicIdx ] ) *
				( subpic_grid_row_height_minus1 + 1 ) * 4
}

The variable NumSlicesofSubPic[ [ SubPicIdx ]  which specifies the number of slices which belong to a subpicture with subpicture index SubPicIdx is derived and updated as follows. 
NumSlicesofSubPic[ SubPicIdx ] = NumSlicesofSubPic[ SubPicIdx ] + 1

## 6.5 Scanning processes

### 6.5.1 CTB raster scanning, tile scanning, brick scanning, and subpicture scanning processes

…

The list CtbToSubPicIdx[ ctbAddrRs ] for ctbAddrRs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in picture raster scan to a subpicture index, is derived as follows:

for( ctbAddrRs = 0; ctbAddrRs < PicSizeInCtbsY; ctbAddrRs++ ) {
	posX = ctbAddrRs % PicWidthInCtbsY * CtbSizeY 
	posY = ctbAddrRs / PicWidthInCtbsY * CtbSizeY 
	CtbToSubPicIdx[ ctbAddrRs ] = −1
	for( i = 0; CtbToSubPicIdx[ ctbAddrRs ] < 0  && i < NumSubPics; i++ ) {
		if( ( posX  >=  SubPicLeft[ i ] * ( subpic_grid_col_width_minus1 + 1 ) * 4 )  &&
				( posX < ( SubPicLeft[ i ] + SubPicWidth[ i ] ) *
						( subpic_grid_col_width_minus1 + 1 ) * 4 )  &&	(6-11)
				( posY  >=  SubPicTop[ i ] *
						( subpic_grid_row_height_minus1 + 1 ) * 4 )  &&
				( posY < ( SubPicTop[ i ] + SubPicHeight[ i ] ) *
						( subpic_grid_row_height_minus1 + 1 ) * 4 ) )
			CtbToSubPicIdx[ ctbAddrRs ] = i
	}
}

The list NumSlicesofSubPic[ SubPicIdx ]  for SubPicIdx  ranging from 0 to NumSubPics −1, inclusive, specifying the number of slices which belong to a subpicture with subpicture index SubPicIdx is initialized as follows: 
for( i = 0; i < NumSubPics; i++ ) 
	NumSlicesofSubPic[ [ SubPicIdx ] = 0

## Proposal 2

It is proposed to conditionally signal cpb_removal_delay_delta_enabled_flag[ i ] for i in the range of TemporalId to pt_max_sub_layers_minus1−1, inclusive, based on cpb_removal_delay_deltas_present_flag. This saves bits.

It is asserted that when cpb_removal_delay_deltas_present_flag is equal to 0, there are no signalled cpb removal delay deltas that can be sent an index to thus cpb_removal_delay_delta_enabled_flag[ i ] will have to be zero. In this case signaling cpb_removal_delay_delta_enabled_flag[ i ] wastes bits and instead bit savings can be achieved by using signaling as shown below.

| pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| pt_max_sub_layers_minus1 | u(3) |
| cpb_removal_delay_minus1[ pt_max_sub_layers_minus1 ] | u(v) |
| for( i = TemporalId; i < pt_max_sub_layers_minus1; i++ ) { |  |
| sub_layer_delays_present_flag[ i ] | u(1) |
| if( sub_layer_delays_present_flag[ i ] ) { |  |
| if(cpb_removal_delay_deltas_present_flag ) |  |
| cpb_removal_delay_delta_enabled_flag[ i ] | u(1) |
| if( cpb_removal_delay_delta_enabled_flag[ i ] ) |  |
| cpb_removal_delay_delta_idx[ i ] | u(v) |
| else |  |
| cpb_removal_delay_minus1[ i ] | u(v) |
| } |  |
| } |  |
| dpb_output_delay | u(v) |
| if( decoding_unit_hrd_params_present_flag ) |  |
| pic_dpb_output_du_delay | u(v) |
| if( decoding_unit_hrd_params_present_flag && decoding_unit_cpb_params_in_pic_timing_sei_flag ) { |  |
| num_decoding_units_minus1 | ue(v) |
| du_common_cpb_removal_delay_flag | u(1) |
| if( du_common_cpb_removal_delay_flag ) |  |
| du_common_cpb_removal_delay_increment_minus1 | u(v) |
| for( i = 0; i <= num_decoding_units_minus1; i++ ) { |  |
| num_nalus_in_du_minus1[ i ] | ue(v) |
| if( !du_common_cpb_removal_delay_flag && i < num_decoding_units_minus1 ) |  |
| du_cpb_removal_delay_increment_minus1[ i ] | u(v) |
| } |  |
| } |  |
| } |  |

cpb_removal_delay_delta_enabled[ i ] equal to 1 specifies that cpb_removal_delay_delta_idx[ i ] is present in the picture timing SEI message. cpb_removal_delay_delta_enabled[ i ] equal to 0 specifies that cpb_removal_delay_delta_idx[ i ] is not present in the picture timing SEI message. When not present, the value of cpb_removal_delay_delta_enabled[ i ] is infered to be equal to 0.

Alternatively following conformance constraint is proposed:

When cpb_removal_delay_deltas_present_flag is equal to 0, cpb_removal_delay_delta_enabled_flag[ i ] shall be equal to 0 for i in the range of TemporalId to pt_max_sub_layers_minus1−1, inclusive.

## Proposal 3

It is asserted that the coding of cpb_removal_delay_delta_idx[ i ] is incompletely specified in VVC WD 6.

The syntax element cpb_removal_delay_delta_idx[ i ] is coded as u(v) however the number of bits used for coding it are not specified. It is proposed to completely specify the coding of cpb_removal_delay_delta_idx[ i ].

Option A:

cpb_removal_delay_delta_idx[ i ] specifies the index of the CPB removal delta that applies to HighestTid equal to i in the list of cpb_removal_delay_delta[ j ] for j ranging from 0 to num_cpb_removal_delay_deltas_minus1, inclusive. The number of bits used to represent the syntax element cpb_removal_delay_delta_idx[ i ] is equal to Ceil( Log2( num_cpb_removal_delay_deltas_minus1+1) ) , inclusive.

Option B:

Alternatively, the coding of cpb_removal_delay_delta_idx[ i ] is changed from u(v) to ue(v) coding as follows:

| cpb_removal_delay_delta_idx[ i ] | u(v)ue(v) |
| --- | --- |

# Conclusion

This document proposes modifications to picture timing information signaling in VVC. It is proposed to adopt the proposals in the next version of VVC WD.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-O2001-vE, Versatile Video Coding (Draft 6), Gothenburg, July 2019