| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0184-v1 |
| --- | --- |

| Title: | On Decoding Process for Unavailable Reference Pictures and Reference Picture List Construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Following is proposed:

- Proposal 1: Modifications are proposed to the general decoding process for unavailable reference pictures and decoding process for reference picture list construction. It is asserted that these are necessary for correct handling of inter-layer reference pictures signalling in reference picture lists.

- Proposal 2: A conformance constraint in decoding process for reference picture list construction related to set of unique pictures referred by referenc picture lists 0 and 1 is proposed to be modified as a bug-fix.

# Introduction

VVC WD [1] includes specification text on general decoding process for generating unavailable reference pictures and decoding process for reference picture list construction.

Reference picture list signaling in VVC WD 6 now support providing inter-layer reference picture information. It is asserted that modifications are necessary to the general decoding process for unavailable reference pictures and decoding process for reference picture list construction for correct handling of inter-layer reference pictures signaling in reference picture lists.

Following is proposed:

- Proposal 1: Modifications are proposed to the general decoding process for unavailable reference pictures and decoding process for reference picture list construction. It is asserted that these are necessary for correct handling of inter-layer reference pictures signalling in reference picture lists.

- Proposal 2: A conformance constraint in decoding process for reference picture list construction related to set of unique pictures referred by referenc picture lists 0 and 1 is proposed to be modified as a bug-fix.

# Proposals

## Proposal 1

Modifications are proposed to the sections related to general decoding process for unavailable reference pictures and decoding process for reference picture list construction. It is asserted that these are necessary for correct handling of inter-layer reference pictures signaling in reference picture lists.

The proposed changes related to proposal 1 are shown highlighted with respect to VVC WD 6.

### 8.3.4 Decoding process for generating unavailable reference pictures

#### 8.3.4.1 General decoding process for generating unavailable reference pictures

This process is invoked once per coded picture when the current picture is a CRA picture with NoIncorrectPicOutputFlag equal to 1 or a GDR picture with NoIncorrectPicOutputFlag equal to 1.

When this process is invoked, the following applies:

- For each 	RefPicList[ i ][ j ], with i in the range of 0 to 1, inclusive, and j in the range of 0 to num_ref_entries[ i ][ RplsIdx[ i ] ] − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in subclause 8.3.4.2 and the following applies:

- If st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 1 and inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ]   is equal to 0, the value of PicOrderCntVal for the generated picture is set equal to RefPicPocList[ i ][ j ] and the generated picture is marked as "used for short-term reference".

- Otherwise if (st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 0) and inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] is equal to 0, the value of PicOrderCntVal for the generated picture is set equal to RefPicLtPocList[ i ][ j ], the value of slice_pic_order_cnt_lsb for the generated picture is inferred to be equal to ( RefPicLtPocList[ i ][ j ] & ( MaxPicOrderCntLsb – 1 ) ), and the generated picture is marked as "used for long-term reference".

- Otherwise (if inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] is equal to 1), the value of PicOrderCntVal for the generated picture is set equal to RefPicILRPPocList[ i ][ j ], the value of slice_pic_order_cnt_lsb for the generated picture is inferred to be equal to ( RefPicILRPPocList[ i ][ j ] & ( MaxPicOrderCntLsb – 1 ) ), and the generated picture is marked as "used for long-term reference".

After all slices of the current picture have been decoded, this generated picture is marked as "used for short-term reference".

- If inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] is equal to 1 the value of nuh_layer_id for the generated picture is set equal to RefPicLayerId[ i ][ j ].

- Otherwise tThe value of nuh_layer_id for the generated picture is set equal to nuh_layer_id of the current picture.’

### 8.3.2 Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each slice of a non-IDR picture.

Reference pictures are addressed through reference indices. A reference index is an index into a reference picture list. When decoding an I slice, no reference picture list is used in decoding of the slice data. When decoding a P slice, only reference picture list 0 (i.e., RefPicList[ 0 ]), is used in decoding of the slice data. When decoding a B slice, both reference picture list 0 and reference picture list 1 (i.e., RefPicList[ 1 ]) are used in decoding of the slice data.

At the beginning of the decoding process for each slice of a non-IDR picture, the reference picture lists RefPicList[ 0 ] and RefPicList[ 1 ] are derived. The reference picture lists are used in marking of reference pictures as specified in clause 8.3.3 or in decoding of the slice data.

NOTE 1 – For an I slice of a non-IDR picture that it is not the first slice of the picture, RefPicList[ 0 ] and RefPicList[ 1 ] may be derived for bitstream conformance checking purpose, but their derivation is not necessary for decoding of the current picture or pictures following the current picture in decoding order. For a P slice that it is not the first slice of a picture, RefPicList[ 1 ] may be derived for bitstream conformance checking purpose, but its derivation is not necessary for decoding of the current picture or pictures following the current picture in decoding order.

The reference picture lists RefPicList[ 0 ] and RefPicList[ 1 ] are constructed as follows:

for( i = 0; i < 2; i++ ) {
		for( j = 0, k = 0, pocBase = PicOrderCntVal; j < num_ref_entries[ i ][ RplsIdx[ i ] ]; j++) {
			if( !inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] ) {
				if( st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] ) {
					RefPicPocList[ i ][ j ] = pocBase − DeltaPocValSt[ i ][ RplsIdx[ i ] ][ j ]
					if( there is a reference picture picA in the DPB with the same nuh_layer_id as the current picture
								and 	PicOrderCntVal equal to RefPicPocList[ i ][ j ] )
						RefPicList[ i ][ j ] = picA
					else
						RefPicList[ i ][ j ] = "no reference picture"				(8-5)
					pocBase = RefPicPocList[ i ][ j ]
				} else {
					if( !delta_poc_msb_cycle_lt[ i ][ k ] ) {
						if( there is a reference picA in the DPB with the same nuh_layer_id as the current picture and
									PicOrderCntVal & ( MaxPicOrderCntLsb − 1 ) equal to PocLsbLt[ i ][ k ] )
							RefPicList[ i ][ j ] = picA
						else
							RefPicList[ i ][ j ] = "no reference picture"
						RefPicLtPocList[ i ][ j ] = PocLsbLt[ i ][ k ]
					} else {
						if( there is a reference picA in the DPB with the same nuh_layer_id as the current picture and
									PicOrderCntVal equal to 	FullPocLt[ i ][ k ] )
							RefPicList[ i ][ j ] = picA
						else
							RefPicList[ i ][ j ] = "no reference picture"
						RefPicLtPocList[ i ][ j ] = FullPocLt[ i ][ k ]
				}
					k++
			}
			} else {
				layerIdx = 	DirectDependentLayerIdx [ GeneralLayerIdx[ nuh_layer_id ] ][ ilrp_idc[ i ][ RplsIdx ][ j ] ]
				RrefPicLayerId[ i ][ j ]  = vps_layer_id[ layerIdx ]
				if( there is a reference picture picA in the DPB with nuh_layer_id equal to RrefPicLayerId[ i ][ j ]   and
							the same PicOrderCntVal as the current picture )
					RefPicList[ i ][ j ] = picA
				else
					RefPicList[ i ][ j ] = "no reference picture"
				RefPicILRPPocList[ i ][ j ] = PicOrderCntVal
		}
	}
}

For each i equal to 0 or 1, the first NumRefIdxActive[ i ] entries in RefPicList[ i ] are referred to as the active entries in RefPicList[ i ], and the other entries in RefPicList[ i ] are referred to as the inactive entries in RefPicList[ i ].

## Proposal 2

A conformance constraint in decoding process for reference picture list construction related to set of unique pictures referred by referenc picture lists 0 and 1 is proposed to be modified as a bug-fix.

It is asserted that without these modification, it is unclear which sps max decobder pic buffering value from the array to use for the conformance constraint.

The proposed changes related to proposal 2 are shown highlighted with respect to VVC WD5.

### Decoding process for reference picture lists construction

It is a requirement of bitstream conformance that the following constraints apply:

- For each i equal to 0 or 1, num_ref_entries[ i ][ RplsIdx[ i ] ] shall not be less than NumRefIdxActive[ i ].

- The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] shall be present in the DPB and shall have TemporalId less than or equal to that of the current picture.

- The picture referred to by each entry in RefPicList[ 0 ] or RefPicList[ 1 ] shall not be the current picture and shall have non_reference_picture_flag equal to 0.

- An STRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice of a picture and an LTRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of the same slice or a different slice of the same picture shall not refer to the same picture.

- There shall be no LTRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] for which the difference between the PicOrderCntVal of the current picture and the PicOrderCntVal of the picture referred to by the entry is greater than or equal to 224.

- Let setOfRefPics be the set of unique pictures referred to by all entries in RefPicList[ 0 ] that have the same nuh_layer_id as the current picture and all entries in RefPicList[ 1 ] that have the same nuh_layer_id as the current picture. The number of pictures in setOfRefPics shall be less than or equal to sps_max_dec_pic_buffering_minus1[ HighestTid ] and setOfRefPics shall be the same for all slices of a picture.

- When the current picture is an STSA picture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that has TemporalId equal to that of the current picture.

- When the current picture is a picture that follows, in decoding order, an STSA picture that has TemporalId equal to that of the current picture, there shall be no picture that has TemporalId equal to that of the current picture included as an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the STSA picture in decoding order.

- The picture referred to by each ILRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice of the current picture shall be in the same access unit as the current picture.

- The picture referred to by each ILRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice of the current picture shall be present in the DPB and shall have nuh_layer_id less than that of the current picture.

- Each ILRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice shall be an active entry.

### 8.3.4 Conclusion

This document proposes modifications to decoding process for unavailable reference pictures and for reference picture list construction in VVC. Additionally a conformance constraint is modified as a bug-fix. It is proposed to adopt the proposals in the next version of VVC WD.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-O2001-vE, Versatile Video Coding (Draft 6), Gothenburg, July 2019