| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0188 |
| --- | --- |

| Title: | On constraint on the maximum bits for CU/CTU |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki Yoshitaka Morigami 2-10-1 Osaki, Shinagawa-ku Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 teruhiko.s@sony.com Yoshitaka.Morigami@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes the maximum bits for CU/CTU and for picture. The proposed constraints are,

- Constraint on the maximum bits for CU/CTU

  - Constraint on the maximum bits for Min CU

  - Constraint on the maximum bits for CTU

- Constraint on the maximum bits for picture

In HEVC, the maximum bits for Min CU is constraint, and the information is signaled in VUI. The constraints are to reduce the worst case of bits at local area for cost feasible implementation. This contribution proposes to define the constraint on the maximum bits for MinCU for VVC and proposes related syntax elements in VUI.

Introduction

In HEVC, the maximum bits for Min CU is constraint, and the information is signaled in VUI. The constraints are to reduce the worst case of bits at local area for cost feasible implementation. This contribution proposes to define the constraint on the maximum bits for MinCU for VVC and proposes related syntax elements in VUI.

In HEVC, two constraints are defined. One is for the maximum bits for MinCU and the other is the maximum bits per picture. To apply the same constraints to VVC, the constraints for MinCU needs to be extended to support DualTree. This contribution proposes the method to constrain the maximum bits for MinCU considering DualTree.

Proposed

This contribution proposes the maximum bits for CU/CTU and for picture. The proposed constraints are,

- Constraint on the maximum bits for CU/CTU

  - Constraint on the maximum bits for Min CU

  - Constraint on the maximum bits for CTU

- Constraint on the maximum bits for picture

Constraint on the maximum bits for CU/CTU

In HEVC, the maximum bits for CU is

- To derive maximum bits for MinCU (RAWMinCuBits)

- And then to scale depending on the CU size

The constraint is defined as follows.

Maximum bits for coding_unit():
	( 128 + RawMinCuBits)  max_bits_per_min_cu_denom * ( 1 << (2*(log2CbSize-MinCbLog2SizeY)))
	RawMinCuBits = MinCbSizeY * MinCbSizeY * ( BitDepthY + 2 * BitDepthC / ( SubWidthC * SubHeightC ) )

Since HEVC has Single Tree structure, both luma and chroma information are included in one coding_unit(). Therefore, RawMinCuBits includes both luma and chroma.

In VVC, RawMinCuBits can be derived same as HEVC for Single Tree (RawMinCuBits includes both luma and chroma). However, in case of Dual Tee, coding_unit() includes luma or chroma only. Therefore, the derivationof RawMinCuBits needs to be extended.

This contribution proposes the following two methods.

Method 1: Constraint on the maximum bits for MinCU considering Dual Tree

Method 2: Constraint on the maximum bits for CTU

Constraint on the maximum bits for MinCU considering DualTree

In this method, the constraint on the maximum bits for MinCU is derived as follows.

(The yellow highlighted part is the same as HEVC. The magenta highlighted part is the extension for VVC)

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = 255 ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| } |  |
| colour_description_present_flag | u(1) |
| if( colour_description_present_flag ) { |  |
| colour_primaries | u(8) |
| transfer_characteristics | u(8) |
| matrix_coeffs | u(8) |
| full_range_flag | u(1) |
| } |  |
| field_seq_flag | u(1) |
| chroma_loc_info_present_flag | u(1) |
| if( chroma_loc_info_present_flag ) { |  |
| if( !field_seq_flag ) { |  |
| chroma_sample_loc_type_frame | ue(v) |
| else { |  |
| chroma_sample_loc_type_top_field | ue(v) |
| chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| } |  |
| overscan_info_present_flag | u(1) |
| if( overscan_info_present_flag ) |  |
| overscan_appropriate_flag | u(1) |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| max_bits_per_mincu_denom | ue(v) |
| } |  |
| } |  |

bitstream_restriction_flag equal to 1, specifies that the bitstream restriction parameters for the CVS are present. bitstream_restriction_flag equal to 0, specifies that the bitstream restriction parameters for the CVS are not present.

max_bits_per_min_cu_denom indicates an upper bound for the number of coded bits of coding_unit( ) data for any coding block in any picture of the CVS. The value of max_bits_per_min_cu_denom shall be in the range of 0 to 16, inclusive.

Depending on the value of max_bits_per_min_cu_denom, the following applies:

–	If max_bits_per_min_cu_denom is equal to 0, no limit is specified by this syntax element.

–	Otherwise (max_bits_per_min_cu_denom is not equal to 0), it is a requirement of bitstream conformance that no coded coding_unit( ) shall be represented in the bitstream by more than the following number of bits:

- If treeType is eaual to SINGLE_TREE

( 128 + RawMinCuBits )  max_bits_per_min_cu_denom * ( 1<< ( ( Log2(cbWidth) − MinCbLog2SizeY ) + ( ( Log2(cbHeight) − MinCbLog2SizeY )))

- Otherwise if treeType is eaual to DUAL_TREE_LUMA

( 128 + RawMinCuBitsY )  max_bits_per_min_cu_denom * ( 1<< ( ( Log2(cbWidth) − MinCbLog2SizeY ) + ( ( Log2(cbHeight) − MinCbLog2SizeY )))

RawMinCuBitsY = MinCbSizeY * MinCbSizeY * BitDepthY

- Otherwise if treeType is eaual to DUAL_TREE_CHROMA

( 128 + RawMinCuBitsC )  max_bits_per_min_cu_denom * ( 1<< ( ( Log2(cbWidth) − MinCbLog2SizeY ) + ( ( Log2(cbHeight) − MinCbLog2SizeY )))

RawMinCuBitsC = MinCbSizeY * MinCbSizeY *(2*BitDepthC/(SubWidthC*SubHeightC))

where log2CbSize is the value of log2CbSize for the given coding block and the number of bits of coding_unit( ) data for the same coding block is given by the number of times read_bits( 1 )

When the max_bits_per_min_cu_denom is not present, the value of max_bits_per_min_cu_denom is inferred to be equal to 1

Constraint on the maximum bits for CTU

In this method, the constraint on the maximum bits for CTU is derived as follows.

(The yellow highlighted part is the same as HEVC. The magenta highlighted part is the extension for VVC)

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = 255 ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| } |  |
| colour_description_present_flag | u(1) |
| if( colour_description_present_flag ) { |  |
| colour_primaries | u(8) |
| transfer_characteristics | u(8) |
| matrix_coeffs | u(8) |
| full_range_flag | u(1) |
| } |  |
| field_seq_flag | u(1) |
| chroma_loc_info_present_flag | u(1) |
| if( chroma_loc_info_present_flag ) { |  |
| if( !field_seq_flag ) { |  |
| chroma_sample_loc_type_frame | ue(v) |
| else { |  |
| chroma_sample_loc_type_top_field | ue(v) |
| chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| } |  |
| overscan_info_present_flag | u(1) |
| if( overscan_info_present_flag ) |  |
| overscan_appropriate_flag | u(1) |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| max_bits_per_ctu_denom | ue(v) |
| } |  |
| } |  |

bitstream_restriction_flag equal to 1, specifies that the bitstream restriction parameters for the CVS are present. bitstream_restriction_flag equal to 0, specifies that the bitstream restriction parameters for the CVS are not present.

max_bits_per_ctu_denom indicates an upper bound for the number of coded bits of dual_tree_implicit_qt_split ( xCtb, yCtb, CtbSizeY, 0 ) or coding_tree( xCtb, yCtb, CtbSizeY, CtbSizeY, 1, 1, 0, 0, 0, 0, 0, SINGLE_TREE, MODE_TYPE_ALL ) data for any coding block in any picture of the CVS. The value of max_bits_per_min_cu_denom shall be in the range of 0 to 16, inclusive.

Depending on the value of max_bits_per_min_cu_denom, the following applies:

–	If max_bits_per_min_cu_denom is equal to 0, no limit is specified by this syntax element.

–	Otherwise (max_bits_per_min_cu_denom is not equal to 0), it is a requirement of bitstream conformance that no coded dual_tree_implicit_qt_split ( xCtb, yCtb, CtbSizeY, 0 ) or coding_tree( xCtb, yCtb, CtbSizeY, CtbSizeY, 1, 1, 0, 0, 0, 0, 0, SINGLE_TREE, MODE_TYPE_ALL ) shall be represented in the bitstream by more than the following number of bits:

( 128 + RawCtuBits )  max_bits_per_ctu_denom

When the max_bits_per_ctb_denom is not present, the value of max_bits_per_min_cu_denom is inferred to be equal to 1.

The variable RawCtuBits is derived as follows:

RawCtuBits = CtbSizeY * CtbSizeY * BitDepthY + 2 * ( CtbWidthC * CtbHeightC ) * BitDepthC

Constraint on the maximum bits for Picture

In this method, the constraint on the maximum bits for Picture is derived as follows. This method is the same as HEVC.

(The yellow highlighted part is the same as HEVC.)

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = 255 ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| } |  |
| colour_description_present_flag | u(1) |
| if( colour_description_present_flag ) { |  |
| colour_primaries | u(8) |
| transfer_characteristics | u(8) |
| matrix_coeffs | u(8) |
| full_range_flag | u(1) |
| } |  |
| field_seq_flag | u(1) |
| chroma_loc_info_present_flag | u(1) |
| if( chroma_loc_info_present_flag ) { |  |
| if( !field_seq_flag ) { |  |
| chroma_sample_loc_type_frame | ue(v) |
| else { |  |
| chroma_sample_loc_type_top_field | ue(v) |
| chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| } |  |
| overscan_info_present_flag | u(1) |
| if( overscan_info_present_flag ) |  |
| overscan_appropriate_flag | u(1) |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| max_bytes_per_pic_denom | ue(v) |
| } |  |
| } |  |

bitstream_restriction_flag equal to 1, specifies that the bitstream restriction parameters for the CVS are present. bitstream_restriction_flag equal to 0, specifies that the bitstream restriction parameters for the CVS are not present.

max_bytes_per_pic_denom indicates a number of bytes not exceeded by the sum of the sizes of the VCL NAL units associated with any coded picture in the CVS.

The number of bytes that represent a picture in the NAL unit stream is specified for this purpose as the total number of bytes of VCL NAL unit data (i.e. the total of the NumBytesInNalUnit variables for the VCL NAL units) for the picture. The value of max_bytes_per_pic_denom shall be in the range of 0 to 16, inclusive.

Depending on the value of max_bytes_per_pic_denom the following applies:

–	If max_bytes_per_pic_denom is equal to 0, no limits are indicated.

–	Otherwise (max_bytes_per_pic_denom is not equal to 0), it is a requirement of bitstream conformance that no coded picture shall be represented in the CVS by more than the following number of bytes.

( PicSizeInCtbY * RawCtuBits )  ( 8 * max_bytes_per_pic_denom )	(E-53)

When the max_bytes_per_pic_denom syntax element is not present, the value of max_bytes_per_pic_denom is inferred to be equal to 2.

Conclusion

This contribution proposes the maximum bits for CU/CTU and for picture. The proposed constraints are,

- Constraint on the maximum bits for CU/CTU

  - Constraint on the maximum bits for Min CU

  - Constraint on the maximum bits for CTU

- Constraint on the maximum bits for picture

Those constraints are necessary for the implementation with reasonable cost and therefore, this should be included in VVC.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).