| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0189-v1 |
| --- | --- |

| Title: | AHG8/AHG17: On buffering period, picture timing, and decoding unit information SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang San Diego, CA, USA | Email: | ye-kui.wang@futurewei.com |
| Source: | Futurewei Technologies, Inc. |  |  |

_____________________________

# Abstract

This contribution includes the following three aspects:

- A proposal of changes to the PT and DUI SEI messages for adding picture-level CPB parameters needed for DU based HRD operations

- A discussion on parsing dependency of the BP, PT, and DUI SEI messages

- A discussion of temporal scalability support of the BP, PT, and DUI SEI messages

# CPB parameters for sub-layers for the DU based HRD operation

Currently, picture-level CPB parameters needed for AU based HRD operations for both entire layers and sub-layers are signalled in PT SEI messages. Picture-level CPB parameters needed for DU based HRD operations for entire layers are signalled in either PT or DUI SEI messages. However, picture-level CPB parameters needed for DU based HRD operations for sub-layers are missing.

The following changes are proposed for adding picture-level CPB parameters needed for DU based HRD operations for sub-layers:

## Picture timing SEI message syntax

| pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| pt_max_sub_layers_minus1 | u(3) |
| cpb_removal_delay_minus1[ pt_max_sub_layers_minus1 ] | u(v) |
| for( i = TemporalId; i < pt_max_sub_layers_minus1; i++ ) { |  |
| sub_layer_delays_present_flag[ i ] | u(1) |
| if( sub_layer_delays_present_flag[ i ] ) { |  |
| cpb_removal_delay_delta_enabled_flag[ i ] | u(1) |
| if( cpb_removal_delay_delta_enabled_flag[ i ] ) |  |
| cpb_removal_delay_delta_idx[ i ] | u(v) |
| else |  |
| cpb_removal_delay_minus1[ i ] | u(v) |
| } |  |
| } |  |
| dpb_output_delay | u(v) |
| if( decoding_unit_hrd_params_present_flag ) |  |
| pic_dpb_output_du_delay | u(v) |
| if( decoding_unit_hrd_params_present_flag && decoding_unit_cpb_params_in_pic_timing_sei_flag ) { |  |
| num_decoding_units_minus1 | ue(v) |
| du_common_cpb_removal_delay_flag | u(1) |
| if( du_common_cpb_removal_delay_flag ) |  |
| for( i = TemporalId; i < pt_max_sub_layers_minus1; i++ ) |  |
| du_common_cpb_removal_delay_increment_minus1[ i ] | u(v) |
| for( i = 0; i <= num_decoding_units_minus1; i++ ) { |  |
| num_nalus_in_du_minus1[ i ] | ue(v) |
| if( !du_common_cpb_removal_delay_flag && i < num_decoding_units_minus1 ) |  |
| for( j = TemporalId; j < pt_max_sub_layers_minus1; j++ ) |  |
| du_cpb_removal_delay_increment_minus1[ i ][ j ] | u(v) |
| } |  |
| } |  |
| } |  |

## Picture timing SEI message semantics

The picture timing SEI message provides CPB removal delay and DPB output delay information for the access unit associated with the SEI message.

...

num_decoding_units_minus1 plus 1 specifies the number of decoding units in the access unit the picture timing SEI message is associated with. The value of num_decoding_units_minus1 shall be in the range of 0 to PicSizeInCtbsY − 1, inclusive.

du_common_cpb_removal_delay_flag equal to 1 specifies that the syntax elements du_common_cpb_removal_delay_increment_minus1[ i ] are present. du_common_cpb_removal_delay_flag equal to 0 specifies that the syntax elements du_common_cpb_removal_delay_increment_minus1[ i ] are not present.

du_common_cpb_removal_delay_increment_minus1[ i ] plus 1 specifies the duration, in units of clock sub-ticks (see clause C.1), between the nominal CPB removal times of any two consecutive decoding units in decoding order in the access unit associated with the picture timing SEI message when Htid i equal to i. This value is also used to calculate an earliest possible time of arrival of decoding unit data into the CPB for the HSS, as specified in Annex C. The lenght of this syntax element is du_cpb_removal_delay_increment_length_minus1 + 1 bits.

num_nalus_in_du_minus1[ i ] plus 1 specifies the number of NAL units in the i-th decoding unit of the access unit the picture timing SEI message is associated with. The value of num_nalus_in_du_minus1[ i ] shall be in the range of 0 to PicSizeInCtbsY − 1, inclusive.

The first decoding unit of the access unit consists of the first num_nalus_in_du_minus1[ 0 ] + 1 consecutive NAL units in decoding order in the access unit. The i-th (with i greater than 0) decoding unit of the access unit consists of the num_nalus_in_du_minus1[ i ] + 1 consecutive NAL units immediately following the last NAL unit in the previous decoding unit of the access unit, in decoding order. There shall be at least one VCL NAL unit in each decoding unit. All non-VCL NAL units associated with a VCL NAL unit shall be included in the same decoding unit as the VCL NAL unit.

du_cpb_removal_delay_increment_minus1[ i ][ j ] plus 1 specifies the duration, in units of clock sub-ticks, between the nominal CPB removal times of the ( i + 1 )-th decoding unit and the i-th decoding unit, in decoding order, in the access unit associated with the picture timing SEI message when Htid i equal to j. This value is also used to calculate an earliest possible time of arrival of decoding unit data into the CPB for the HSS, as specified in Annex C. The lenght of this syntax element is du_cpb_removal_delay_increment_length_minus1 + 1 bits.

## Decoding unit information SEI message syntax

| decoding_unit_info( payloadSize ) { | Descriptor |
| --- | --- |
| decoding_unit_idx | ue(v) |
| dui_max_sub_layers_minus1 | u(3) |
| if( !decoding_unit_cpb_params_in_pic_timing_sei_flag ) |  |
| for( i = TemporalId; i < dui_max_sub_layers_minus1; i++ ) |  |
| du_spt_cpb_removal_delay_increment[ i ] | u(v) |
| dpb_output_du_delay_present_flag | u(1) |
| if( dpb_output_du_delay_present_flag ) |  |
| pic_spt_dpb_output_du_delay | u(v) |
| } |  |

### Decoding unit information SEI message semantics

The decoding unit information SEI message provides CPB removal delay information for the decoding unit associated with the SEI message.

...

The set of NAL units associated with a decoding unit information SEI message consists, in decoding order, of the SEI NAL unit containing the decoding unit information SEI message and all subsequent NAL units in the access unit up to but not including any subsequent SEI NAL unit containing a decoding unit information SEI message with a different value of decoding_unit_idx. Each decoding unit shall include at least one VCL NAL unit. All non-VCL NAL units associated with a VCL NAL unit shall be included in the decoding unit containing the VCL NAL unit.

The TemporalId in the decoding unit information SEI message syntax is the TemporalId of the SEI NAL unit containing the decoding unit information SEI message.

decoding_unit_idx specifies the index, starting from 0, to the list of decoding units in the current access unit, of the decoding unit associated with the decoding unit information SEI message. The value of decoding_unit_idx shall be in the range of 0 to PicSizeInCtbsY − 1, inclusive.

A decoding unit identified by a particular value of duIdx includes and only includes all NAL units associated with all decoding unit information SEI messages that have decoding_unit_idx equal to duIdx. Such a decoding unit is also referred to as associated with the decoding unit information SEI messages having decoding_unit_idx equal to duIdx.

For any two decoding units duA and duB in one access unit with decoding_unit_idx equal to duIdxA and duIdxB, respectively, where duIdxA is less than duIdxB, duA shall precede duB in decoding order.

A NAL unit of one decoding unit shall not be present, in decoding order, between any two NAL units of another decoding unit.

dui_max_sub_layers_minus1 plus 1 specifies the the TemporalId of the highest sub-layer representation for which the CPB removal delay information is contained in the decoding unit information SEI message. The value of dui_max_sub_layers_minus1 shall be in the range of 0 to vps_max_sub_layers_minus1, inclusive.

du_spt_cpb_removal_delay_increment[ i ] specifies the duration, in units of clock sub-ticks, between the nominal CPB times of the last decoding unit in decoding order in the current access unit and the decoding unit associated with the decoding unit information SEI message when Htid i equal to i. This value is also used to calculate an earliest possible time of arrival of decoding unit data into the CPB for the HSS, as specified in Annex C. The lenght of this syntax element is du_cpb_removal_delay_increment_length_minus1 + 1. When the decoding unit associated with the decoding unit information SEI message is the last decoding unit in the current access unit, the value of du_spt_cpb_removal_delay_increment[ i ] shall be equal to 0.

...

# A discussion on temporal scalability support

If the above proposal is adopted, then we would have loops for sub-layer CPB parameters in all the BP/PT/DUI SEI messages.

In HEVC, such loops for sub-layers do not exist in the BP/PT/DUI SEI messages. Support of picture-level CPB parameters in HEVC is realized the temporal nesting capability of the scalable nesting SEI message.

A disadvantage of putting the loop into the BP/PT/DUI SEI messages is that after discarding some higher sub-layers, the information for those discarded higher sub-layers in the AUs with lower TemporalId values would there be included in the BP/PT/DUI SEI messages, and become junk bits. This can be avoided by using the temporal nesting capability of the scalable nesting SEI message in HEVC.

Should we have a scalable nesting SEI message that has the temporal nesting capability, and don't include loops of sub-layers in the BP/PT/DUI SEI messages? If yes, then the proposal above is not needed, and the existing loops in the BP/PT are also removed.

# A discussion on parsing dependency

Currently there are some parsing dependencies of the BP, PT, and DUI SEI messages on VPS or SPS, through the use of the following VPS or SPS syntax elements in the syntax of one or more of these SEI messages:

- The syntax element decoding_unit_hrd_params_present_flag in the BP SEI message syntax

- The syntax elements decoding_unit_hrd_params_present_flag and decoding_unit_cpb_params_in_pic_timing_sei_flag in the PT SEI message syntax

- The syntax element decoding_unit_cpb_params_in_pic_timing_sei_flag in the DUI SEI message

Note also that there are parsing dependencies of at least the PT SEI message on the BP SEI message. For example, the lengths of some PT SEI message syntax elements are represented by the syntax element cpb_removal_delay_length_minus1, dpb_output_delay_length_minus1, du_cpb_removal_delay_increment_length_minus1, and dpb_output_delay_du_length_minus1 that are present in the BP SEI. However, the signalling of the pt_max_sub_layers_minus1 seems for avoiding parsing dependency of the PT SEI message on the BP SEI message.

Shouldn't we simply add SPS ID into the BP SEI message syntax, as in AVC and HEVC, and allow for all kinds of parsing dependencies of the BP/PT/DUI SEI messages on SPS/VPS, and also allow parsing dependency of PT/DUI SEI messages on the BP SEI message?

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).