| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0192-v1 |
| --- | --- |

| Title: | CE5-related: Reducing the number of luma filters in ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Olena Chubach, Ching-Yeh Chen, Tzu-Der Chuang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {olena.chubach, chingyeh.chen, peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VTM6.0 adaptive loop filter (ALF), the number of luma filters is in the range of 0 to 25, inclusive, andafter the previous Gothenburg meeting, the number of chroma filters is increased from 1 to 8. As a result, the memory required for storing all ALF coefficients is considereably increased. In this contribution, it is proposed to reduce the maximum number of luma filters signalled for ALF from 25 to 16. It is asserted that proposed method allows to reduce the required on-chip memory, while keeping flexibility of 25 initial luma filter classes, since there is no constraint on the ways of combining 25 luma filter classes into 16 different merged classes.

BD-rates are reported as follows.
AI: 0.00% (Y), 0.00% (Cb), 0.00% (Cr)
RA: 0.00% (Y), 0.01% (Cb), -0.01% (Cr)
LB: 0.00% (Y), 0.03% (Cb), 0.04% (Cr)

# Introduction

In VTM6.0 ALF, alf_luma_num_filters_signalled_minus1 is used to indicate the number of filters signalled in one luma filter set. The value of alf_luma_num_filters_signalled_minus1 shall be in the range of 0 to NumAlfFilters − 1, inclusive, where NumAlfFilters is 25. Therefore, the maximum number of luma filters is equal to the maximum number of luma filter classes.

# Reduced number of luma filters

In VTM6.0 ALF, filter coefficients are signalled in one adaptation parameter set (APS). One APS includes one luma filter set (consisting of up to 25 luma filters) and up to 8 chroma filters. One slice can refer up to 6 APSs, and up to 8 APSs can be activated in one picture at the same time. Therefore, the required memory storage of one APS is 435 bytes (25 luma filters * 12 coefficients per luma filter * (1 byte per coefficient + 2 bits per clipping index) + 8 chroma filters * 6 coefficients per chroma filter * (1 byte per coefficient + 2 bits per clipping index)). If ALF is applied at the decoder to each slice sequentially, the required on-chip memory size is 2610 bytes (435 bytes per APS * 6 APSs). If ALF is applied at the decoder to slices in the raster scan of one picture, the required on-chip memory size becomes 3480 bytes (435 bytes per APS * 8 APSs).

In order to reduce the amount of the required on-chip memory, it is proposed to constrain the number of filters in one filter set. This way, the required on-chip memory can be reduced to (MaxNumLumaFilters * 12 coefficients per luma filter * (1 byte per coefficient + 2 bits per clipping index) + 8 chroma filters * 6 coefficients per chroma filter * (1 byte per coefficient + 2 bits per clipping index) \) per APS. It is proposed to assign MaxNumLumaFilters to 16. Then the required on-chip memory size can be reduced by 31%: from 2610 bytes to 1800 bytes in the slice-level pipeline or from 3480 to 2400 bytes in the picture-level pipeline.

The related VVC specification text modification is as follows.

| alf_data( ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| alf_chroma_filter_signal_flag | u(1) |
| if( alf_luma_filter_signal_flag ) { |  |
| alf_luma_clip_flag | u(1) |
| alf_luma_num_filters_signalled_minus1 | ue(v) |
| sixteen_minus_alf_num_signalled_filters | tb(v) |
| if(AlfNumSignalledFilters > 1 alf_luma_num_filters_signalled_minus1 > 0 ) { |  |
| for( filtIdx = 0; filtIdx < NumAlfFilters; filtIdx++ ) |  |
| alf_luma_coeff_delta_idx[ filtIdx ] | u(v) |
| } |  |
| alf_luma_coeff_signalled_flag | u(1) |
| if( alf_luma_coeff_signalled_flag ) { |  |
| for( sfIdx = 0; sfIdx <= AlfNumSignalledFilters-1 alf_luma_num_filters_signalled_minus1; sfIdx++ ) |  |
| alf_luma_coeff_flag[ sfIdx ] | u(1) |
| } |  |
| for( sfIdx = 0; sfIdx <= AlfNumSignalledFilters-1 alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| alf_luma_coeff_abs[ sfIdx ][ j ] | uek(v) |
| if( alf_luma_coeff_abs[ sfIdx ][ j ] ) |  |
| alf_luma_coeff_sign[ sfIdx ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( alf_luma_clip_flag ) { |  |
| for( sfIdx = 0; sfIdx <= AlfNumSignalledFilters-1 alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | u(2) |
| } |  |
| } |  |
| } |  |
| } |  |

alf_luma_num_filters_signalled_minus1 plus 1 specifies the number of adpative loop filter classes for which luma coefficients can be signalled. The value of alf_luma_num_filters_signalled_minus1 shall be in the range of 0 to NumAlfFilters − 1, inclusive.

The maximum value maxVal of the truncated binary binarization tb(v) is set equal to NumAlfFilters − 1.

sixteen_minus_alf_num_signalled_filters specifies the number of adpative loop filter classes for which luma coefficients can be signalled in the slice and subtracted from 16. The maximum value maxVal of the truncated binary binarization tb(v) is set equal to 15.

The filter number of adpative loop filter classes, AlfNumSignalledFilters, is derived as follows:

AlfNumSignalledFilters = 16 − sixteen_minus_alf_num_signalled_filters

The value of AlfNumSignalledFilters shall be in the range of 1 to 16, inclusive.

# Experimental results

The proposed approach was implemented on VTM6.0 [1]. The simulations were performed following the JVET common test conditions (CTC) [2]. Table 1 shows the BD-rates of the proposed method.

Table 1.  Results of the proposed method. Runtime data are inaccurate.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 107% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 106% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 113% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 113% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 108% |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 120% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 107% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.01% | 100% | 107% |
| Class A2 | 0.00% | 0.03% | -0.03% | 100% | 106% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 108% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 113% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.01% | 100% | 109% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 117% |
| Class F | 0.00% | 0.00% | 0.01% | 100% | 107% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.07% | 0.09% | 100% | 110% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 109% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 112% |
| Overall | 0.00% | 0.03% | 0.04% | 100% | 110% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 114% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 111% |

# References

- VTM6.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).