| ### Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1-11 October 2019 | Document: JVET-P0193 |
| --- | --- |

| Title: | CE2: Gradual Decoding Refresh (GDR) using encoder and normative restrictions (Tests 2.1.a, 2.1.b and 2.1.c) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | David Gommelet, Jean-Marc Thiesse, Didier Nicholson | Tel: Email: | +33 1 46 73 06 06 david.gommelet@vitec.com jean-marc.thiesse@vitec.com didier.nicholson@vitec.com |
| Source: | VITEC 99 rue Pierre Sémard 92320 Chatillon, France |  |  |

___________________________________

# Abstract

This contribution presents the results of CE2 tests 2.1a, 2.1.b and 2.1.c on gradual decoding refresh. In CE11-2.1.a, the Gradual Decoding Refresh (GDR) method applies motion vector (MV) and intra mode restrictions at the encoder as well as virtual boundaries with PPS signalling. Test 2.1.b adds virtual boundary signalling at slice level on top of test 2.1.a. Test 2.1.c introduces new normative changes to virtual boundaries which allow the removal of intra mode restrictions. The implementation is available on the CE2 software repository https://vcgit.hhi.fraunhofer.de/jvet-o-ce2/VVCSoftware_VTM/-/branches.

The following simulation results report BD-rate on Y, Cb and Cr components as well as encoding/decoding time over VTM5.0 using test conditions defined in CE11.

- Test 2.1.a: 	x%, x%, and x%, with x% and x% Enc/Dec Time

- Test 2.1.b: 	x%, x%, and x%, with x% and x% Enc/Dec Time

- Test 2.1.c: 	x%, x%, and x%, with x% and x% Enc/Dec Time

# Introduction

GDR or Intra Refresh (IR, also Progressive IR (PIR)) is a technique used to reduce encoder-decoder end-to-end delay. The key idea is to limit the large bitrate variations between I frames and B frames while maintaining the same random access period. Indeed, intra picture bits sizes are significantly higher than inter ones, and therefore introduce a transmission time overhead.

In the context of last meeting CE11 [1], several proposals for normative handling of GDR were presented at Gothenburg [2]. Two solutions used Constrained Intra Prediction (CIP) tool and a third one was using a dedicated intra refresh signalling. However, these solutions were put aside because managing CIP with the VVC partitioning and Intra prediction complexity was asserted to be not insignifiant, especially at the decoding side. A straightforward solution was then proposed [3] by extending the virtual boundaries concept [4] to intra prediction. This option is less complex than the support of CIP tools since its impact on intra reference pixels is similar to that of picture boundaries.

Growing interest in GDR has led to the creation of a first Core Experiment at Geneva meeting CE11 [2]. This CE was maintained at Gothenburg meeting as CE2 [5] to evaluate the solution, among others, proposed in [3]. Specific test conditions have been discussed and agreed on by all proponents. This CE main objective is to compare the various GDR methods listed in 3 categories: virtual boundary based, slice boundary based, and wave-front based. This contribution is focused on virtual boundary based methods.

# Proposed Solutions

The implementation is available on the CE2 software repository https://vcgit.hhi.fraunhofer.de/jvet-o-ce2/VVCSoftware_VTM/-/branches

## CE2-1.a: motion vector and intra mode restriction.

The solution studied in this proposal consists in periodically forcing a group of pixels (column, line, etc.) to intra prediction while using only P or B pictures. The position of these groups of pixels moves over during the encoding process while producing a clean/refreshed area in the frame, as illustrated in the following figure.

Figure 1: Intra Refresh principle (column based)

The clean region is encoded such that it can be decoded only from clean regions of the past frame. In this way, a B picture can be random-accessed and progressively reconstructed. After a full refresh cycle, the whole picture is clean or refreshed and can be displayed and a second refresh cycle can begin. In this new refresh cycle the clean region becomes the dirty one and a new clean area is progressively reconstructed.

This solution is achieved by a combination of encoder-only changes exploiting minor normative changes. It is activated in the implementation with the macro JVET_CE2_1A which activates several other macros.

The encoder-only modifications, activated through the macro JVET_IR_ENC_ONLY and JVET_IR_ENC_ONLY_INTRA_RESTRICTION, include:

- The forcing of Intra CU according to the refreshed pattern. In this test, the refresh pattern is a column, as illustrated in Figure 1. The granularity of the column is aligned on 8x8 CU basis (macro JVET_IR_SIZE_MULTIPLE) using two different column sizes over a cycle allows to suit any image size or refresh cycle. The two width of the intra column w1 and w2 are computed using the width of the video sequence and the refresh cycle period:

w1=ceilwidthperiod*8*8	and		w2=w1-8

Inside the cycle, the number of frames Nw1 using w1 and the number of frames Nw2 using w2 are computed with:

Nw1=width-period*w18 	and 	Nw2=period-Nw1

As an example, for a 416x240 video sequence, a refresh cycle period of 50 would not be achievable with only one column size. Using the proposed solution:  w1=16, w2=8, Nw1=2  and Nw2=48, meaning that the refresh cycle use 2 columns with a width of 16 pixels and 48 columns with a width of 8 pixels.

- Motion vectors constraints for the clean region such that its prediction does not use any pixels coming from the dirty region.

- Intra mode selection constraints for the clean region such that its prediction does not use any pixels coming from the dirty region.

- Removing of former reference pictures when starting a new refresh cycle.

- Deactivation of JVET_O0265_TPM_SIMPLIFICATION since it generated leaks of dirty region on the clean region.

In this CE, we use the recently introduced virtual boundaries for disabling the loop-filters [4] across the clean and dirty region to ensure that dirty pixels are not propagated into the clean region. However, several modifications were necessary in our implementation. They are activated through the macro JVET_PPS_FOR_EACH_FRAME and JVET_IR_ENC_ONLY_VIR_BOUND, and they include:

- Send PPS information for each frame. Indeed, since virtual boundaries are signalled at PPS level and since, in GDR, the virtual boundaries are updated for each frame, the PPS need to be updated for each frame.

- Slight modification to JVET_O1109_UNFIY_CRS where a Chroma residues scale is computed using the corresponding Luma prediction. However, the selected Luma pixels can cross the virtual boundary to use dirty pixels. The modification consists in padding the last value of the clean region instead of crossing the virtual boundary.

8.7.5.3 Picture reconstruction with luma dependent chroma residual scaling process for chroma samples - For the derivation of the variable varScale the following ordered steps apply: - The variable invAvgLuma is derived as follows: - The array recLuma[ i ] with i=0..( 2 * sizeY − 1 ) and the variable cnt are derived as follows: - The variable cnt is set equal to 0. - With horizontal virtual boundaries vbh1,vbh2 and vbh3, When availL is equal to TRUE, the array recLuma[ i ] with i = 0..sizeY − 1 is set equal to currPic[ xCuCb − 1 ][ Min( yCuCb + i, pic_height_in_luma_samples – 1, vbh1-1, vbh2-1, vbh3-1 ) ] with i = 0..sizeY − 1, and cnt is set equal to sizeY - With vertical virtual boundaries vbv1,vbv2 and vbv3, When availT is equal to TRUE, the array recLuma[ cnt + i ] with i = 0..sizeY – 1 is set equal to currPic[ Min( xCuCb + i, pic_width_in_luma_samples – 1 , vbv1-1, vbv2-1, vbv3-1 ) ][ yCuCb − 1 ] with i = 0..sizeY − 1, and cnt is set equal to ( cnt + sizeY )

## CE2-1.b: locally constrained intra prediction

On top of the CE11-2.1.a, this second test introduces an additional normative change which consists in modifying the virtual boundary PPS signalling with an option to use slice level signalling:

- The existing virtual boundaries implementation is signalled at the PPS level since the boundaries positions are assumed static along the video. However, in GDR case, the boundary moves for each frame. This feature is activated through JVET_VIRTUAL_BOUNDARY_SLICE_LEVEL and consists in adding the following signalization:
PPS level:

| pps_loop_filter_across_virtual_boundaries_disabled_flag | u(1) |
| --- | --- |
| pps_virtual_boundaries_slice_signalling | u(1) |
| if( pps_loop_filter_across_virtual_boundaries_disabled_flag && ! pps_virtual_boundaries_slice_signalling) { |  |
| pps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_ver_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_x[ i ] | u(v) |
| pps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_hor_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_y[ i ] | u(v) |
| } |  |

Slice level:

| if( pps_loop_filter_across_virtual_boundaries_disabled_flag && pps_virtual_boundaries_slice_signalling) { |  |
| --- | --- |
| slice_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < slice_num_ver_virtual_boundaries; i++ ) |  |
| slice_virtual_boundaries_pos_x[ i ] | u(v) |
| slice_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < slice_num_hor_virtual_boundaries; i++ ) |  |
| slice_virtual_boundaries_pos_y[ i ] | u(v) |
| } |  |

## CE2-1.c:

One of the main drawbacks of CE2-1.a and CE2-1.b is the intra mode restriction. Indeed, a lot of intra modes need to be discarded since they use dirty pixels, especially with PDPC which uses the opposite pixels to the intra mode direction.

The test CE2-1.c consists in removing this intra mode restriction by using the virtual boundary to restrict the reference samples. Indeed, during the filling process of reference samples, pixels across the virtual boundaries are discarded as illustrated in the following figure. As a result, the intra prediction is avoided in both direction of the virtual boundary.

This proposed modification leads to three cases:

- The virtual boundary crosses the CB: all reference pixels are marked as non-available for intra prediction.

- The virtual boundary is at the left boundary of the CB: all the pixels of the left column are marked as non-available for intra prediction.

- The virtual boundary crosses the top-right reference pixels of the CB: all the pixels on the right of the boundary are marked as non-available.

This functionality is activated through the macro JVET_IR_ENC_ONLY_VIR_BOUND_INTRA and consists in the following draft change:

- If one or more of the following conditions are true, the sample p[x][y] is marked as “not avilable for intra prediction”: - If availableN is equal to FALSE, the sample refUnfilt[ x ][ y ] is marked as "not available for intra prediction". - For all vertical virtual boundaries V_Bound, if x < V_Bound and x + cu_width >= V_Bound or if x + cu_width < V_Bound and xNBY >= V_Bound or if x >= V_Bound and xNBY < V_Bound - For all horizontal virtual boundaries H_Bound, if y < H_Bound and y + cu_height >= H_Bound or if y + cu_height < H_Bound and yNBY >= H_Bound or if y >= H_Bound and yNBY < H_Bound

# Experimental Results

The simulations are compared to the reference described in [5], using common test conditions (CTC) and low-delay B Main10 configuration as a basis. The following changes were agreed on by all proponents:

- The GOP size is 1 and each picture refers up to 4 immediate preceding pictures in decoding order.

- An IDR picture is inserted every N pictures. N being the frame rate of the tested sequence.

- IntraQPOffset and QPoffset are set equal to 0.

- SEIDecodedPictureHash is set to 1.

- PSNR and bitrates computed without the first I picture + the first cycle (meaning that the first N pictures are not considered in BD-Rate computations), this is activated through the macro JVET_CE11_REMOVE_RD_FOR_FIRST_CYCLE

- The first cycle should be encoded as the first cycle of the reference. Then, the second cycle begins with an intra frame for the reference and with GRA picture for tests. This feature is activated through the macro JVET_FIRST_CYCLE_ISNOT_IR.

The additional parameters for all CE2-1 tests added to the configuration file are as follows:

IntraRefreshType : 1		# Intra Refresh type flag 0: disable, 1: activated

LoopFilterAcrossVirtualBoundariesDisabledFlag :1   # Disables in-loop filtering operations across the virtual boundaries.

The additional parameters for CE2-1b and CE2-1c tests added to the configuration file are as follows:

VirtualBoundariesSliceSignalling :1	  # Modifies the PPS signalling with a slice signaling

The macro JVET_CE11_CHECK_DEC_RA _FOR_IR is present for testing purpose. It consists in providing enabling a new decoding parameter “RandomAccessPos”. For the frame with POC equal to this value, the decoding process discards the buffer of reference frames. After a full refresh cycle, the displayed picture should be totally reconstructed.

## Simulation result of CE2-1.a

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

## Simulation result of CE2-1.b

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B |  |  |  |  |  |
| Class C | 5,62% | 6,74% | 7,33% | 102% |  |
| Class E | 13,90% | 16,21% | 18,72% | 108% |  |
| Overall |  |  |  |  |  |
| Class D | 9,84% | 12,67% | 13,65% | 104% |  |
| Class F |  |  |  |  |  |

## Simulation result of CE2-1.c

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B |  |  |  |  |  |
| Class C | 5,21% | 6,43% | 6,92% | 97% |  |
| Class E | 12,50% | 15,37% | 17,50% | 109% |  |
| Overall |  |  |  |  |  |
| Class D | 9,25% | 12,07% | 12,58% | 103% |  |
| Class F | 32,58% | 39,74% | 41,16% | 125% |  |

# Conclusion

The current contribution document presents and reports the result of the CE tests CE2-1.a, CE2-1.b and CE2-1.c. Three methods are proposed to support GRA using different levels of normative changes. It is recommended to integrate the suggested modifications within the next VTM software version and to consider the proposed solutions for integration in the standard.

# Acknowledgement

The research leading to this work has been supported by the FUI23 EFIGI project, BPI France and Region Ile de France.

# References

[1] K. Kazui, J.M. Thiesse, Hendry, K. Kawamura, “Description of Core Experiment 11 (CE11): Gradual random access”, JVET-N1031, March 2019.

[2] D. Gommelet, J.-M. Thiesse, D.Nicholson, “CE11: Gradual Random Access (GRA) using encoder and normative restrictions (Tests CE11-2.1.a, CE11-2.1.b and CE11-2.2)”, JVET-O0100, July 2019.

[3] D. Gommelet, J.-M. Thiesse, D.Nicholson, “Non-CE11: Non-CIP normative Gradual Random Access”, JVET-O1048, July 2019.

[4] S.Y. Lin, L. Liu, J.L. Lin, Y.C. Chang, C.C. Ju, P. Hanhart, Y. He, “AHG12: Loop filter disabled across virtual boundaries”, JVET-N0438, March 2019.

[5] K. Kazui, J.M. Thiesse, Hendry, L. Wang, K. Kawamura, “Description of Core Experiment 2 (CE2): Gradual decoding refresh”, JVET-02022, July 2019.

# Patent rights declaration(s)

VITEC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).