| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0196 |
| --- | --- |

| Title: | CE6-related: Latency reduction for LFNST signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema | Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

In VTM-6, LFNST is signalled at the end of the CU and signalling depends on transform block level data from all components present in a CU. This is reportedly causing latency in both encoder and decoder as, for example, knowing if LFNST is available for the luma component depends on chroma transform coefficients.

It is proposed to remove this “backwards” dependency by signalling LFNST indexes in the TU level. In the proposed approach, LFNST indexes for luma and LFNST indexes for TUs in single-tree coding mode are signalled in the luma TB. In the case of separate chroma tree mode, the chroma LFNST index is signalled in the first coded chroma TB in a TU. LFNST indexes are signalled after the last coefficient position syntax, which allows the last coefficient position based LFNST availability rule of VTM-6 to be applied using the last coefficient position of the current TB. In addition, signalling of the MTS index is proposed to be moved after LFNST index to handle all transform type signalling in a single place.

The proposed change is reported to have BD-rate impacts of -0.11 % and -0.xx % in AI and RA configurations, respectively. Encoder and decoder runtimes are reportedly not affected with respect to those of VTM-6.

# Proposed changes

It is proposed to remove the LFNST signalling related dependency between chroma and luma transform blocks and do both LFNST and MTS signalling in the same place within a transform block.

The last active coefficient based LFNST availability check of VTM-6 is kept, but only the last position of the current transform block is included in the condition to break the dependency of luma transform mode from chroma coefficients.

The VTM-6 granularity for signalling LFNST is kept unchanged. Thus, in single tree coding LFNST is signalled for luma and the selected LFNST mode is applied also to chroma. In the case of separate chroma tree, luma LFNST is signalled in luma and chroma LFNST is signalled for the first coded chroma TB in a TU.

As the granularity for LFNST signalling is not modified compared to VTM-6, also the VTM-6 encoder search is used without any modifications.

# Objective results

Tables below summarize the objective results using JVET common test conditions. Full results will be provided in a revision of this contribution.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14 % | -0.10 % | 0.10 % | 100 % | 99 % |
| Class A2 | -0.08 % | -0.05 % | 0.05 % | 100 % | 99 % |
| Class B | -0.10 % | -0.11 % | -0.05 % | 100 % | 99 % |
| Class C | -0.09 % | -0.18 % | 0.05 % | 100 % | 101 % |
| Class E | -0.13 % | -0.35 % | -0.19 % | 100 % | 100 % |
| Overall | -0.11 % | -0.15 % | -0.01 % | 100 % | 100 % |
| Class D | -0.10 % | 0.16 % | 0.00 % | 100 % | 101 % |
| Class F | -0.06 % | -0.09 % | -0.12 % | 99 % | 100 % |

# Specification text

coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) {

…

LfnstDcOnly = 1

LfnstZeroOutSigCoeffFlag = 1

…

if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && ( !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && tu_mts_idx[ x0 ][ y0 ] = = 0 && Max( cbWidth, cbHeight ) <= MaxTbSizeY) {

if( LfnstDcOnly = = 0 && LfnstZeroOutSigCoeffFlag = = 1 )

lfnst_idx[ x0 ][ y0 ]

…

transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) {

…

if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) ) {

if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ] && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize )

transform_skip_flag[ x0 ][ y0 ]

if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) && ( !transform_skip_flag[ x0 ][ y0 ] ) )

tu_mts_idx[ x0 ][ y0 ]

}

…

residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) {

…

if( lastSubBlock = = 0 && log2TbWidth >= 2 && log2TbHeight >= 2 && !transform_skip_flag[ x0 ][ y0 ] && lastScanPos > 0 )

LfnstDcOnly = 0

if( ( lastSubBlock > 0 && log2TbWidth >= 2 && log2TbHeight >= 2 ) ¦ ¦ ( lastScanPos > 7 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && log2TbWidth = = log2TbHeight ) )

LfnstZeroOutSigCoeffFlag = 0

…

if( Min( cbWidth, cbHeight ) >= 2 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && ( !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( cbWidth, cbHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY && ( cIdx = = 0 ¦ ¦ (treeType = = DUAL_TREE_CHROMA && (cIdx == 1 ¦ ¦ tu_cbf_cb[ x0 ][ y0 ] == 0) ) ) {

if( lastScanPos > 0 && lastScanPos < 16 && !( lastScanPos > 7 && (log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && log2TbWidth = = log2TbHeight ) )

lfnst_idx[ x0 ][ y0 ]

}

if( cIdx = = 0 && lfnst_idx[ x0 ][ y0 ] = = 0 && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( LastSignificantCoeffX < 16 ) && ( LastSignificantCoeffY < 16 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) ) {

if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) )

tu_mts_idx[ x0 ][ y0 ]

}

…

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).