| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0197 |
| --- | --- |

| Title: | CE6-related: LFNST transform mapping |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Nokia Technologies Tampere, Finland | Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document proposes to change the mapping of LFNST indexes to transform matrices. For LFNST index 1, the proposed approach selects transform index 1 for even and 2 for odd numbered intra prediction modes. For LFNST index 2 the selection is inverted and thus transform index 1 is selected for odd and 2 for even intra prediction modes. In addition, context coding for the second bin of the lfnstIdx syntax element is re-introduced.

It is asserted that for CTC conditions the coding efficiency is practically unaffected, but a low complexity encoder can operate at 90 % encoder runtime with +0.14 % bitrate impact under AI conditions and at 97 % encoder runtime with +0.11 % bitrate impact under RA conditions. This is reported to be a significantly lower BD-rate penalty than what was achieved with the comparable encoder approaches in CE6-1, where the best performing encoder side approach had +0.46 % impact on BD-rate for AI and +0.27 % for RA.

# Introduction

LFNST provides interesting gains but with relatively significant encoder side complexity. CE6-1 addressed that concern by showing that significant encoder runtime reductions can be made by calculating the LFNST matrix index based on intra prediction mode instead of signalling that in the bitstream. A potential downside of the normative CE6-1 approaches is relating to the maximum coding efficiency when encoder runtime is not an issue. This proposal is addressing that by applying the CE6-1.1d approach when lfnstIdx is equal to 1 and using the other available LFNST transform matrix when lfnstIdx is equal to 2. The tables below illustrate the selection in VTM-6 and in proposed approach.

Table 1: LFNST transform mode as function of intra prediction mode in VTM-6

| intraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lfnstIdx == 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | … |
| lfnstIdx == 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | … |

Table 2: LFNST transform mode as function of intra prediction mode as proposed

| intraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lfnstIdx == 1 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | … |
| lfnstIdx == 2 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | … |

# Coding efficiency results

## Encoder checking both lfnstIdx 1 and 2

Results below summarize the impact of the proposed approach when encoder is checking both lfnstIdx 1 and 2 (using the VTM-6.0 encoder search). Full results are available in the attached JVET-P0197.xls spreadsheet.

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01 % | 0.00 % | 0.01 % | 99 % | 99 % |
| Class A2 | 0.00 % | 0.04 % | 0.01 % | 99 % | 100 % |
| Class B | 0.00 % | 0.01 % | 0.03 % | 99 % | 101 % |
| Class C | -0.01 % | -0.09 % | 0.11 % | 100 % | 100 % |
| Class E | 0.00 % | -0.01 % | -0.08 % | 100 % | 102 % |
| Overall | 0.00 % | -0.01 % | 0.02 % | 99 % | 100 % |
| Class D | 0.01 % | -0.02 % | -0.13 % | 100 % | 100 % |
| Class F | 0.00 % | 0.00 % | 0.05 % | 99 % | 98 % |
|  |  |  | RA |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00 % | -0.09 % | -0.01 % | 99 % | 100 % |
| Class A2 | 0.00 % | 0.11 % | 0.07 % | 99 % | 100 % |
| Class B | -0.01 % | 0.15 % | 0.03 % | 100 % | 102 % |
| Class C | -0.01 % | 0.00 % | -0.04 % | 100 % | 99 % |
| Class E |  |  |  |  |  |
| Overall | 0.00 % | 0.06 % | 0.01 % | 99 % | 100 % |
| Class D | -0.06 % | -0.24 % | 0.12 % | 100 % | 96 % |
| Class F | -0.04 % | 0.01 % | 0.16 % | 100 % | 99 % |

## Encoder checking only lfnstIdx 1

Results below summarize the impact of the proposed approach when encoder is checking only lfnstIdx 1 (using the VTM-6.0 encoder search with setting the maximum lfnstIdx to 1). Full results are available in the attached JVET-P0197-enc_limitation.xls spreadsheet.

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.22 % | 0.83 % | 1.06 % | 89 % | 98 % |
| Class A2 | 0.05 % | 0.64 % | 0.76 % | 89 % | 99 % |
| Class B | 0.10 % | 0.75 % | 0.83 % | 90 % | 100 % |
| Class C | 0.16 % | 0.40 % | 0.64 % | 90 % | 102 % |
| Class E | 0.18 % | 0.70 % | 0.60 % | 91 % | 103 % |
| Overall | 0.14 % | 0.66 % | 0.78 % | 90 % | 100 % |
| Class D | 0.10 % | 0.52 % | 0.65 % | 91 % | 103 % |
| Class F | 0.17 % | 0.38 % | 0.27 % | 94 % | 101 % |
|  |  |  | RA |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.19 % | 0.36 % | 0.58 % | 96 % | 99 % |
| Class A2 | 0.08 % | 0.40 % | 0.46 % | 97 % | 99 % |
| Class B | 0.09 % | 0.56 % | 0.59 % | 97 % | 99 % |
| Class C | 0.11 % | 0.24 % | 0.30 % | 98 % | 98 % |
| Class E |  |  |  |  |  |
| Overall | 0.11 % | 0.41 % | 0.48 % | 97 % | 99 % |
| Class D | 0.05 % | 0.34 % | 0.28 % | 97 % | 96 % |
| Class F | 0.07 % | 0.22 % | 0.32 % | 98 % | 99 % |

# Specification text

The specification text change includes the following update to 8.7.4.3. together with changing later references to lfnstIdx in that section to the defined lfnstTrIdx variable:

8.7.4.3	Low frequency non-separable transformation matrix derivation process

Inputs to this process are:

- a variable nTrS specifying the transform output length,

- a variable predModeIntra specifying the intra prediction mode for LFNST set selection,

- a variable lfnstIdx specifying the LFNST index for transform selection in the selected LFNST set.

Output of this process is the transformation matrix lowFreqTransMatrix.

The variable lfnstTrIdx specifying the transform index for transform selection in the selected LFNST set is specified as follows:

- If lfnstIdx equals to 1 following applies:

lfnstTrIdx = ( predModeIntra % 2 ) + 1

- Otherwise (lfnstIdx equals to 2), the following applies:

lfnstTrIdx = 2 - ( predModeIntra % 2 )

…

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).