| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0198-v1 |
| --- | --- |

| Title: | Non-CE3: Enable MIP prediction for 64×N or N×64 blocks at maximum transform size 32 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Z. Zhang, K. Andersson, D. Saffar, R. Sjöberg, J. Ström, R. Yu | Tel: Email: | +46761499428 zhi.a.zhang@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In the current VVC, the maximum transform size is configurable to be equal to 32 or 64. The MIP prediction is able to be used by a coding block when a) the block width ∕ height ratio and height ∕ width ratio is equal or less than 4; b) the block width is equal or less than the maximum transform size and the block height is equal or less than the maximum transform size. This means that when the maximum transform size is configured to be equal to 32, a coding block which has a dimension 64×64, 64×32, 64×16, 32×64 or 16×64 is not be able to be coded as a MIP predicted block.

This contribution proposes to enable the MIP prediction for 64×N and N×64 coding blocks when the maximum transform size is configured to be equal to 32, where N ranges from 16 to 64.

The simulation (non-CTC) uses VTM6.0 as the anchor software and the maximum transform size is configured to be equal to 32 (Log2MaxTbSize=5).

The proposed method has a luma BDR impact for AI/RA of -0.02%/-0.03% compared to VTM6.0 with Log2MaxTbSize=5 and decoding time impact for AI/RA of 100%/100% compared to VTM6.0 with Log2MaxTbSize=5.

# Introduction

In the current version of VVC, given a W × H coding block, the MIP prediction can be used by the block when the following criteria are fulfilled:

- W ∕ H ≤ 4 and H ∕ W ≤ 4,

- W ≤ MaxTbSizeY and H ≤ MaxTbSizeY,

where MaxTbSizeY specifies the maximum transform size.

The configurable maximum transform size was adopted in JVET-O Gothenburg meeting. The maximum transform size can be configured to be equal to 32 or 64 in the current version of VVC [2].

When the maximum transform size is configured to be equal to 32, a block which has a dimension 64×64, 64×32, 64×16, 32×64 or 16×64 can not be coded as MIP predicted block. Table 1 shows the MIP predictable blocks when the maximum transform size is configured to be equal to 32 and 64 respectively.

| Maximum transform size = 32 |  |  |  |  |  | Maximum transform size = 64 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (4×4) | (4×8) | (4×16) | (4×32) | (4×64) |  | (4×4) | (4×8) | (4×16) | (4×32) | (4×64) |
| (8×4) | (8×8) | (8×16) | (8×32) | (8×64) |  | (8×4) | (8×8) | (8×16) | (8×32) | (8×64) |
| (16×4) | (16×8) | (16×16) | (16×32) | (16×64) |  | (16×4) | (16×8) | (16×16) | (16×32) | (16×64) |
| (32×4) | (32×8) | (32×16) | (32×32) | (32×64) |  | (32×4) | (32×8) | (32×16) | (32×32) | (32×64) |
| (64×4) | (64×8) | (64×16) | (64×32) | (64×64) |  | (64×4) | (64×8) | (64×16) | (64×32) | (64×64) |

Table 1. Block restriction of MIP at maximum transform size is equal to 32 or 64

When the maximum transform size is configured to be equal to 32, a coding block which has a width or height is greater than 32 is able to be coded as a conventional intra predicted block. The coding block is implicitly split into 2 or 4 sub-blocks where the width and height of the sub-blocks are equal or less than the maximum transform size. The intra prediction information in the coding block is applied to each sub-block.

# Proposed method

This contribution proposes to enable the MIP prediction for 64×N and N×64 coding blocks when the maximum transform size is configured to be equal to 32, where N ranges from 16 to 64.

A MIP predicted coding block which has a width or height is greater than 32 is implicitly split into 2 or 4 sub-blocks where the width and height of the sub-blocks are equal or less than the maximum transform size. The MIP prediction information in the coding block is applied to each sub-block. Figure 1 shows the implicit split for a coding block which has a width or height is greater than 32:

Fig 1. An example of implicit split a coding block into 2 or 4 sub-blocks

We use number of multiplications per sample to perform the complexity analysis. Table 2 shows number of multiplications per sample for a W × H coding block:

|  | max(W,H) = 4 | 4 < max(W,H) ≤ 8 | 8 < max(W,H) ≤ 32 | 64×16 16×64 | 64×32 32×64 | 64×64 |
| --- | --- | --- | --- | --- | --- | --- |
| MipSizeId | 0 | 1 | 2 | 2 | 2 | 2 |
| Number of multiplications | 64 | 128 | 448 | 896 | 896 | 1792 |
| (worst case) Multiplications per sample | 4 | 4 | 3.5 | 0.875 | 0.4375 | 0.4375 |

Table 2. Complexity analysis based on the number of multiplications per sample

From table 2, we can detect that the number of multiplications per sample, when the coding block with a width or height is greater than 32, is much smaller than the worst case, for example a 4 × 4 coding block.

The simulation results in chapter 3 show an improvement of coding efficiency by the proposed method. The improvement is higher for random access configuration than all intra configuration, because for all intra configuration the proposed method only enables 64 × 64 MIP predicted blocks, whereas for random access configuration the proposed method enables 64 × 64, 64 × 32, 32 × 64, 64 × 16 and 16 × 64 MIP predicted blocks. For random access configuration, the coding efficiency of ClassA1 is improved by -0.1%.

## Proposed solution 1 — restrict MIP block size by a fix threshold

In the first solution, we propose to restrict MIP block size to be equal or less than 64 × 64 and width ∕ height ratio to be equal or less than 4: given a W × H coding block, the MIP prediction is able to be used by the block when the following criteria are fulfilled:

- W ∕ H ≤ 4 and H ∕ W ≤ 4,

- W ≤ 64 and H ≤ 64.

Proposed VVC text changes:

7.2.8.5 Coding unit syntax

| ……. |  |
| --- | --- |
| if( intra_bdpcm_flag ) |  |
| intra_bdpcm_dir_flag | ae(v) |
| else { |  |
| if( sps_mip_enabled_flag && ( Abs( Log2( cbWidth ) − Log2( cbHeight ) ) <= 2 ) && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) cbWidth <= 64 && cbHeight <= 64 ) |  |
| intra_mip_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_mip_flag[ x0 ][ y0 ] ) |  |
| intra_mip_mode[ x0 ][ y0 ] | ae(v) |
| …… |  |

## Proposed solution 2 – remove MIP block size restriction

In the second solution, we propose to remove MIP block size restriction by only checking the width ∕ height ratio to be equal or less than 4: given a W × H coding block, the MIP prediction can be used by the block when the following criteria are fulfilled:

- W ∕ H ≤ 4 and H ∕ W ≤ 4.

Proposed VVC text changes:

7.2.8.5 Coding unit syntax

| ……. |  |
| --- | --- |
| if( intra_bdpcm_flag ) |  |
| intra_bdpcm_dir_flag | ae(v) |
| else { |  |
| if( sps_mip_enabled_flag && ( Abs( Log2( cbWidth ) − Log2( cbHeight ) ) <= 2 ) ) && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_mip_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_mip_flag[ x0 ][ y0 ] ) |  |
| intra_mip_mode[ x0 ][ y0 ] | ae(v) |
| …… |  |

# Simulation results

The simulation uses VTM-6.0 as the reference software. Both the anchor and test have a configuration setting for maximum transform size equal to 32 (Log2MaxTbSize = 5).

THE DECODING TIME TO BE UPDATE

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 (maximum transform size 32) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.03% | 0.00% | 101% | 98% |
| Class A2 | 0.00% | -0.08% | -0.02% | 101% | 102% |
| Class B | 0.00% | 0.02% | 0.04% | 101% | 101% |
| Class C | 0.00% | 0.01% | 0.00% | 101% | 99% |
| Class E | -0.05% | -0.02% | -0.16% | 99% | 100% |
| Overall | -0.02% | 0.00% | -0.02% | 100% | 100% |
| Class D | 0.00% | 0.00% | -0.01% | 100% | 101% |
| Class F | 0.02% | -0.02% | -0.06% | 101% | 98% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 (maximum transform size 32) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.20% | -0.33% | 100% | 99% |
| Class A2 | 0.00% | -0.10% | -0.03% | 100% | 101% |
| Class B | -0.01% | -0.12% | 0.00% | 100% | 100% |
| Class C | -0.02% | -0.06% | -0.01% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.11% | -0.08% | 100% | 100% |
| Class D | 0.01% | -0.07% | 0.01% | 100% | 99% |
| Class F | 0.02% | 0.13% | -0.06% | 100% | 101% |

# Conclusion

The proposed method enables MIP prediction for a block which has a width or height is greater than maximum transform size. The proposed method has a coding efficiency improvement without increasing the coding complexity under a non-CTC configuration (maximum transform size is equal to 32).

We propose to adopt enable MIP prediction for 64×N or N×64 blocks at maximum transform size 32 to VVC and VTM.

# References

[1]. Versatile Video Coding (Draft 6)

[2]. JVET-O0545: Configurable maximum transform size in VVC

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).