| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0208 |
| --- | --- |

| Title: | Non CE3: Intra mode coding of non-MPM modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Saverio Blasi, Andre Seixas Dias, Gosala Kulupana 201 Wood Lane The Lighthouse W12 7TQ, London, UK | Email: | saverio.blasi@bbc.co.uk andre.seixasdias@bbc.co.uk gosala.kulupana@bbc.co.uk |
| Source: | British Broadcasting Corporation |  |  |

_____________________________

# Abstract

In the latest VVC draft specification, the process of decoding a mode that is not in the MPM list depends on the computation of the MPM list, and sorting of the MPM modes. This contribution proposes to remove this dependency, allowing the process of decoding modes that are not in the MPM list to be performed with no need to evaluate the MPM list and no additional sorting or comparison operations. An average luma BD-rate of 0.05% is reported for the AI configuration, and an average luma BD-rate of 0.03% is reported for the RA configuration, with no impact on the encoding and decoding time.

# Proposed method

The process of decoding a mode that is not in the MPM list in the VVC WD [1] depends on the computation of the MPM list and sorting of the modes in the list. In JVET-O0515 [2] it was proposed to remove this dependency altogether, and encode non-MPM modes independently from the computation of the MPM list. In the latest VVC draft the MPM list was modified so that only the Planar mode is guaranteed to be present in the MPM list, whereas any other angular mode and the DC mode are not guaranteed to be present in the list. In this contribution, the method in JVET-O0515 was modified to address this modification. In case the intra mode for a given PU is not an MPM mode, it is proposed to set IntraPredModeY[ xCb ][ yCb ] directly equal to intra_luma_mpm_remainder[ xCb ][ yCb ] + 1. The maximum value of intra_luma_mpm_remainder[ ][ ] is changed from 60 to 65, namely all the 67 available modes apart from the Planar mode.

While this change introduces additional redundancies in the luma intra mode coding, this modification removes completely the need at the decoder for computing the MPM list in case intra_luma_mpm_flag[ x0 ][ y0 ] = = 0, namely if the current mode is not in the list. The step of sorting the modes in the list, and comparing the current intra mode with the elements in the list, at both encoder and decoder side is also removed. This reduces the complexity of the decoder, with very limited impact on the encoder compression efficiency. Moreover, additional flexibility is provided at the encoder side, as a bitstream could be produced which does not require any MPM list computation at all.

The following changes to Clause 8.4.2 are proposed:

- IntraPredModeY[ xCb ][ yCb ] is derived by applying the following procedure:

- If intra_luma_mpm_flag[ xCb ][ yCb ] is equal to 1, the IntraPredModeY[ xCb ][ yCb ] is set equal to  candModeList[ intra_luma_mpm_idx[ xCb ][ yCb ] ].

- Otherwise, IntraPredModeY[ xCb ][ yCb ] is derived by applying the following ordered steps:

- When candModeList[ i ] is greater than candModeList[ j ] for i = 0..3 and for each i, j = ( i + 1 )..4, both values are swapped as follows:

( candModeList[ i ], candModeList[ j ] ) = Swap( candModeList[ i ], candModeList[ j ] )	(8-37)

- IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

  - IntraPredModeY[ xCb ][ yCb ] is set equal to intra_luma_mpm_remainder[ xCb ][ yCb ].

  - The value of IntraPredModeY[ xCb ][ yCb ] is incremented by one.

  - For i equal to 0 to 4, inclusive, when IntraPredModeY[ xCb ][ yCb ] is greater than or equal to candModeList[ i ], the value of IntraPredModeY[ xCb ][ yCb ] is incremented by one.

Table 9-77 is modified as follows:

| intra_luma_mpm_remainder[ ][ ] | TB | cMax = 60 65 |
| --- | --- | --- |

# Complexity analysis

An analysis of class C and class D sequences encoded with the AI configuration under CTC shows that 22.7% of intra-coded PUs are encoded with a non-MPM mode. The proposed method affects decoding of all such PUs.

A software decoder can exploit the modification proposed in this contribution to the VVC draft spec by completely removing the need for computing the MPM list for non-MPM modes. This removes the need to perform the entire MPM construction process for a large percentage of PUs. While a thorough analysis of the complexity to construct the intra MPM list is outside the scope of this proposal, the construction of such list includes parsing the intra-prediction modes from the neighbouring blocks on the top and to the left of the current PU. It also involves a number of comparison operations, to check whether these modes are identical or not, and to check whether they are angular modes or Planar or DC modes.

Hardware decoders will most likely compute the MPM list before parsing of the MPM flag for a given block. On the other hand, any decoder could exploit the modification proposed in this contribution to remove the need for sorting the MPM modes. Moreover, up to five comparison operations are needed in the current VVC draft to go from intra_luma_mpm_remainder to IntraPredModeY. All these steps are removed from the decoder process in the proposed modification to the VVC draft specs.

# Performance results

Tests were performed using the JVET common test conditions [3] with parallel encoding/decoding used for RA. The software modifications were made on top of VTM 6.0 [4], compiled with gcc 4.8.2. All experiments were run using AVX.

Table 1: Performance of the proposed luma intra mode coding with no MPM derivation

|  |  |  | AI |  |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.13% | 0.11% | 101% | 101% | 0.06% | 0.11% | 0.06% | 99% | 99% |
| Class A2 | 0.04% | 0.09% | 0.07% | 101% | 101% | 0.03% | 0.13% | 0.08% | 99% | 100% |
| Class B | 0.05% | 0.04% | -0.02% | 100% | 99% | 0.02% | 0.13% | -0.06% | 100% | 100% |
| Class C | 0.06% | 0.01% | 0.00% | 100% | 101% | 0.02% | 0.14% | 0.07% | 100% | 101% |
| Class E | 0.05% | 0.09% | 0.10% | 99% | 101% |  |  |  |  |  |
| Overall | 0.05% | 0.06% | 0.04% | 100% | 100% | 0.03% | 0.13% | 0.03% | 100% | 100% |
| Class D | 0.06% | 0.02% | -0.10% | 100% | 102% | 0.01% | -0.16% | -0.06% | 100% | 101% |
| Class F | 0.05% | -0.04% | -0.04% | 100% | 99% | 0.00% | -0.06% | 0.18% | 100% | 100% |

# Conclusions

This contribution proposes to decouple the derivation of luma intra modes that are not in the MPM list from the computation of the MPM list. The step of sorting the modes in the list at both encoder and decoder side is also removed. This reduces the complexity of the decoder and introduces more flexibility at the encoder side. An average luma BD-rate of 0.05% is reported for the AI configuration, and an average luma BD-rate of 0.03% is reported for the RA configuration, with no impact on the encoding and decoding time.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001-vE, Gothenburg, July 2019.

- S. Blasi, A. Seixas Dias, G. Kulupana, “Non CE3: Intra mode coding of non-MPM modes”, JVET-O0515, Gothenburg, July 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, March 2019.

- VTM-6.0 software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git, February 2019

# Patent rights declaration(s)

The British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).