| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0214 |
| --- | --- |

| Title: | CE4-related: Simplification of the CIIP Chroma Intra Prediction Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jeeyoon Park (SKKU) Byeungwoo Jeon (SKKU) 2066 Seoburo, Jangangu Suwon, Korea | Tel: Email: | +82-31-290-7186 jiyoonpark@skku.edu bjeon@skku.edu |
| Source: | Sungkyunkwan University (SKKU) |  |  |

_____________________________

- Abstract

This contribution proposes to change the intra prediction mode of a chroma block from DM mode to Planar mode in CIIP prediction for simplification purpose. By setting the chroma mode as planar mode in CIIP, there is no need to refer to the mode of the luma block and it simplifies the encoding and decoding process. There is no performance change.

Introduction

In the VVC working draft 6 [1], the combined inter and intra prediction (CIIP) combines the intra prediction and the inter prediction of an inter merge coding unit (CU). The combined prediction Pcombi,j is given by,

Pcombi,j=(WInter∙PInteri,j+WIntra∙PIntrai,j)≫3

In case of the luminance block, the intra prediction mode used in CIIP is set to the Planar mode, and in case of the chroma block, it is set to the DM mode [3].

CIIP mode for Chroma Block

In case of CIIP, this contribution proposes to change the intra prediction mode of chroma from DM mode to Planar mode for simplification purpose.

| VTM 6.0 | Proposed |
| --- | --- |
| Luma: Planar mode Chrom: DM mode | Luma: Planar mode Chroma: Planar mode |

It is noted that by setting the chroma mode as planar as proposed, there is no need to refer to the mode of the Luma block (Simplification). Evidently, there is no performance change.

Conclusion

It is proposed to change intra prediction mode of chroma from DM mode to Planar mode in CIIP prediction for simplification purpose. It is suggested to adopt the proposed simplification into next VTM7.0.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, 15th Meeting, Gothenburg, SE, 3–12 July 2019.

- M. Chiang, C. Hsu, “CE10.1.1: Multi-hypothesis prediction for improving AMVP mode, skip or merge mode, and intra mode,” JVET-L0100, 12th Meeting, Macao, CN, 3–12 Oct 2018.

- L.Van, “CE10: CIIP with position-independent weights (Test CE10-1.1),” JVET-N0302, 14th Meeting, Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

Sungkyunkwan University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).