| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0217 |
| --- | --- |

| Title: | AHG17: On Profile, tier, and level syntax structure |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Sachin Deshpande Andrew Segall | Email: | samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution includes three proposals for modifications related to the profile, tier, and level syntax structure. The three proposals are:

- Move general_constraint_info( ) and general_level_idc above the syntax elements related to sub-profiles.

- Modify end criterion on loop over sub-profiles to allow 0 sub-profiles to be signalled.

- Add inference rule for sub_layer_level_idc[ i ] when it is not present.

# Introduction

This contribution focuses on the Profile, tier, and level syntax and includes 3 proposals as described below.

## Proposal 1

In order to make general_constraint_info( ) and general_level_idc easily accessible it is proposed to move them before the syntax elements that relates to sub-profiles. All syntax elements that precede the profile_tier_level syntax strucutre in the SPS and DPS are fixed length coded with no conditionally present elements. The first elements in the profile_tier_level syntax structure are also fixed length coded with no conditionally present elements. In order to make general_constraint_info( ) and general_level_idc accessible without parsing any conditionally present syntax elements it is proposed to move them before the syntax elements related to sub-profiles as shown in Table 1.

| profile_tier_level( maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| general_profile_idc | u(7) |
| general_tier_flag | u(1) |
| general_constraint_info( ) |  |
| general_level_idc | u(8) |
| num_sub_profiles | u(8) |
| for( i = 0; i <= num_sub_profiles; i++ ) |  |
| general_sub_profile_idc[ i ] | u(32) |
| general_constraint_info( ) |  |
| general_level_idc | u(8) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| sub_layer_level_present_flag[ i ] | u(1) |
| while( !byte_aligned( ) ) |  |
| ptl_alignment_zero_bit | f(1) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |

Table 1

## Proposal 2

In order to allow for not signalling any general_sub_profile_idc[ i ], it is proposed to change the for-loop end condition from <= to < as shown in Table 2.

| profile_tier_level( maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| … |  |
| num_sub_profiles | u(8) |
| for( i = 0; i < num_sub_profiles; i++ ) |  |
| general_sub_profile_idc[ i ] | u(32) |
| … |  |
| } |  |

## Proposal 3

The VVC draft does not specify what value to infer for sub_layer_level_idc[ i ], when not present. It is proposed to add the following inference rule to the semantics of sub_layer_level_idc[ i ]

When not present sub_layer_level_idc[ i ] is inferred as follows:

- sub_layer_level_idc[ maxNumSubLayersMinus1 ] is inferred to be equal to general_level_idc of the same profile_tier_level() structure,

- for i in the range of maxNumSubLayersMinus1 − 1 to 0 (in decreasing order of i values), sub_layer_level_idc[ i ] is inferred to be equal to sub_layer_level_idc[ i+1 ].

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).