| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0219 |
| --- | --- |

| Title: | AHG8/AHG17: On signaling reference picture resampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

To use the reference picture resampling (RPR) for various use cases, alternative syntax elements indicating the reference region for RPR are proposed. The proposed syntax elements are used for calculation of scaling factors as well as the left-top location of the referenced region, when using the conformance window for scaling factor calculation is not available.

Problem statement

At the last Gothenburg meeting, it was adopted that the conformance window parameters are signaled in PPS and the scale factor for reference picture resampling (RPR) is calculated based on the output widths and output heights, which are derived from the conformance window parameters, between the current picture and the referenced picture. The intention was to calculate the scaling factor more exactly, compared to using the decoded picture size. It may work well for most video sequences, which the output picture size is almost identical to the decoded picture size, with small padded regions.

In this contribution, discussed are the following issues, related to the scaling factor calculation based on the conformance window.

- When a conformance window size is quite different from the decoded picture size with large offset values, for immersive media applications (e.g. 360 cubemap, stereoscopic, point cloud), the calculation of scaling factor based on the conformance window size may not guarantee the quality of inter-prediction with different resolutions. In extreme cases, the collocated region of the current CU in a reference picture may not be present.

- When the RPR is used for scalability with multi-layers, the conformance window offsets may not be used for calculation of referenced region across layers. Note that the referenced region of each directly dependent layer is explicitly signaled in PPS-extension, in SHVC.

- When a sub-bitstream targeting a specific region (sub-picture) is extracted from the entire bitstream, the conformance window size is not matched with the picture size at all.  Note that the conformance window parameters can not be updated as long as the parameters are used for scaling calculation, once the bitstream is encoded.

Based on the above potential problems, the calculation of the scaling factors based on the conformance window size may have corner cases, for which an alternative method is necessary. As a fallback, when the conformance window parameters may not be used for calculation of scaling factors, it is proposed to signal reference region parameters that can be used to calculate scaling factors and offset values of the left-top position of the reference region, for RPR and Scalability.

Proposal

Proposed text

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| use_conf_win_for_rpr_flag | u(1) |
| if( !use_conf_win_for_rpr_flag ) { |  |
| ref_region_left_offset | ue(v) |
| ref_region_right_offset | ue(v) |
| ref_region_top_offset | ue(v) |
| ref_region_bottom_offset | ue(v) |
| } |  |
| … |  |
| } |  |

use_conf_win_for_rpr_flag equal to 0 specifies that the reference region syntax elements ref_region_left_offset, ref_region_top_offset, ref_region_top_offset and ref_region_bottom_offset follow next in the PPS. use_conf_wid_for_rpr_flag equal to 1 specifies that the reference region syntax elements ref_region_left_offset, ref_region_top_offset, ref_region_top_offset and ref_region_bottom_offset are not present.

ref_region_left_offset specifies the horizontal offset between the top-left luma sample of the reference region in the decoded picture. The value of ref_region_left_offset shall be in the range of −214 to 214 − 1, inclusive. When not present, the value of ref_region_left_offset is inferred to be equal to conf_win_left_offset.

ref_region_top_offset specifies the vertical offset between the top-left luma sample of the reference region in the decoded picture. The value of ref_region_top_offset shall be in the range of −214 to 214 − 1, inclusive. When not present, the value of ref_region_top_offset is inferred to be equal to conf_win_right_offset.

ref_region_right_offset specifies the horizontal offset between the bottom-right luma sample of the reference region in the decoded picture. The value of ref_layer_right_offset shall be in the range of −214 to 214 − 1, inclusive. When not present, the value of ref_region_right_offset is inferred to be equal to conf_win_top_offset.

ref_region_bottom_offset specifies the vertical offset between the bottom-right luma sample of the reference region in the decoded picture. The value of ref_layer_bottom_offset shall be in the range of −214 to 214 − 1, inclusive. When not present, the value of ref_region_bottom_offset[ ref_loc_offset_layer_id[ i ] ] is inferred to be equal to conf_win_bottom_offset.

The variables PicRefWidthL and PicRefHeightL are derived as follows:

PicRefWidthL = pic_width_in_luma_samples − 	
			 SubWidthC * ( ref_region_right_offset + ref_region_left_offset )

PicRefHeightL = pic_height_in_pic_size_units −				SubHeightC * ( ref_region_bottom_offset + ref_region_top_offset )

Fractional sample interpolation process

        - General

…

The variable fRefWidth is set equal to PicOutputWidthL PicRefWidthL of the reference picture in luma samples.

The variable fRefHeight is set equal to PicOutputHeightL PicRefHeightL of the reference picture in luma samples.

The motion vector mvLX is set equal to ( refMvLX − mvOffset ).

- If cIdx is equal to 0, the following applies:

  - The scaling factors and their fixed-point representations are defined as

hori_scale_fp = ( ( fRefWidth  <<  14 ) + ( PicOutputWidthL PicRefWidthL >>  1 ) ) / PicOutputWidthL PicRefWidthL		(8-753)

vert_scale_fp = ( ( fRefHeight  <<  14 ) + ( PicOutputHeightL PicRefHeightL >>  1 ) ) / PicOutputHeightL PicRefHeightL		(8-754)

…

  - The top-left coordinate of the bounding block for reference sample padding ( xSbIntL, ySbIntL ) is set equal to ( xSb + ( mvLX[ 0 ]  >>  4 ), ySb + ( mvLX[ 1 ]  >>  4 ) ).

  - For each luma sample location ( xL = 0..sbWidth − 1 + brdExtSize, yL = 0..sbHeight − 1 + brdExtSize ) inside the prediction luma sample array predSamplesLX, the corresponding prediction luma sample value predSamplesLX[ xL ][ yL ] is derived as follows:

- Let ( refxSbL, refySbL ) and ( refxL, refyL ) be luma locations pointed to by a motion vector ( refMvLX[0], refMvLX[1] ) given in 1/16-sample units. The variables refxSbL, refxL, refySbL, and refyL are derived as follows:

refxSbL = ( ( (xSb+ ref_region_left_offset)  <<  4 ) + refMvLX[ 0 ] ) * hori_scale_fp	(8-755)

refxL = ( ( Sign( refxSb ) * ( ( Abs( refxSb ) + 128 )  >>  8 )
		+ xL * ( ( hori_scale_fp + 8 )  >>  4 ) ) + 32 )  >>  6	(8-756)

refySbL = ( ( (ySb+ ref_region_top_offset)  <<  4 ) + refMvLX[ 1 ] ) * vert_scale_fp	(8-757)

refyL = ( ( Sign( refySb ) * ( ( Abs( refySb ) + 128 )  >>  8 ) + yL *
		( ( vert_scale_fp + 8 )  >>  4 ) ) + 32 )  >>  6	(8-758)

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).