| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0221 |
| --- | --- |

| Title: | AHG8: On signaling interlayer reference picture list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

To improve the inter-layer reference picture list construction, the following semantics improvements and editorial corrections are proposed.

- Modification of ilrp_idc[ listIdx ][ rplsIdx ][ i ] semantics

- Modification of inter_layer_ref_pics_present_flag semantics

- Editorial correction of the derivation process of the variable DirectDependentLayerIdx[ i ][ j ]

Introduction

To improve the inter-layer reference picture list construction, the following semantics improvements and editorial corrections are proposed.

- Modification of ilrp_idc[ listIdx ][ rplsIdx ][ i ] semantics, as the value of ilrp_idc[ listIdx ][ rplsIdx ][ i ] shall be in the range of 0 to the NumDependentLayers[ GeneralLayerIdx[ nuh_layer_id ] ] − 1, inclusive.

- Modification of inter_layer_ref_pics_present_flag semantics, as inter_layer_ref_pics_present_flag shall be equal to 0 with one of the following conditions:

  - nuh_layer_id is equal to 0.

  - sps_video_parameter_set_id is equal to 0.

  - vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is present in the VPS referred to by the SPS.

- Editorial correction of the derivation process of the variable DirectDependentLayerIdx[ i ][ j ]

Proposal-1

In the reference picture list syntax element set ref_pic_list_struct( listIdx, rplsIdx ), ilrp_idc[ listIdx ][ rplsIdx ][ i ] indicates an index of a directly dependent layer. To reduce the signalled bits, ilrp_idc[ listIdx ][ rplsIdx ][ i ] shall indicate the index of the tartget layer among directly dependent layers of the current layer, not among the entire layers with nuh_layer_id less than the current nuh_layer_id. To address this point correctly, the following improvements of the semantics of ilrp_idc[ listIdx ][ rplsIdx ][ i ] and the derivation process of DirectDependentLayerIdx[ i ][ j ] are proposed.

Proposed text of Proposal-1

The variable DirectDependentLayerIdx[ i ][ j ], specifying the j-th direct dependent layer of the i-th layer, and the variable NumDependentLayers[ i ], specifying the number of dependent layers of the i-th layer, are derived as follows:

for( i = 1; i <= vps_max_layers_minus1; i++) )
	if( !vps_independent_layer_flag[ i ] ) {
		for( j = i-1, k = 0; j  >=  0; j− − )		(7-2)
			if( vps_direct_dependency_flag[ i ][ j ] )
				DirectDependentLayerIdx[ i ][ k++ ] = j		NumDependentLayers[ i ] = k

}

ilrp_idc[ listIdx ][ rplsIdx ][ i ] specifies the index, to the list of directly dependent layers, of the ILRP of i-th entry in ref_pic_list_struct( listIdx, rplsIdx ) syntax structure to the list of directly dependent layers. The value of ilrp_idc[ listIdx ][ rplsIdx ][ i ] shall be in the range of 0 to the NumDependentLayers[ GeneralLayerIdx[ nuh_layer_id ] ] − 1, inclusive.

Proposal-2

A flag inter_layer_ref_pics_present_flag in SPS specifies whether interlayer reference pictures (ILRPs) may be used for inter prediction of one or more coded pictures in the CVS or not. The ILRP shall not be used with the following conditions:

- The current layer is the base layer with nuh_layer_id equal to 0

- The SPS does not refer to a VPS and no VPS is referred to when decoding each CVS referring to the SPS. (i.e. sps_video_parameter_set_id is equal to 0)

- When a VPS is referred by the SPS, vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1.

To address the issue, the semantics of inter_layer_ref_pics_present_flag is modified as follows:

Proposed text of Proposal-2

inter_layer_ref_pics_present_flag equal to 0 specifies that no ILRP is used for inter prediction of any coded picture in the CVS. inter_layer_ref_pics_flag equal to 1 specifies that ILRPs may be used for inter prediction of one or more coded pictures in the CVS. When sps_video_parameter_set_id is equal to 0, the value of inter_layer_ref_pics_present_flag is inferred to be equal to 0. It is a requirement of bitstream conformance that the value of inter_layer_ref_pics_present_flag shall be equal to 0 with one of the following conditions:

- nuh_layer_id is equal to 0.

- sps_video_parameter_set_id is equal to 0.

- vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is present in the VPS referred to by the SPS.

Proposal-3

Editorial corrections on the derivation process of the variable DirectDependentLayerIdx[ i ][ j ].

Proposed text of Proposal-3

for( i = 1; i <= vps_max_layers_minus1; i++ )
	if( !vps_independent_layer_flag[ i ] )
		for( j = i-1, k = 0; j  >=  0; j− − )		(7-2)
			if( vps_direct_dependency_flag[ i ][ j ] )
				DirectDependentLayerIdx[ i ][ k++ ] = j

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).