| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0223 |
| --- | --- |

| Title: | AHG8/AHG12: Efficient signaling of picture size and partitioning information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

For bit-efficient signaling and pre-notification of the picture size and partitioning information, a hierarchical signaling mechanism of picture size and partitioning information is proposed. In VPS, all candidates of picture size and partitioning information pic_size_partitioning_info( ) are listed, which may be present in any layer of the CVS referring to the VPS. In SPS, indexes referring to candidates in the VPS and additional pic_size_partitioning_info( ) candidates. In PPS, an index indicating pic_size_partitioning_info( ) listed in the SPS, which the PPS refers to, is signaled. If necessary, some parts of pic_size_partitioning_info( ) may be updated in PPS.

Introduction

In the current VVC, a set of syntax elements specifying picture size, sub-picture partitioning information, tile partitioning information, rectangular slice partitioning information and conformance window offsets are repeatedly signaled in parameter sets. For bit-efficient signaling and pre-notification of the picture size and partitioning information present in the CVS, a hierarchical signaling mechanism of picture size and partitioning information is proposed. In VPS, all candidates of picture size and partitioning information pic_size_partitioning_info( ) are listed, which may be present in any layer of the CVS referring to the VPS. In SPS, indexes referring to candidates in the VPS and additional pic_size_partitioning_info( ) candidates, which may be referred to by an index in a PPS referring to the SPS, are listed. In PPS, an index indicating pic_size_partitioning_info( ) listed in the SPS, which the PPS refers to, is signaled. If necessary, some parts of pic_size_partitioning_info( ) can be updated in PPS. The proposed specification text follows in the next section. The detailed syntax elements of conf_win_info(), tile_brick_partitioning_info(), rect_slice_partitioning_info(), subpic_partitioning_info() and subpic_conf_win_partitioning_info() are not described in the proposed text, because those syntax elements are identical to them of the current VVC text, or they will be determined in this meeting.

Proposed Text

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_num_pic_size_partitioning_info_minus1 | ue(v) |
| for( i = 0; i <= vps_num_pic_size_partitioning_info_minus1; i++ ) |  |
| pic_size_partitioning_info( ) |  |
| … |  |
| } |  |

vps_num_pic_size_partitioning_info_minus1 plus 1 specifies the number of pic_size_partitioning_info( ) listed in this VPS.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_num_pic_size_partitioning_info_minus1 | ue(v) |
| for( i = 0; i <= sps_num_pic_size_partitioning_info_minus1; i++ ) |  |
| sps_additional_pic_size_partitioning_info_flag[ i ] | u(1) |
| if( sps_additional_pic_size_partitioning_info_flag[ i ] ) |  |
| pic_size_partitioning_info( ) |  |
| else |  |
| sps_vps_pic_size_partitioning_info_idx[ i ] | u(1) |
| … |  |
| } |  |

sps_num_pic_size_partitioning_info_minus1 plus 1 specifies the number of pic_size_partitioning_info( ) listed in this SPS.

sps_additional_pic_size_partitioning_info_flag[i] equal to 0 specifies that sps_vps_pic_size_partitioning_info_idx[ i ] is present. sps_additional_pic_size_partitioning_info_flag[ i ] equal to 1 specifies that the i-th pic_size_partitioning_info( ) listed in this SPS is present, without referring to pic_size_partitioning_info( ) listed in the VPS.

sps_vps_pic_size_partitioning_info_idx[ i ] specifies that the i-th pic_size_partitioning_info( ) listed in this SPS is equal to the sps_vps_pic_size_partitioning_info_idx[ i ]-th pic_size_partitioning_info( ) listed in the VPS that this SPS refers to.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_sps_pic_size_partitioning_info_idx | u(1) |
| pps_pic_size_partitioning_info_update_flag | u(1) |
| if( pps_pic_size_partitioning_info_update_flag ) |  |
| pic_size_partitioning_update_info( ) |  |
| … |  |
| } |  |

pps_sps_pic_size_partitioning_info_idx specifies that the pic_size_partitioning_info( ) of each coded picture referring to this PPS is equal to the pps_sps_pic_size_partitioning_info_idx-th pic_size_partitioning_info( ) listed in the SPS that this PPS refers to.

| pic_size_partitioning_info( ) { | Descriptor |
| --- | --- |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| conf_win_present_flag | u(1) |
| if( conf_win_present_flag ) |  |
| conf_win_info() |  |
| tile_brick_partitioning_present_flag | u(1) |
| if( tile_brick_partitioning_present_flag ) { |  |
| tile_brick_partitioning_info() |  |
| rect_slice_partitioning_present_flag | u(1) |
| if( rect_slice_partitioning_present_flag ) |  |
| rect_slice_partitioning_info() |  |
| subpic_partitioning_present_flag | u(1) |
| if(subpic_partitioning_present_flag) { |  |
| subpic_partitioning_info() |  |
| subpic_conf_win_present_flag | u(1) |
| if(subpic_conf_win_present_flag) |  |
| subpic_conf_win_partitioning_info() |  |
| } |  |
| } |  |
| } |  |

conf_win_present_flag equal to 1 specifies that conf_win_info() is present in this pic_size_partitioning_info( ). conf_win_present_flag equal to 0 specifies that conf_win_info() is not present in this pic_size_partitioning_info( ).

tile_brick_partitioning_present_flag equal to 1 specifies that 	tile_brick_partitioning_info() is present in this pic_size_partitioning_info( ). tile_brick_partitioning_present_flag equal to 0 specifies that tile_brick_partitioning_info() is not present in this pic_size_partitioning_info( ).

rect_slice_partitioning_present_flag equal to 1 specifies that rect_slice_partitioning_info() is present in this pic_size_partitioning_info( ). rect_slice_partitioning_present_flag equal to 0 specifies that rect_slice_partitioning_info() is not present in this pic_size_partitioning_info( ).

subpic_partitioning_present_flag equal to 1 specifies that subpic_partitioning_info() is present in this pic_size_partitioning_info( ). subpic_partitioning_present_flag equal to 0 specifies that subpic_partitioning_info() is not present in this pic_size_partitioning_info( ).

subpic_conf_win_present_flag equal to 1 specifies that subpic_conf_win_info() is present in this pic_size_partitioning_info( ). subpic_conf_win_present_flag equal to 0 specifies that subpic_conf_win_info() is not present in this pic_size_partitioning_info( ).

| pic_size_partitioning_update_info( ) { | Descriptor |
| --- | --- |
| pic_size_update_flag | u(1) |
| if( pic_size_update_flag ) { |  |
| updated_pic_width_in_luma_samples | u(16) |
| updated_pic_height_in_luma_samples | u(16) |
| } |  |
| conf_win_update_flag | u(1) |
| if( conf_win_update_flag ) |  |
| conf_win_info() |  |
| tile_brick_partitioning_update_flag | u(1) |
| if( tile_brick_partitioning_update_flag ) |  |
| tile_brick_partitioning_info() |  |
| rect_slice_partitioning_update_flag | u(1) |
| if( rect_slice_partitioning_update_flag ) |  |
| rect_slice_partitioning_info() |  |
| subpic_partitioning_update_flag | u(1) |
| if( subpic_partitioning_update_flag ) |  |
| subpic_partitioning_info() |  |
| subpic_conf_win_update_flag | u(1) |
| if(subpic_conf_win_update_flag) |  |
| subpic_conf_win_partitioning_info() |  |
| } |  |

conf_win_update_flag equal to 1 specifies that the current conf_win_info() is updated with the conf_win_info() in this pic_size_partitioning_update_info( ). conf_win_update_flag equal to 0 specifies that the current conf_win_info() is not updated.

tile_brick_partitioning_update_flag equal to 1 specifies that the current tile_brick_partitioning_info() is updated with the tile_brick_partitioning_info() in this pic_size_partitioning_update_info( ). conf_win_update_flag equal to 0 specifies that the current tile_brick_partitioning_info() is not updated.

rect_slice_partitioning_update_flag equal to 1 specifies that the current rect_slice_partitioning_info() is updated with the rect_slice_partitioning_info() in this pic_size_partitioning_update_info( ). rect_slice_partitioning_update_flag equal to 0 specifies that the current rect_slice_partitioning_info() is not updated.

subpic_partitioning_update_flag equal to 1 specifies that the current subpic_partitioning_info() is updated with the subpic_partitioning_info() in this pic_size_partitioning_update_info( ). subpic_partitioning_update_flag equal to 0 specifies that the current subpic_partitioning_info() is not updated.

subpic_conf_win_update_flag equal to 1 specifies that the current subpic_conf_win_info() is updated with the subpic_conf_win_info() in this pic_size_partitioning_update_info( ). subpic_conf_win_update_flag equal to 0 specifies that the current subpic_conf_win_info() is not updated.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).