| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0223-v2 |
| --- | --- |

| Title: | AHG8/AHG12: Efficient signaling of picture size and partitioning information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In V-2, a simplified version is proposed.

- Proposal 1: constant_pic_size_flag is signaled in SPS. When the value of constant_pic_size_flag is equal to 1, pic_width_in_luma_samples and pic_height_in_luma_samples are not present in PPS, and the values of pic_width_in_luma_samples and pic_height_in_luma_samples in a PPS referring to the SPS are inferred to be equal to the values of pic_width_max_in_luma_samples and pic_height_max_in_luma_samples, respectively.

- Proposal 2: constant_conf_win_size_flag is signaled in SPS. When the value of constant_conf_win_size_flag is equal to 1, constant_conf_win_left_offset, constant_conf_win_left_offset, constant_conf_win_top_offset  and constant_conf_win_bottom_offset are present in the SPS, and the values of conf_win_left_offset, conf_win_left_offset, conf_win_top_offset  and conf_win_bottom_offset in a PPS referring to the SPS are inferred to be equal to the values of constant_conf_win_left_offset, constant_conf_win_left_offset, constant_conf_win_top_offset  and constant_conf_win_bottom_offset, respectively.

It is asserted that the proposed method can save bits from 23bits to about 55bits per PPS.

# Introduction

In V-2, a simplified version is proposed. It is asserted that the proposed method can save bits from 23bits to about 55bits per PPS.

# Proposed Text

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| constant_pic_size_flag | u(1) |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| constant_conf_win_size_flag | u(1) |
| if( constant_conf_win_size_flag ) { |  |
| constant_conf_win_left_offset | ue(v) |
| constant_conf_win_right_offset | ue(v) |
| constant_conf_win_top_offset | ue(v) |
| constant_conf_win_bottom_offset | ue(v) |
| } |  |
| … |  |
| } |  |

constant_pic_size_flag equal to 1 species that each coded picture referring to a PPS referring to the SPS has the same picture size, which is specified by pic_width_max_in_luma_samples and pic_height_max_in_luma_samples. constant_pic_size_flag equal to 0 species that each coded picture referring to a PPS referring to the SPS may or may not have the same picture size.

constant_conf_win_size_flag equal to 1 species that each coded picture referring to a PPS referring to the SPS has the same conformance window size, which is specified by constant_conf_win_left_offset, constant_conf_win_right_offset, constant_conf_win_top_offset and constant_conf_win_bottom_offset. constant_conf_win_size_flag equal to 0 species that each coded picture referring to a PPS referring to the SPS may or may not have the same conformance window size.

constant_conf_win_left_offset, constant_conf_win_right_offset, constant_conf_win_top_offset, and constant_conf_win_bottom_offset specify the samples of the pictures in the CVS that are output from the decoding process, in terms of a rectangular region specified in picture coordinates for output, when constant_conf_win_size_flag equal to 1.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pic_size_present_flag | u(1) |
| if( pic_size_present_flag ) { |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| } |  |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| … |  |
| } |  |

pic_size_present_flag equal to 1 specifies that pic_width_in_luma_samples and pic_height_in_luma_samples are present in the PPS. pic_size_present_flag equal to 0 specifies that pic_width_in_luma_samples and pic_height_in_luma_samples are not present in the PPS.  It is a requirement of bitstream conformance that the value of pic_size_present_flag is equal to 0, when the value of constant_pic_size_flag is equal to 1.

pic_width_in_luma_samples specifies the width of each decoded picture referring to the PPS in units of luma samples. pic_width_in_luma_samples shall not be equal to 0, shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_width_max_in_luma_samples. When subpics_present_flag is equal to 1, the value of pic_width_in_luma_samples shall be equal to pic_width_max_in_luma_samples. When not present, the value of pic_width_in_luma_samples shall be equal to pic_width_max_in_luma_samples.

pic_height_in_luma_samples specifies the height of each decoded picture referring to the PPS in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_height_max_in_luma_samples. When subpics_present_flag is equal to 1, the value of pic_height_in_luma_samples shall be equal to pic_height_max_in_luma_samples. When not present, the value of pic_height_in_luma_samples shall be equal to pic_height_max_in_luma_samples.

conformance_window_flag equal to 1 indicates that the conformance cropping window offset parameters follow next in the SPS. conformance_window_flag equal to 0 indicates that the conformance cropping window offset parameters are not present. It is a requirement of bitstream conformance that the value of conformance_window_flag is equal to 0, when the value of constant_conf_win_size_flag is equal to 1.

conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, and conf_win_bottom_offset specify the samples of the pictures referring to the PPS, that are output from the decoding process, in terms of a rectangular region specified in picture coordinates for output. When conformance_window_flag is equal to 0, the values of conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, and conf_win_bottom_offset are inferred to be equal to 0 the values of constant_conf_win_left_offset, constant_conf_win_right_offset, constant_conf_win_top_offset, and constant_conf_win_bottom_offset.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).