| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0234-v1 |
| --- | --- |

| Title: | AHG-2/Non-CE4: On clarification of weightedPredFlag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-70-1617-3470 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

In this contribution, the variable weightedPredFlag which defines the explicit weighted prediction enabling is clarified. The current description of the variable weightedPredFlag is based on HEVC text. However, it is contradicted to the description of other VVC tools such as DMVR, BDOF and BCW. Therefore, in this contribution, a new definition for weightedPredFlag is proposed.

Problem Statement

The VVC coding tools such as DMVR, BDOF, and BCW define their applicable conditions by using luma_weight_lX_flag or chroma_weight_lX_flag with X being 0 and 1. However, the variable weightedPredFlag does not consider these descriptions because it is based on HEVC text. Therefore, the definition for weightedPredFlag using luma_weight_lX_flag and chroma_weight_lX_flag should be updated.

Proposal changes of specification

The working draft [1] should be changed by the followings.

- 8.5.6.6	Weighted sample prediction process

- 8.5.6.6.1	General

The variable weightedPredFlag is derived as follows:

- If slice_type is equal to P, weightedPredFlag is set equal to pps_weighted_pred_flag.

- Otherwise (slice_type is equal to B), weightedPredFlag is set equal to pps_weighted_bipred_flag.

to

- If slice_type is equal to P, weightedPredFlag is set equal to (pps_weighted_pred_flag && ((cIdx == 0) ? luma_weight_l0_flag [refIdxL0] : chroma_weight_l0_flag[refIdxL0]))

- Otherwise (slice_type is equal to B), weightedPredFlag is set equal to (pps_weighted_bipred_flag && ((cIdx == 0) ? (luma_weight_l0_flag[refIdxL0] || luma_weight_l1_flag[refIdxL1]) : (chroma_weight_l0_flag[refIdxL0] || chroma_weight_l1_flag[refIdxL1])).

Conclusion

In this contribution, the variable weightedPredFlag has been clarified. We would like to recommend that the working draft and reference software should support this specification.

Reference

[1]	B. Bross, J. Chen and S. Liu, Versatile Video Coding (Draft 6), Joint Video Experts Team Document, JVET-O2001-vE, Gothenburg, July 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).