| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0238-v2 |
| --- | --- |

| Title: | CE4-related: On definition of motion vector range for PROF and BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Eiich Sasaki Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-70-1617-3470 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

In the current WD6, the variables of motion vector range for PROF and BDOF is defined by the function of bit-depth. However, the range of motion vector is an independent parameter from bit-depth and the output of the function is a constant value when the bit-depth is 8 to 12 bits. Therefore, in this contribution, it is proposed to define those variable to the fixed values.

Problem Statement

In the current WD6, the variables (dmvLimit and mvRefineThres) of motion vector range for PROF and BDOF is defined by the function of bit-depth. However, the range of motion vector is an independent parameter from bit-depth and the output of the function is a constant value when the bit-depth is 8 to 12 bits. Therefore, those variables should be defined to the fixed values.

Proposal changes of specification

The working draft [1] should be changed by the followings.

- 8.5.5.9 Derivation process for motion vector arrays from affine control point motion vectors

dmvLimit  =  641  <<  Max( 6, BitDepthY − 6 )		(8-731)

    - The value of diffMv[ x ][ y ][ i ] is clipped as follows:

diffMv[ x ][ y ][ i ] = Clip3( −dmvLimit, dmvLimit − 1, diffMv[ x ][ y ][ i ]) 	(8-736)

In the case of CE4.2, the variable dmvLimit should be 32.

- 8.5.6.5 Bi-directional optical flow prediction process

- The variable mvRefineThres is set equal to 321 << Max( 5, bitDepth − 7 ).

  - The horizontal and vertical motion offset of the current subblock are derived as:

vx = sGx2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres,	(8-817)
	−( sGxdI << 3 ) >> Floor( Log2( sGx2 ) ) )  :  0

vy = sGy2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres, ( ( sGydI << 3 ) − 	(8-818)
	( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ) >> Floor( Log2( sGy2 ) ) )  :  0

Conclusion

In this contribution, the variable dmvLimit and the variable mvRefineThres have been clarified. We would like to recommend that the working draft and reference software should support this specification.

Reference

[1]	B. Bross, J. Chen and S. Liu, Versatile Video Coding (Draft 6), Joint Video Experts Team Document, JVET-O2001-vE, Gothenburg, July 2019.

[2]	J. Luo and Y. He, CE4: Prediction refinement with optical flow for affine mode (Test 2.1), Joint Video Experts Team Document, JVET-O0070, Gothenburg, July 2019.

[3]	T. Chujoh, E. Sasaki and T. Ikai, Non-CE9: A bug fix of BDOF specification, Joint Video Experts Team Document, JVET-O0207, Gothenburg, July 2019.

[4]	X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu and X. Wang, CE9-related: On BDOF precision alignment for high internal bit-depth, Joint Video Experts Team Document, JVET-O0609, Gothenburg, July 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).