| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0241-v1 |
| --- | --- |

| Title: | AHG17/CE1-related: Specifying Scaling Regions for Reference Picture Resampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tim Hellman, Wade Wan, Peisong Chen | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

The reference picture resampling functionality was adopted into the draft text at the Gothenburg July 2019 meeting.  It was identified during the meeting that a potential issue that required further study is how to define the scaling ratio between pictures, especially for picture sizes that are not a multiple of 8 pixels.  An approach using the conformance cropping window to determine the scaling ratio was agreed to be the starting point but it was acknowledged this may not be the right approach.

This contribution proposes to redefine the semantics of the width and height transmitted in the PPS to represent the true picture dimensions, derive decode dimensions by rounding the true dimensions with the appropriate granularity, and using the true dimensions for scaling ratio computations.  It is suggested that this approach provides a simple method to represent integer scaling ratios and avoids complications with the use of the conformance window for these calculations.

# Introduction

The reference picture resampling (RPR) approach documented in JVET-O1164 [1] was adopted into the draft text [2] as a starting point for the feature.  During discussion of this topic, it was identified that the scaling ratio computation resulted in undesirable values for picture sizes that are not a multiple of Max(8, MinCbSizeY) pixels due to the following (highlighted) constraints in the semantics for the picture width and height:

pic_width_in_luma_samples specifies the width of each decoded picture referring to the PPS in units of luma samples. pic_width_in_luma_samples shall not be equal to 0, shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_width_max_in_luma_samples.

pic_height_in_luma_samples specifies the height of each decoded picture referring to the PPS in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_height_max_in_luma_samples.

For example, consider the use case where there is a reference picture that is half the resolution of the commonly used HD format (1920x1080 pixels).  For simplicity, this is assuming MinCbSizeY == 8 and thus pictures are restricted to be multiples of 8 pixels.  If not, the same issue is present but at a larger resolution.  This restriction would result in a reference picture that is 960x544 because half the height of the HD format (1080) is not evenly divisible by 8 and thus the quarter HD format picture would be coded with a 960x544 picture but intending only for a 960x540 region of the picture to be referenced, displayed, etc. as shown in the figure below.

Note the two problems that result from the image height to be rounded up:

- The decoded picture ends up pointing to the wrong pixels in the reference picture, as shown by the red squares in the figure.  More specifically, at the bottom of the decoded picture, the pixels in the HD picture are pointing to the padding region of the reference quarter HD picture.

- The derived scaling ratio of 1.98 is awkward, as it's derived directly from the padded height and this is not ideal for the pixel processing.  It is claimed that this is problematic for interop as well as general understanding of the specification.

At the meeting it was proposed to use the conformance window for the calculation to solve these problems.  It was also recognized during the discussion at the meeting as well as during the software implementation to support CE1 experiments that use of the conformance window can be problematic.

The conformance cropping window potentially has left, right, top and bottom offset values that need to be accounted for.  It has also been noted that the conformance cropping window was originally intended to be used to assist the display process and not to affect the decode process and this is reflected in this note in the current draft text:

NOTE – The conformance cropping window offset parameters are only applied at the output. All internal decoding processes are applied to the uncropped picture size.

# Proposal

This contribution proposes to redefine the semantics of the width and height that are transmitted in the PPS (note no change to the syntax) to represent the true picture dimensions.  These dimensions represent the exact dimensions intended for display and therefore would not be constrained to be a multiple of Max(8, MinCbSizeY) pixels.  The dimensions needed for the decode process remain the same and need to be constrained to be a multiple of Max(8, MinCbSizeY) pixels.  These would be derived by rounding up both the height and width when necessary.

In the following figure, pic_height represents the true picture height and PicHeight represents the decode picture height.  The example shows that separation of the true and decode dimensions:

- Allows the true picture dimensions to be used for the display process

- Allows the decoded picture dimensions to be used for the decode process

- More specifically, computation of the scaling ratios using true picture dimensions results in more intuitive scaling ratios, in this case, 2:1

# Specification Changes

Detailed specification text changes are provided as an attachment to this contribution.  The major aspects are:

- In the SPS, redefine pic_width_max_in_luma_samples and pic_height_max_in_luma_samples to sps_pic_width_max_in_luma_samples and sps_pic_height_max_in_luma_samples:

sps_pic_width_max_in_luma_samples specifies the original maximum width without padding, in units of luma samples, of each decoded picture referring to the SPS. sps_pic_width_max_in_luma_samples shall not be equal to 0 and sps_pic_width_max_in_luma_samples %SubWidthC shall be an equal to 0.

The variable PicWidthMaxInLumaSamples is derived as follows:

PicWidthMaxInLumaSamples = ((sps_pic_width_max_in_luma_samples + PicLumaMult – 1) / PicLumaMult) * PicLumaMult,

where PicLumaMult is equal to Max(8, MinCbSizeY).

sps_pic_height_max_in_luma_samples specifies the original maximum height without padding, in units of luma samples, of each decoded picture referring to the SPS. sps_pic_height_max_in_luma_samples shall not be equal to 0 and sps_pic_height_max_in_luma_samples% SubHeightC shall be an equal to 0.

The variable PicHeightMaxInLumaSamples is derived as follows:

PicHeightMaxInLumaSamples = ((sps_pic_height_max_in_luma_samples + PicLumaMult – 1) / PicLumaMult) * PicLumaMult

- In the PPS, redefine pic_width_in_luma_samples and pic_height_in_luma_samples to pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples:

pps_pic_width_in_luma_samples specifies the original unpadded width of each decoded picture referring to the PPS in units of luma samples. pps_pic_width_in_luma_samples shall not be equal to 0, and shall be less than or equal to sps_pic_width_max_in_luma_samples, and pps_pic_width_in_luma_samples%SubWidthC shall be equal to 0.

When subpics_present_flag is equal to 1, the value of pps_pic_width_in_luma_samples shall be equal to sps_pic_width_max_in_luma_samples.

The variable PicWidthInLumaSamples is derived as follows:

PicWidthInLumaSamples = ((pps_pic_width_in_luma_samples + PicLumaMult – 1) / PicLumaMult) * PicLumaMult

pps_pic_height_in_luma_samples specifies the original unpadded height of each decoded picture referring to the PPS in units of luma samples. pps_pic_height_in_luma_samples shall not be equal to 0, and shall be less than or equal to sps_pic_height_max_in_luma_samples, and pps_pic_height_in_luma_samples%SubHeightC shall be equal to 0.

When subpics_present_flag is equal to 1, the value of pps_pic_height_in_luma_samples shall be equal to sps_pic_height_max_in_luma_samples.

The variable PicHeightInLumaSamples is derived as follows:

PicHeightInLumaSamples = ((pps_pic_height_in_luma_samples + PicLumaMult – 1) / PicLumaMult) * PicLumaMult

- It should also be noted that the proposed text changes includes two bug fixes discovered when reviewing the text.  More specifically, there is one clause in both Section 8.5.1 and another in Section 8.5.6 in the current draft text using the padded dimensions.  The authors believe these are bugs and use of the true dimensions is the design intention.

# References

[1] P. Chen et al., “AHG 8: Integrated Specification Text for Reference Picture Resampling”, document JVET-O1164, JVET, Gothenburg, Sweden, July 2019.

[2] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, document JVET-O2001, JVET, Gothenburg, Sweden, July 2019.

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).