| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11October 2019 | Document: JVET-P0247 |
| --- | --- |

| Title: | Intra deblocking coefficients for weak filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luhang Xu, Jianqing Zhu, Kimihiko Kazui | Tel: Email: | +86-21-63350606 xuluhang@cn.fujitsu.com |
| Source: | Fujitsu |  |  |

_____________________________

- Abstract

New De-blocking coefficients are designed for intra block weak filtering. The simulated results show -0.83%/-0.87%/-0.70% BD-rate for All Intra configuration and -0.27%/-0.59%/-0.62% BD-rate for Random Access configuration over VTM-6.0 [1].

Introduction

De-blocking filters are designed for removing blocking effects. In latest VTM, De-blocking are categorized into longer tap filters, strong filters, weak filters for Luma and strong filters, weak filters for Chroma. Filters are in different shapes and controlled by different conditions. For each condition of de-blocking filters, blocking data of intra images (train data) were generated for training new coefficients which are suitable for intra blocks. Experimental results show that coding gain are improved while equivalent performance of blocking removal are achieved. Due to the trade-off computational complexity and performance, weak filter coefficients are selected for proposal.

Proposed method

It is proposed to use new coefficients for intra block weak filtering. There are 8 conditions longer tap filters, 1 condition strong filtering and 3 conditions weak filtering for Luma de-blocking, 1 condition strong filtering and 1 condition for weak filtering for Chroma de-blocking. For each condition, blocking boundary is generated from image set DIV2K [2] by the VTM encoder as training data. The corresponding pixels from original image/frame is used as ground truth. The coefficients are trained by a Fully-Connected Layer with loss function MSE.

It is found that no coding gain get from trained longer tap filter coefficients and little gain from trained strong filter coefficient, However, these coefficients bring much computational complexity increment. As a result, only trained weak filter coefficients are proposed to apply for intra block de-blocking.

Weak de-blocking control

All existing conditions are not changed and a new condition is defined as:

bool IntraDeblock = (cuP.predMode == MODE_INTRA &&cuQ.predMode == MODE_INTRA);

This condition is defined in both xEdgeFilterLuma and xEdgeFilterChroma, and when do the filtering in the function xPelFiterLuma and xPelFilterChroma, the filtering process are shown below:

if(sw){

Do long tap filtering and strong filtering;

}

else{

if(IntraDeblock){

Do the IntraDeblock with weak filter coefficients;

}

else{

Do weak filtering;

}

}

Intra De-blocking calculations

The intra block coefficients are quantized into 8 bit numbers. Calculations are shown in the equations below,

for luma weak filtering:

| Weak p0 | p0'=11×p1+80×p0+42×q0-5×q1+64≫7 |
| --- | --- |
| Weak q0 | q0'=-7×p1+21×p0+103×q0-11×q1+64≫7 |
| Weak secondP p1 | p1'=13×p2+104×p1+7×p0+6×q0-3×q1+q2+64≫7 |
| Weak secondQ q1 | q1'=-p2+p1-3×p0+35×q0-64×q1+32×q2+64≫7 |

for chroma weak filtering:

| Weak p0 | p0'=4×p1+109×p0+16×q0-q1+64≫7 |
| --- | --- |
| Weak q0 | q0'=-2×p1+12×p0+116×q0+2×q1+64≫7 |

Experimental results

The following test results are evaluated with SAO and ALF switched on.

No subjective quality improvement are observed.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.00% | -0.77% | -0.07% | 103% | 100% |
| Class A2 | -0.82% | -0.77% | -0.99% | 108% | 103% |
| Class B | -0.97% | -1.01% | -0.87% | 99% | 100% |
| Class C | -0.63% | -1.05% | -1.09% | 94% | 107% |
| Class E | -0.71% | -0.57% | -0.25% | 91% | 104% |
| Overall | -0.83% | -0.87% | -0.70% | 99% | 103% |
| Class D | -0.55% | -0.84% | -0.86% | 91% | 109% |
| Class F | -0.39% | -0.98% | -1.27% | 97% | 105% |

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.21% | 0.07% | 103% | 100% |
| Class A2 | -0.24% | -0.36% | -0.49% | 103% | 100% |
| Class B | -0.30% | -0.72% | -0.88% | 100% | 99% |
| Class C | -0.26% | -0.86% | -0.92% | 96% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.27% | -0.59% | -0.62% | 100% | 100% |
| Class D | -0.26% | -0.71% | -0.72% | 80% | 99% |
| Class F | -0.20% | -0.88% | -1.10% | 91% | 100% |

References

- “Versatile Video Coding (Draft 6)”, B. Bross, J. Chen, S. Liu, Doc. JVET-O2001, July. 2019.

- DIV2K, http://data.vision.ee.ethz.ch/cvl/DIV2K/.

Patent rights declaration(s)

Fujitsu may have current or pendingpatent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).