| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0261-v1 |
| --- | --- |

| Title: | Non-CE4: On Affine Motion Vector Restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuewei Meng, Suhong Wang, Xiaozhen Zheng, Shanshe Wang, Siwei Ma | Tel: Email: | xwmeng@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | Peking University, SZ DJI Technology Co., Ltd., |  |  |

_____________________________

- Abstract

This contribution presents a MV spread limitation simplification method for Affine Uni-prediction. In Nth meeting, 6-tap interpolation filter and MV spread limitation technology are adopted to reduce the worst case memory bandwidth consumption caused by Affine mode. With the combination of the two methods, memory bandwidth burden of bi-prediction caused by affine mode is removed. However, for uni-prediction, there is no need to do affine block MV spread limitation, as there is no memory bandwidth burden with 6-tap interpolation filter. This contribution proposes to remove the MV spread limitation for Affine uni-prediction, which can simplify the MV derivation process by 4 Multiples, 20 Additions, 12 Shifts, 4 Max functions, 4 Min functions at most for each Affine uni-prediction CU without increasing the memory bandwidth. Compare to VTM-6.0, the proposed method achieves 0.00%, xx% coding performance change for AI and RA configuration with almost the same memory bandwidth consumption.

Introduction

Motion compensation (MC) is one of the largest memory bandwidth consumers in decoder side. Providing a reasonable limit on MC bandwidth requirement is important for the implementations of video coding standard.

Memory bandwidth requirement for MC is usually determined by motion compensation block size, the type of prediction (e.g. uni-directional or bi-directional) and the tap of interpolation filter. In the current VVC (VTM-6.0), the 4x4 inter CUs and the 4x8/8x4 bi-prediction are disallowed. As shown in Table1 and Table 3, the worst case memory bandwidth is calculated according to the method in [2]. The worst case memory bandwidth is caused by 4x16/16x4 bi-prediction CUs. As for affine mode, in order to   reduce the memory bandwidth consumption introduced by sub-CU based MC, the 6-tap luma interpolation filter [3] is adopted. As shown in Table2, 6-tap interpolation filter can reduce the memory bandwidth consumption significantly, while the memory bandwidth consumption of bi-prediction is still higher than normal inter. To solve this problem, affine block MV spread limitation [4] is adopted. For Bi-prediction, the MV spread area of each 8x8 block is limited to 15x15. For uni-prediction, the MV spread area of each 4x8/8x4 block is limited to 15x11. With the two methods, the worst case of affine mode is the same as normal inter mode.

However, there is no need to do affine block MV spread limitation for uni-prediction, because there is no memory bandwidth burden with 6-tap interpolation filter for Affine uni-prediction, as shown in Table2.

Table 1. Memory bandwidth consumption (per-pixel) of normal inter mode

| Block Size | Prediction mode | Luma (8 tap) | Chroma (4 tap) | All |
| --- | --- | --- | --- | --- |
| 4x8/8x4 | Uni | 5.16 | 2.18 | 7.34 |
| 4x16/16x4 | Bi/Triangle | 7.9 | 3.44 | 11.34 |
| 8x8 | Bi/Triangle | 7.03 | 3.06 | 10.09 |
| 4x32/32x4 | Bi/Triangle | 6.70 | 2.97 | 9.67 |
| 8x16/16x8 | Bi/Triangle | 5.39 | 2.41 | 7.80 |
| 4x64/64x4 | Bi/Triangle | 6.10 | 2.73 | 8.84 |
| 4x128/128x4 | Bi/Triangle | 5.80 | 2.62 | 8.42 |

Table 2. Memory bandwidth consumption (per-pixel) of Affine mode without MV Spread Limitation [4]

| Block Size | Prediction mode | Luma (6 tap) | Chroma (4 tap) | All |
| --- | --- | --- | --- | --- |
| MxN | Affine Uni | 5.06 | 2.18 | 7.24 |
| MxN | Affine Bi | 10.13 | 3.06 | 13.19 |

Table 3. Memory bandwidth consumption (per-pixel) of Affine mode with MV Spread Limitation

| Block Size | Prediction mode | δx | δy | Thred4 | Luma (6 tap) | Chroma (4 tap) | All |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MxN | Affine Uni | 2 | 2 | 15x11 | 5.06 | 2.18 | 7.24 |
| MxN | Affine Bi | 2 | 2 | 15x15 | 7.03 | 3.06 | 10.09 |

Table 4. Computation complexity (per-CU) introduced by MV Spread Limitation [4]

| Block Size | Prediction mode | Mult. | Add. | Shift | Max | Min |
| --- | --- | --- | --- | --- | --- | --- |
| MxN | Affine Bi | 2 | 20 | 18 | 6 | 6 |
| MxN | Affine Uni | 4 | 20 | 12 | 4 | 4 |

Proposed Method

In order to simplify the MV spread limitation process for Affine mode in encoder and decoder, this contribution proposes to remove the MV spread limitation process for Affine uni-prediction. With the proposed method, 4 Multiples, 20 Additions, 12 Shifts, 4 Max functions and 4 Min functions are reduced for each Affine Uni-prediction CUs.

Table 5. Computation complexity (per-CU) introduced by MV Spread Limitation (Proposed)

| Block Size | Prediction mode | Mult. | Add. | Shift | Max | Min |
| --- | --- | --- | --- | --- | --- | --- |
| MxN | Affine Bi | 2 | 20 | 18 | 6 | 6 |
| MxN | Affine Uni | - | - | - | - | - |

Simulation results

The experiments were performed based on VTM-6.0 software [5] with VTM configuration files under JVET common test condition [6]. And, the experiments were conducted under Ubuntu Linux. The encoding time is unreliable.

The experimental results of the proposed method compared to VTM-6.0 anchor are summarized in Table 6. The average BD-rates of 0.00%, and 0.xx% are observed for RA, and LDB cases, respectively.

Table 6. Experimental result of the proposed method

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | -0.01% | 103% | 100% |
| Class A2 | 0.02% | 0.02% | 0.04% | 110% | 100% |
| Class B | -0.02% | 0.04% | 0.08% | 100% | 100% |
| Class C | 0.00% | 0.07% | 0.17% | 104% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | 0.08% | 104% | 100% |
| Class D | -0.03% | 0.06% | 0.11% | 99% | 100% |
| Class F | 0.01% | -0.05% | -0.03% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.03% | -0.05% | 0.07% | 100% | 100% |
| Class E | -0.03% | 0.29% | -0.22% | 82% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.04% | 0.18% | 0.09% | 100% | 100% |
| Class F | 0.06% | 0.15% | 1.14% | 108% | 100% |

In order to measure the memory bandwidth more accurately, the decoder side memory bandwidth consumption is tested with macro ” JVET_J0090_MEMORY_BANDWITH_MEASURE ” on by using the configuration cache_1d.cfg. As shown in Table7, Anchor is VTM-6.0 with JVET_J0090_MEMORY_BANDWITH_MEASURE on by using configuration cache_1d.cfg. Test is the VTM-6.0 with proposed method. The memory bandwidth consumption change is minor.

Table 7. Experimental result of the proposed method (Memory bandwidth consumption)

|  | Random access Main10 |  |  | Low delay B Main10 |  | Low delay P Main10 |
| --- | --- | --- | --- | --- | --- | --- |
|  | cache_1d.cfg |  |  | cache_1d.cfg |  | cache_1d.cfg |
| Class A1 | 100% |  |  |  |  |  |
| Class A2 | 100% |  |  |  |  |  |
| Class B | 100% |  |  | 100% |  | 100% |
| Class C | 99% |  |  | 100% |  | 100% |
| Class E |  |  |  | 100% |  | 100% |
| Overall | 100% |  |  | 100% |  | 100% |
| Class D | 100% |  |  | 101% |  | 100% |
| Class F | 100% |  |  | 101% |  | 100% |

Conclusions

In this contribution, simplified MV spread limitation method is proposed for Affine Uni-prediction, which can simplify the MV derivation process by 4 Multiples, 20 Additions, 12 Shifts, 4 Max functions, 4 Min functions at most for each Affine uni-prediction CU without increasing the memory bandwidth. Compare to VTM-6.0, the proposed method achieves 0.00%, xx% coding performance change for AI and RA configuration with almost the same memory bandwidth consumption. Therefore, we propose to adopt this proposal to the next VVC.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-O2001, July. 2019

- Y. He, “BoG report on CE2 related contributions”, JVET-M0862, Jan. 2019

- J. Li, C.-W. Kuo, C. S.Lim, “CE2: Using the shorter-tap filter for 4x4 sized partitions (Test 2.4.6),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N0196, Mar. 2019

- M. Zhou, “CE2: On restriction of memory bandwidth consumption of affine mode (CE2-4.8),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N0068, Mar. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).