| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0262-v1 |
| --- | --- |

| Title: | AHG16/Non-CE5: Deblocking boundary design cleanup for affine and TPM mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuewei Meng, Xiaozhen Zheng, Shanshe Wang, Siwei Ma | Tel: Email: | xwmeng@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | Peking University, SZ DJI Technology Co., Ltd., |  |  |

_____________________________

- Abstract

This contribution addresses the inconsistency issue of current deblocking design for affine and TPM in some corner cases as discussed in AHG16. The inconsistency mainly introduced by the deblocking boundary strength derivation process does not explicitly differentiate between transform unit edges and sub-CU edges.

In order to resolve this problem, two methods are proposed. The first one is to redesign the edge decision process by separating the deblocking filter for a TU edge from that of a sub-CU edge by making the following changes: 1) using CBFs to make decision only for TU edges; 2) using MV differences to make decisions only for sub-CU edges and 3) if an edge is both a TU edge and a sub-CU edge, de-blocking is on if either 1) or 2) is true.

The second one is to redesign the boundary strength decision process by disable the applying of MV difference related conditions for internal edges belong to a TPM coded CU or internal sub-CU edges not overlap on 8x8 sample grid.

Introduction

The affine mode and triangle partition merge (TPM) mode are two newly adopted tools in VVC. Because of the sub-block based MC of Affine and triangular based MC of TPM, the MVs stored in CU may be different from each other. For Affine mode, each sub-CU owns a MV derived by 4-parameter model or 6-parameter model. For Triangle mode, as the blending process for diagonal area, the MV storage process is different in diagonal area. It is asserted that the current deblocking design for those two modes is not consistent in some corner cases.

The following examples (as shown in Figure1 and Figure2) will be used to elaborate the deblocking design inconsistency in affine and TPM mode.

Figure 1.  Deblocking filter example in affine mode when SBT is used

As shown in Figure1, the CU size is 16x32 coded in Affine mode. Therefore, there are three inner vertical sub-block boundaries marked 1, 2, 3 in the figure above.

The CU uses quad SBT, therefore, sub-block boundary 1 is a TU boundary. In this case, there are no non-zero coefficients in the current component. However, the sub-block MV difference is large enough along the sub-block boundaries to trigger deblocking. This may lead to over smoothing at edge 1.

Although sub-block boundary 3 may have absolutely the same conditions as sub-block boundary 1, i.e. no non-zero coefficients along the edge, and MV difference may be large enough to trigger deblocking, deblocking is not applied for the edge 3, because edge 3 does not overlap with a TU edge (which is in contrast to edge 1). This leads to inconsistency.

Similar behavior may exist in TPM along the diagonal TPM boundaries, as shown in Figure2.  Yellow lines are TU edges and black lines are 4x4 grids for MV storage. The edge between Block P and Block Q is a TU boundary. This edge is filtered even if the TU boundary has no coefficients, because of MV differences in the triangle block MV field (Bi-MV for BlockP and Uni-MV for BlockQ). And these artificial triangle block MV differences don’t line up with the actual diagonal MV edge anyway. This may result in over-smoothing, therefore resulting in a poor visual quality.

Figure 2.  Deblocking filter example in TPM mode

Proposed Method

In order to resolve these problems mentioned above, two methods are proposed. The first one is to redesign edge decision process to separate the TU edges and MV edges. The second one is to modify the boundary strength derivation process to disable the mentioned corner cases.

Method1: Redesign edge decision process

It is proposed to redesign edge decision process as follows:

- Use CBFs to make decision only for TU edges.

- Using MV differences to make decisions only for MV edges.

- If an edge is both a TU edge and an MV edge, de-blocking is on if either (1) or (2) is true.

Method2: Modify boundary strength derivation process

The spec changes are shown bellow to elaborate the proposed method.

    - Otherwise, if the prediction mode of the coding subblock containing the sample p0 is different from the prediction mode of the coding subblock containing the sample q0 (i.e. one of the coding subblock is coded in IBC prediction mode and the other is coded in inter prediction mode), bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if edgeType is equal to EDGE_VER, xDi is not equal to 0 and ( xDi % 8 ) is not equal to 0, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if edgeType is equal to EDGE_HOR, yDj is not equal to 0 and ( yDi % 8 ) is not equal to 0, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if MergeTriangleFlag[ xCb ][ yCb ] is equal to 1, edgeType is equal to EDGE_VER, and xDi is not equal to 0, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if MergeTriangleFlag[ xCb ][ yCb ] is equal to 1, edgeType is equal to EDGE_HOR, and yDj is not equal to 0, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if cIdx is equal to 0 and one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

Simulation results

The experiments were performed based on VTM-6.1 software [2] with VTM configuration files under JVET common test condition [3]. And, the experiments were conducted under Ubuntu Linux. The encoding time is unreliable.

The experimental results of the proposed method1 compared to VTM-6.1 anchor are summarized in Table 1. The average BD-rates of 0.00%, and 0.xx% are observed for RA, and LDB cases, respectively.

The experimental results of the proposed method2 compared to VTM-6.1 anchor are summarized in Table 2. The average BD-rates of 0.00%, and 0.xx% are observed for RA, and LDB cases, respectively.

Table 1. Experimental result of the proposed method (Method1)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.01% | -0.02% | 101% | 116% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 116% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 109% | 124% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.01% | -0.19% | -0.10% | 124% | 137% |
| Class E | 0.01% | 0.02% | 0.02% | 114% | 137% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | 0.03% | 0.19% | 123% | 137% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 2. Experimental result of the proposed method (Method2)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.01% | -0.01% | 102% | 117% |
| Class C | 0.00% | 0.00% | 0.01% | 108% | 122% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 114% |
| Class F | 0.00% | 0.00% | 0.01% | 99% | 127% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.05% | -0.16% | -0.09% | 121% | 131% |
| Class E | 0.03% | -0.15% | 0.02% | 116% | 140% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | 0.03% | 0.09% | 126% | 142% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusions

In this contribution, two methods are proposed to resolve the inconsistency issue identified for the Affine and TPM deblocking. There is almost no coding performance change under CTC conditions in VTM-6.1. It is suggested to adopt one of the suggested methods to the next VVC.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-O2001, July. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.1

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).