| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0269-v2 |
| --- | --- |

| Title: | AHG18/Non-CE5: Deblocking for TPM and BCW |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuewei Meng, Xiaozhen Zheng, Shanshe Wang, Siwei Ma | Tel: Email: | xwmeng@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | Peking University, SZ DJI Technology Co., Ltd., |  |  |

_____________________________

- Abstract

This contribution addresses the potential blocking artifacts of current deblocking design for TPM and BCW in some corner cases as discussed in AHG18. The potential discontinuity of block boundary is mainly caused by the discrepancy of the weights in TPM and BCW mode. In order to resolve this problem, this contribution proposes to conduct deblocking filtering on these overlooked boundaries in current VVC, including the boundaries between two CU with different BcwIdx and boundaries between TPM blending area and another CU.

For TPM boundaries，this contribution proposes to set boundary strength for luma to be 1 between two boundaries between TPM blending area and another CU. Compare to VTM-6.1, the proposed method achieves 0.00%, 0.00% coding performance change for RA and LDB configuration with better subjective quality.

For BCW boundaries，this contribution proposes to set boundary strength for luma to be 1 between two CUs with different BcwIdx. Compare to VTM-6.1, the proposed method achieves 0.02%, -0.01% coding performance change for RA and LDB configuration with better subjective quality.

Introduction

The triangle partition merge (TPM) mode and Bi-prediction with CU-level Weights (BCW) are two newly adopted tools in VVC. TPM conducts motion compensation based on triangular PU and then do blending process on the diagonal area, as shown in Figure1. BCW introduces different weight options for bi-prediction CU.

Figure 1.  Blending process of TPM

The following examples (as shown in Figure3 and Figure4) will be used to elaborate the potential discontinuity of block boundary caused by the discrepancy of the weights in TPM and BCW mode.

As shown in Figure2, the motion vectors of a CU coded in triangle partition mode are stored in 4x4 units. Depending on the position of each 4x4 unit, either uni-prediction or bi-prediction motion vectors are stored. Denote Mv1 and Mv2 as uni-prediction motion vectors for partition 1 and partition 2, respectively. If a 4x4 unit is located in the non-weighted area shown in the example of Figure2, either Mv1 or Mv2 is stored for that 4x4 unit. Otherwise, if the 4x4 unit is located in the weighted area, a bi-prediction motion vector is stored. The bi-prediction motion vector is derived from Mv1 and Mv2 according to the following process:

- If Mv1 and Mv2 are from different reference picture lists (one from L0 and the other from L1), then Mv1 and Mv2 are simply combined to form the bi-prediction motion vector.

- Otherwise, if Mv1 and Mv2 are from the same list, and without loss of generality, assume they are both from L0. In this case, instead of bi-prediction motion, only uni-prediction motion Mv2 is stored.

Because of the special MV storage method and spatially varying weighted average process for diagonal area, some potential blocking artifacts may occur.

Figure 2.  Motion vector storage of TPM

As shown in Figure 3(a), block P is coded by TPM (L0: MVp1, L1: MVp2) and block Q is coded by full-block bi-prediction (L0: MVq1, L1: MVq2). When MVp1 and MVp2 are identical or mostly equal to MVq1 and MVq2, respectively, deblocking is not triggered if both blocks are skipped. However, the prediction of block P is generated by spatially varying weighted average and that of the block Q is generated by equal weighted average. This misalignment may cause discontinuity of the block boundary with is overlooked in the current VTM design.

Similar as Figure3(a), Figure3(b) shows the example where block P is coded by TPM (L0: MVp) and block Q is coded by full-block bi-prediction (L0: MVq). When MVp is identical or mostly equal to MVq, deblocking is not triggered if both blocks are skipped. However, the prediction of block P is generated by spatially varying weighted average and that of the block Q is uni-predicted. This may cause discontinuity.

- (b)

Figure 3.  Deblocking filter example in TPM

Similar behavior may exist in BCW as shown in Figure 4. In the derivation process of boundary strength, the CU-level weight of inter prediction is not considered in current deblocking design. There are five BCW options corresponding to five weights w∈-2, 3, 4, 5, 10. When the absolute difference of MVs of block P and block Q is smaller than threshold and P and Q are skipped, deblocking is not triggered. When the weights of P and Q are different, discontinuity may occur.

Figure 4.  Deblocking filter example in BCW

Proposed Method

In order to resolve these problems mentioned above, two methods are proposed.

Method1: Deblocking for TPM

The spec changes are shown bellow to elaborate the proposed method.

      - Two motion vectors for the same reference picture are used to predict the coding subblock containing the sample p0, two motion vectors for the same reference picture are used to predict the coding subblock containing the sample q0 and both of the following conditions are true:

        - The absolute difference between the horizontal or vertical component of list 0 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in 1/16 luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

        - The absolute difference between the horizontal or vertical component of list 0 motion vector used in the prediction of the coding subblock containing the sample p0 and the list 1 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 8 in units of 1/16 luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vector used in the prediction of the coding subblock containing the sample p0 and list 0 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 8 in units of 1/16 luma samples.

        - The coding subblock containing the sample p0 is triangle mode, and the coding subblock containing the sample p0 is different from that of the sample q0, and sample is in the blending area of corresponding coding subblock.

        - The coding subblock containing the sample q0 is triangle mode, and the coding subblock containing the sample p0 is different from that of the sample q0, and sample is in the blending area of corresponding coding subblock.

    - Otherwise, the variable bS[ xDi ][ yDj ] is set equal to 0.

Method2: Deblocking for BCW

The spec changes are shown bellow to elaborate the proposed method.

      - Two motion vectors for the same reference picture are used to predict the coding subblock containing the sample p0, two motion vectors for the same reference picture are used to predict the coding subblock containing the sample q0 and both of the following conditions are true:

        - The absolute difference between the horizontal or vertical component of list 0 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in 1/16 luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

        - The absolute difference between the horizontal or vertical component of list 0 motion vector used in the prediction of the coding subblock containing the sample p0 and the list 1 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 8 in units of 1/16 luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vector used in the prediction of the coding subblock containing the sample p0 and list 0 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 8 in units of 1/16 luma samples.

        - The BcwIdx of coding subblock containing the sample p0 is different that of coding subblock containing the sample q0

    - Otherwise, the variable bS[ xDi ][ yDj ] is set equal to 0.

Simulation results

The experiments were performed based on VTM-6.1 software [2] with VTM configuration files under JVET common test condition [3]. And, the experiments were conducted under Ubuntu Linux. The encoding time is unreliable.

The experimental results of the proposed method1 compared to VTM-6.1 anchor are summarized in Table 1. The average BD-rates of 0.00%, and 0.00% are observed for RA, and LDB cases, respectively.

The experimental results of the proposed method2 compared to VTM-6.1 anchor are summarized in Table 2. The average BD-rates of 0.02%, and -0.01% are observed for RA, and LDB cases, respectively.

Table 1. Experimental result of the proposed method (Method1 for TPM)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | -0.01% | 129% | 100% |
| Class A2 | 0.01% | 0.00% | -0.09% | 102% | 100% |
| Class B | 0.00% | 0.01% | -0.01% | 100% | 100% |
| Class C | 0.00% | 0.06% | -0.03% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | -0.03% | 106% | 100% |
| Class D | 0.01% | -0.09% | -0.02% | 101% | 97% |
| Class F | -0.02% | -0.02% | -0.01% | 102% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  | 0.01% | 0.04% |  | 0.07% |  | 150% |  | 100% |  |
| Class C |  | 0.01% | -0.18% |  | 0.09% |  | 122% |  | 99% |  |
| Class E |  | -0.03% | 0.05% |  | 0.13% |  | 116% |  | 99% |  |
| Overall |  | 0.00% | -0.03% |  | 0.09% |  | 131% |  | 100% |  |
| Class D |  | 0.03% | -0.39% |  | 0.22% |  | 123% |  | 99% |  |
| Class F |  | -0.01% | 0.06% |  | 0.32% |  | 116% |  | 99% |  |

Table 2. Experimental result of the proposed method (Method2 for BCW)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  | 0.03% | 0.03% |  |  | 0.05% |  |  | 101% |  | 100% |  |  |
| Class A2 |  | 0.04% | 0.11% |  |  | -0.11% |  |  | 103% |  | 100% |  |  |
| Class B |  | 0.01% | -0.02% |  |  | 0.00% |  |  | 102% |  | 100% |  |  |
| Class C |  | 0.01% | 0.08% |  |  | -0.03% |  |  | 101% |  | 100% |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  | 0.02% | 0.04% |  |  | -0.02% |  |  | 102% |  | 100% |  |  |
| Class D |  | -0.01% | -0.04% |  |  | -0.01% |  |  | 101% |  | 100% |  |  |
| Class F |  | 0.01% | -0.02% |  |  | 0.00% |  |  | 98% |  | 100% |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  | 0.03% |  | -0.03% |  |  | 0.08% |  |  | 126% |  |  | 100% |  |  |
| Class C |  | -0.06% |  | -0.03% |  |  | -0.10% |  |  | 121% |  |  | 99% |  |  |
| Class E |  | 0.01% |  | 0.32% |  |  | 0.17% |  |  | 115% |  |  | 100% |  |  |
| Overall |  | -0.01% |  | 0.06% |  |  | 0.04% |  |  | 121% |  |  | 100% |  |  |
| Class D |  | 0.06% |  | -0.42% |  |  | 0.23% |  |  | 123% |  |  | 98% |  |  |
| Class F |  | 0.00% |  | -0.03% |  |  | 0.31% |  |  | 104% |  |  | 100% |  |  |

Subjective results

VTM6.1-Anchor             VTM6.1 with proposed method1

Figure 5.  Subjective result of proposed method1 （KristenAndSara_Qp32, LDB）

VTM6.1-Anchor           VTM6.1 with proposed method2

Figure 5.  Subjective result of proposed method2 （BasketballDrill_Qp37, LDB）

Conclusions

In this contribution, two methods are proposed to resolve the potential blocking artifacts for the TPM and BCW deblocking. The proposed method can improve subjective quality. It is suggested to adopt the suggested methods to the next VVC.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-O2001, July. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.1

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).