| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0273 |
| --- | --- |

| Title: | Non-CE6: On MTSIntraMaxCand |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christopher Hollmann, Davood Saffar, Jacob Ström Torshamnsgatan 23 16480 Stockholm Sweden | Tel: Email: | +46107146437 {christopher.hollmann, davood.saffar, jacob.strom} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In this document a bugfix for the configuration parameter MTSIntraMaxCand is proposed, which by itself is claimed to have no impact on the CTC. Furthermore, it is proposed to set the CTC value of the parameter for AI and RA to from currently 3 to 1, avoiding testing transform combinations which are reportedly rarely used. A minor BD-rate impact on the CTC results is reported, namely 0.00% (Y), 0.00% (U), 0.01% (V) for AI and 0.01% (Y), 0.01% (U), -0.01% (V) for RA.

# Introduction

At the 13th JVET meeting in Marrakech, Morocco, a proposal changing the encoder search for the Multiple Transform Selection (MTS) tool was adopted into the VVC draft and test model [1]. While previously the encoder always tested all four combinations of DST-7 and DCT-8 before deciding which one to use, the proposal added a parameter in the configuration which limits the number of transform candidates that are tested. This parameter is called MTSIntraMaxCand. There is a corresponding parameter for inter-predicted blocks, MTSInterMaxCand.

At the 14th JVET meeting in Geneva, Switzerland, the Low Frequency Non-Separable Transform (LFNST) was adopted [2]. This tool also modifies the encoder search for the MTS tool, as both transform tools are tightly connected. The encoder search introduced by the LFNST contains early-exit points, at which the encoder can stop the search and select the best already tested combination. However, in theory it is possible that the encoder now always searches all four combinations of DST-7 and DCT-8, regardless of the value of MTSIntraMaxCand. If LFNST is disabled, the encoder algorithm described in [1] is applied. To summarize, MTSIntraMaxCand is only used if LFNST is disabled, even though there is no reason why it should not be used if LFNST is enabled.

As LFNST is only applied to intra-predicted blocks, the search for inter-predicted blocks is still regulated by MTSInterMaxCand. In the Common Test Conditions (CTC), however, MTS is disabled for inter-predicted blocks.

The number of candidates can be limited without significant losses in compression efficiency as not all transform combinations are equally probable. Table 1 shows the usage of different transform combinations in VTM-6.0 using CTC.

Table 1 Usage of different transform combinations (based on AI)

| Transform combination | Usage |
| --- | --- |
| Transform Skip | 3,5% |
| (DST-7, DST-7) | 48,1% |
| (DCT-8, DST-7) | ~0,0% |
| (DST-7, DCT-8) | ~0,0% |
| (DCT-8, DCT-8) | 0,0% |
| (DCT-2, DCT-2) | 25,4% |
| (DCT-2, DCT-2) + LFNST #1 | 12,1% |
| (DCT-2, DCT-2) + LFNST #2 | 11,0% |

# Proposal

Two aspects are proposed:

- It is proposed to apply the configuration parameter MTSIntraMaxCand to limit the encoder search if LFNST is enabled. This by itself has no impact on the CTC.

- The MTS combinations with both DST-7 and DCT-8 are very rarely used, as shown in Table 1. The table also shows that the combination (DCT-8, DCT-8) is in fact never used in the CTC. Therefore, it is proposed to set the value of the MTSIntraMaxCand parameter to 1 for the All Intra (AI) and Random Access (RA) configurations, forcing the encoder to skip the combinations with DCT-8. This has a very minor impact on the CTC, as shown in Section 2.1.

As additional information, Section 2.2 shows the test result for setting the parameter to 2. Setting the parameter to 3 has no impact on CTC or setting it to 4 would correspond to the current CTC, so these results are omitted.

From a theoretical standpoint, the change should reduce the encoder run time. This can, however, not be measured with our current cluster. The decoder run time should not be affected at all. There is also no complexity impact.

As the LFNST is not applied in the Low Delay (LD) configuration, it is proposed to keep the parameter as it currently is without changing the value. Therefore, results for LD are omitted as well.

## Test results for MTSIntraMaxCand = 1

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.00% | 99% | 99% |
| Class A2 | 0.00% | 0.03% | 0.01% | 100% | 102% |
| Class B | 0.00% | 0.00% | 0.01% | 102% | 102% |
| Class C | 0.00% | -0.04% | 0.00% | 100% | 107% |
| Class E | 0.00% | 0.01% | 0.02% | 100% | 106% |
| Overall | 0.00% | 0.00% | 0.01% | 100% | 103% |
| Class D | 0.00% | 0.01% | 0.00% | 102% | 102% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 104% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | -0.07% | 101% | 99% |
| Class A2 | 0.02% | 0.00% | 0.04% | 101% | 103% |
| Class B | -0.01% | 0.04% | -0.04% | 100% | 100% |
| Class C | 0.02% | 0.02% | 0.04% | 101% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | -0.01% | 101% | 100% |
| Class D | -0.01% | -0.02% | -0.03% | 102% | 102% |
| Class F | -0.01% | -0.01% | -0.03% | 110% | 100% |

## Test results for MTSIntraMaxCand = 2

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.01% | 100% | 105% |
| Class A2 | 0.00% | 0.00% | 0.00% | 98% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 103% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 102% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 100% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class A2 | 0.00% | 0.00% | 0.01% | 101% | 100% |
| Class B | 0.00% | 0.01% | 0.00% | 101% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.01% | 100% | 103% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 101% |

# References

[1] B. Bross et al. Non-CE8: Unified Transform Type Signalling and Residual Coding for Transform Skip. JVET-M0464, Marrakesh, Morocco, January 2019

[2] M. Koo et al. CE6: Reduced Secondary Transform (RST) (CE6-3.1). JVET-N0193, Geneva, Switzerland, March 2019

# Patent rights declaration(s)

Ericsson does not have any current or pending patent rights relating to the technology described in this contribution.