| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0277 |
| --- | --- |

| Title: | Non-CE3: LFNST restriction based on MIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ankit Kumar Sandeep Shrestha Bumshik Lee Yonggil Lee Junsuk Park | Email: | ankitku@chosun.kr sandeepshrestha407@chosun.kr bslee@chosun.ac.kr yglee@humaxdigital.com parkjs@humaxdigital.com |
| Source: | Chosun University and Humax |  |  |

_____________________________

- Abstract

In VTM-6.0 [1], MIP [2] and LFNST [3] have been harmonized, where LFNST is allowed for MIP-modes on blocks of width W and height H if both W and H are greater than or equal to 16. Based on the luma block size, LFNST is applied on MIP luma block. In this contribution, we propose two tests to apply LFNST on MIP blocks. First, where either of height or width is 32 or both are equal to 16, which includes 4×32, 8×32, 16×16, 16×32, 32×4, 32×8, 32×16, 32×32, 32×64 and 64×32 block sizes. In the second test , we apply LFNST on blocks whose either (height and width both greater than or equal to 16) or               ( maximum(height, width) greater than or equal to 32 && minimum(height, width) greater than or equal to 8), which includes 8×32, 8×64, 16×16, 16×32, 16×64, 32×8, 32×16, 32×32, 32×64, 64×8, 64×16, 64×32 and 64×64 block sizes.

Proposed Method

The Fig. 1 shows LFNST allowed for MIP-modes on blocks of width W and height H if both W and H are greater than or equal to 16, currently in VTM-6.0.

| 4×4 | 8×4 | 16×4 | 32×4 | 64×4 |
| --- | --- | --- | --- | --- |
| 4×8 | 8×8 | 16×8 | 32×8 | 64×8 |
| 4×16 | 8×16 | 16×16 | 32×16 | 64×16 |
| 4×32 | 8×32 | 16×32 | 32×32 | 64×32 |
| 4×64 | 8×64 | 16×64 | 32×64 | 64×64 |

Fig. 1. LFNST is applied on the marked MIP blocks in VTM-6.0

Test 1:

In this test, we propose to apply LFNST to MIP coded blocks whose either height or width is 32 or both are equal to 16. Fig. 2 shows the block sizes taken in consideration for proposed test 1.

| 4×4 | 8×4 | 16×4 | 32×4 | 64×4 |
| --- | --- | --- | --- | --- |
| 4×8 | 8×8 | 16×8 | 32×8 | 64×8 |
| 4×16 | 8×16 | 16×16 | 32×16 | 64×16 |
| 4×32 | 8×32 | 16×32 | 32×32 | 64×32 |
| 4×64 | 8×64 | 16×64 | 32×64 | 64×64 |

Fig 2. LFNST is applied on the marked MIP coded blocks for proposed Test 1

Test 2:

In this test, we propose to apply LFNST to MIP coded blocks whose either (height and width both greater than or equal to 16) or (maximum (height, width) greater than or equal to 32 && minimum (height, width) greater than or equal to 8). Fig. 3 shows the block sizes taken in consideration for proposed test 2.

| 4×4 | 8×4 | 16×4 | 32×4 | 64×4 |
| --- | --- | --- | --- | --- |
| 4×8 | 8×8 | 16×8 | 32×8 | 64×8 |
| 4×16 | 8×16 | 16×16 | 32×16 | 64×16 |
| 4×32 | 8×32 | 16×32 | 32×32 | 64×32 |
| 4×64 | 8×64 | 16×64 | 32×64 | 64×64 |

Fig 3. LFNST is applied on the marked MIP coded blocks for proposed Test 2

Proposed Text

The spec changes for the proposed method for Test 1:

- 7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| If ( Min( lfnstWidth, lfnstHeight ) >= 4) && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && ( !intra_mip_flag[ x0 ][ y0 ] ¦¦ ( (lfnstWidth, lfnstHeight) == 16) ¦¦ lfnstWidth == 32 ¦¦ lfnstHeight == 32) && tu_mts_idx[ x0 ][ y0 ] = = 0 && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( LfnstDcOnly = = 0 && LfnstZeroOutSigCoeffFlag = = 1 ) { |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |

The spec changes for the proposed method for Test 2:

- 7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| If ( Min( lfnstWidth, lfnstHeight ) >= 4) && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && ( !intra_mip_flag[ x0 ][ y0 ] ¦¦ ( Min (lfnstWidth, lfnstHeight) >= 16 ¦¦ ( Max(lfnstWidth,lfnstHeight) >= 32 && Min(lfnstWidth,lfnstHeight) >=8 ) ) && tu_mts_idx[ x0 ][ y0 ] = = 0 && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( LfnstDcOnly = = 0 && LfnstZeroOutSigCoeffFlag = = 1 ) { |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |

Experimental results

The proposed method has been implemented on VTM-6.0 reference software and evaluated for VTM test according to common test conditions for AI and RA configurations [4].

For Test 1:

Table 1. Experimental results for AI configurations over VTM-6.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.07% | 0.05% | 90% | 108% |
| Class A2 | 0.00% | 0.03% | 0.00% | 90% | 106% |
| Class B | 0.00% | 0.02% | -0.03% | 90% | 107% |
| Class C | 0.00% | -0.07% | -0.01% | 94% | 104% |
| Class E | -0.01% | 0.00% | 0.06% | 92% | 106% |
| Overall | 0.00% | 0.00% | 0.01% | 91% | 106% |
| Class D | -0.01% | 0.01% | 0.03% | 96% | 103% |
| Class F | -0.01% | -0.07% | 0.02% | 92% | 105% |

Table 2. Experimental results of RA configurations over VTM-6.0

|  | Random access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.01% | 0.08% | 101% | 100% |
| Class A2 | 0.03% | 0.10% | 0.06% | 102% | 100% |
| Class B | -0.01% | 0.16% | 0.08% | 100% | 100% |
| Class C | 0.01% | 0.04% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.09% | 0.05% | 100% | 100% |
| Class D | -0.01% | -0.01% | 0.05% | 100% | 99% |
| Class F | 0.00% | 0.01% | -0.01% | 100% | 101% |

For Test 2:

Table 3. Experimental results for AI configurations over VTM-6.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | 0.09% | 98% | 108% |
| Class A2 | 0.01% | 0.05% | 0.01% | 97% | 106% |
| Class B | 0.00% | 0.05% | -0.02% | 98% | 107% |
| Class C | 0.00% | -0.07% | -0.01% | 98% | 104% |
| Class E | 0.00% | 0.09% | 0.04% | 98% | 104% |
| Overall | 0.00% | 0.02% | 0.02% | 98% | 106% |
| Class D | -0.01% | 0.00% | 0.03% | 95% | 101% |
| Class F | -0.01% | -0.08% | 0.02% | 98% | 104% |

Table 4. Experimental results of RA configurations over VTM-6.0

|  | Random access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.07% | 0.00% | 101% | 100% |
| Class A2 | 0.03% | 0.15% | 0.05% | 103% | 199% |
| Class B | 0.00% | 0.04% | -0.02% | 100% | 100% |
| Class C | -0.01% | 0.07% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.08% | 0.00% | 101% | 100% |
| Class D | -0.01% | -0.02% | 0.06% | 100% | 100% |
| Class F | -0.01% | 0.04% | 0.01% | 100% | 101% |

Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, 15th Meeting: Gothenburg, SE, 3–12 July 2019

- J. Pfaff et al., “CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)”, JVET-N0217, Geneva, Switzerland, March 2019

- M. Koo et al., “CE6: Reduced Secondary Transform (RST)(CE6-3.1)”, JVET-N0193, 14th meeting, Geneva, CH, 19-27 March 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th JVET meeting, Geneva, 19–27 Mar. 2019.

Patent rights declaration(s)

Chosun University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Humax may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).