| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0279-r1 |
| --- | --- |

| Title: | Non-CE4: Interaction between PROF and other tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Junghak Nam Hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, junghak.nam, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In VTM6.0, newly adopted PROF scheme is allowed when BCW or WP is applied. Also, PROF is performed when the size of the reference picture and current picture is different. On the other hand, BDOF for normal mode is allowed when both BCW and WP are turned off, and the size of reference picture and current picture are same. In this proposal, PROF enabling conditions are adjusted to align with BDOF by considering the following restrictions.

- PROF is restricted when the weighted prediction (BCW or WP) is available

- PROF is restricted when the size of the reference picture and current picture is different.

The experimental results reportedly show 0.03% (0.02%) BD-rate changes with 100% and 100% encoding and decoding run-time compared to VTM6.0 as anchor in RA(LB) configurations.

Introduction

BDOF process is performed when all of the following conditions are satisfied as described in VVC Draft [1]. As following conditions, BDOF process is restricted when weighted prediction scheme (BCW and WP) is applied. Also, BDOF process is restricted when the size of the reference picture and the current picture is different.

- If all of the following conditions are true, bdofFlag is set equal to TRUE. - sps_bdof_enabled_flag is equal to 1 and slice_disable_bdof_dmvr_flag is equal to 0. - predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1. - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0. - MotionModelIdc[ xCb ][ yCb ] is equal to 0. - merge_subblock_flag[ xCb ][ yCb ] is equal to 0. - sym_mvd_flag[ xCb ][ yCb ] is equal to 0. - ciip_flag[ xCb ][ yCb ] is equal to 0. - BcwIdx[ xCb ][ yCb ] is equal to 0. - luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are both equal to 0. - cbWidth is greater than or equal to 8. - cbHeight is greater than or equal to 8. - cbHeight * cbWidth is greater than or equal to 128. - For X being each of 0 and 1, the pic_width_in_luma_samples and pic_height_in_luma_samples of the reference picture refPicLX associated with the refIdxLX are equal to the pic_width_in_luma_samples and pic_height_in_luma_samples of the current picture, respectively. - cIdx is equal to 0.

The PROF scheme is similar with BDOF which is using the optical flow but as following table, the conditions between PROF and BDOF are different [2].

- If one or more of the following conditions are true, cbProfFlagLX is set equal to FALSE. - sps_affine_prof_enabled_flag is equal to 0. - fallbackModeTriggered is equal to 1. - numCpMv is equal to 2 and cpMvLX[ 1 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 1 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ]. - numCpMv is equal to 3 and cpMvLX[ 1 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 1 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ] and cpMvLX[ 2 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 2 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ]. - Otherwise, cbProfFlagLX set equal to TRUE.

Proposed method

In this proposal, conditions for PROF are aligned to those of BDOF.

- Aspect 1: PROF is restricted when the weighted prediction (BCW or WP) is available.

- Aspect 2: PROF is restricted when the size of the reference picture and current picture is different.

By applying the proposed aspect 1, the VVC Draft can be modified as follows.

- If one or more of the following conditions are true, cbProfFlagLX is set equal to FALSE. - sps_affine_prof_enabled_flag is equal to 0. - fallbackModeTriggered is equal to 1. - numCpMv is equal to 2 and cpMvLX[ 1 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 1 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ]. - numCpMv is equal to 3 and cpMvLX[ 1 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 1 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ] and cpMvLX[ 2 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 2 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ]. - BcwIdx[ xCb ][ yCb ] is not equal to 0. - luma_weight_lX_flag[ refIdxLX ] is equal to 1. - Otherwise, cbProfFlagLX set equal to TRUE.

Also, by applying the proposed aspect 2, the VVC Draft can be modified as follows. And It was already implemented on VTM6.1.

- If one or more of the following conditions are true, cbProfFlagLX is set equal to FALSE. - sps_affine_prof_enabled_flag is equal to 0. - fallbackModeTriggered is equal to 1. - numCpMv is equal to 2 and cpMvLX[ 1 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 1 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ]. - numCpMv is equal to 3 and cpMvLX[ 1 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 1 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ] and cpMvLX[ 2 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 2 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ]. - The pic_width_in_luma_samples or pic_height_in_luma_samples of the reference picture refPicLX associated with the refIdxLX are not equal to the pic_width_in_luma_samples or pic_height_in_luma_samples of the current picture, respectively. - Otherwise, cbProfFlagLX set equal to TRUE.

Experimental results

The proposed aspect 1 has been implemented on top of the VTM6.0 with default setting and tested using JVET SDR common test condition [2]. The experimental results reportedly show 0.03% (0.02%) BD-rate changes without encoding/decoding complexity changes compared to VTM 6.0 as Anchor in RA(LB).

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.06% | -0.08% | 100% | 102% |
| Class A2 | 0.05% | 0.05% | 0.06% | 100% | 101% |
| Class B | 0.03% | 0.17% | 0.02% | 100% | 101% |
| Class C | 0.02% | 0.07% | -0.03% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.07% | 0.00% | 100% | 101% |
| Class D | -0.01% | 0.05% | 0.07% | 100% | 101% |
| Class F | 0.05% | 0.08% | 0.05% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.09% | -0.04% | 100% | 100% |
| Class C | 0.00% | 0.04% | -0.02% | 100% | 100% |
| Class E | 0.02% | 0.66% | 0.43% | 99% | 100% |
| Overall | 0.02% | 0.14% | 0.08% | 100% | 100% |
| Class D | 0.05% | 0.24% | 0.25% | 100% | 98% |
| Class F | 0.07% | 0.03% | 0.20% | 100% | 100% |

Also, proposed aspect 1 is tested on top of the VTM6.1, which is fixed what reported bugs for PROF in bug tracker. The experimental results reportedly show 0.03% (0.xx%) BD-rate changes without encoding/decoding complexity changes compared to VTM 6.1 as Anchor in RA(LB).

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.03% | -0.07% | 100% | 99% |
| Class A2 | 0.05% | 0.09% | 0.02% | 100% | 100% |
| Class B | 0.02% | 0.17% | 0.04% | 100% | 100% |
| Class C | 0.03% | 0.03% | -0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.08% | -0.01% | 100% | 100% |
| Class D | -0.01% | 0.01% | -0.07% | 100% | 100% |
| Class F | 0.04% | 0.07% | 0.07% | 100% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 0.01% | 0.07% | -0.11% | 100% | 101% |
| Class E | -0.19% | -0.20% | 0.01% | 100% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.05% | 0.16% | 0.43% | 100% | 97% |
| Class F | 0.13% | -0.23% | 0.01% | 100% | 102% |

Conclusion

In this contribution, conditions for PROF are modified considering the BCW index and weighting flags of the explicit weighted prediction. Also, PROF is restricted the size of the reference picture and current picture is different. The experimental results reportedly show 0.03% (0.02%) BD-rate changes with 100% and 100% encoding and decoding run-time compared to VTM6.0 as anchor in RA(LB) configurations. It is recommended to consider this method in the next VTM.

Reference

- B. Bross, et al., “Versatile Video Coding (Draft 6)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-MO2001, 15th Meeting: Gothenburg, SE, 3–12 July. 2019.

- J. Luo, et al., “CE4-2.1: Prediction refinement with optical flow for affine mode”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-O0070, 15th Meeting: Gothenburg, SE, 3–12 July. 2019.

- Frank Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).