| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0280 |
| --- | --- |

| Title: | Non-CE4: Fix the behavior between BCW and WP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Junghak Nam Hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, junghak.nam, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In current spec text, there are two methods for weighted prediction, which are default weighted prediction (BCW) and explicit weighted prediction (WP). Since BCW is a tool to control by the coding unit, BCW index is parsed when explicit weighting flags of both reference pictures are false. It means that if weighting flags of the reference pictures for the current CU are not activated, BCW index is valid even though the WP is enabled in PPS. However, in the weighted prediction process, the BCW index of the current CU is ignored when the WP is enabled in PPS. In this proposal, three solutions to fix the behavior between BCW and WP are introduced for normal block and affine block (with PROF).

Introduction

In coding unit syntax, bcw index, bcw_idx is parsed when weighting flag of each reference picture are FALSE as follows.

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| …… |  |
| if( sps_bcw_enabled_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && luma_weight_l0_flag[ ref_idx_l0 [ x0 ][ y0 ] ] = = 0 && luma_weight_l1_flag[ ref_idx_l1 [ x0 ][ y0 ] ] = = 0 && chroma_weight_l0_flag[ ref_idx_l0 [ x0 ][ y0 ] ] = = 0 && chroma_weight_l1_flag[ ref_idx_l1 [ x0 ][ y0 ] ] = = 0 && cbWidth * cbHeight >= 256 ) |  |
| bcw_idx[ x0 ][ y0 ] | ae(v) |
| …… |  |
| } |  |

However, in the weighted prediction process, weightedPredFlag (to decide BCW or WP) is derived using syntaxes (pps_weighted_pred_flag, pps_weighted_bipred_flag) parsed in PPS as following process.

The variable weightedPredFlag is derived as follows: - If slice_type is equal to P, weightedPredFlag is set equal to pps_weighted_pred_flag. - Otherwise (slice_type is equal to B), weightedPredFlag is set equal to pps_weighted_bipred_flag. The following applies: - If weightedPredFlag is equal to 0, the array pbSamples of the prediction samples is derived by invoking the default weighted sample prediction process as specified in clause 8.5.6.6.2 with the coding block width nCbW, the coding block height nCbH, two (nCbW)x(nCbH) arrays predSamplesL0 and predSamplesL1, the prediction list utilization flags predFlagL0 and predFlagL1, the bi-prediction weight index bcwIdx and the bit depth bitDepth as inputs. - Otherwise (weightedPredFlag is equal to 1), the array pbSamples of the prediction samples is derived by invoking the weighted sample prediction process as specified in clause 8.5.6.6.3 with the coding block width nCbW, the coding block height nCbH, two (nCbW)x(nCbH) arrays predSamplesL0 and predSamplesL1, the prediction list utilization flags predFlagL0 and predFlagL1, the reference indices refIdxL0 and refIdxL1, the colour component index cIdx and the bit depth bitDepth as inputs.

Proposed method

The BCW index for current CU can be used even though the WP syntax (pps_weighted_pred_flag or pps_weightd_bipred_flag) in PPS is TRUE because the BCW index is parsed when the weighting flag of the reference pictures of the current CU are all FALSE. However, although BCW as a tool was designed to have a higher priority than WP, BCW index is ignored when the WP syntax is TRUE in weighted prediction process.

In current spec text, since the weighted prediction flags are checked earlier than BCW index, decoding process of the weighted prediction is performed as Figure 1.

Figure 1

And, in the current VTM6.0, the priority of the BCW index is higher than the weighted prediction flags as Figure 2.

Figure 2

In this proposal, to fix the mismatch in behavior between BCW and WP, following three methods are proposed. By using the proposal 1, spec text is aligned to the SW and let BCW have higher priority. And, in the proposal 2, the variable, weightedPredFlag is derived using the weighted prediction flag of the reference pcitures not weighted prediction flags in PPS and it also allows BCW to have higher priority. By the proposal 2, the condition in both bcw index parsing and weighted prediction process are matched. Finally, by change the parsing condition of the bcw_idx as proposal 3, BCW index can be parsed when the weighted prediction flags in PPS are disabled. According to the proposal 2 and 3, both spec text and SW are changed.

Proposal 1: Check the BCW index

The variable weightedPredFlag is derived as follows: - If slice_type is equal to P, weightedPredFlag is set equal to pps_weighted_pred_flag. - Otherwise (slice_type is equal to B), weightedPredFlag is set equal to pps_weighted_bipred_flag. The following applies: - If weightedPredFlag is equal to 0 or bcwIdx is not equal to 0, the array pbSamples of the prediction samples is derived by invoking the default weighted sample prediction process as specified in clause 8.5.6.6.2 with the coding block width nCbW, the coding block height nCbH, two (nCbW)x(nCbH) arrays predSamplesL0 and predSamplesL1, the prediction list utilization flags predFlagL0 and predFlagL1, the bi-prediction weight index bcwIdx and the bit depth bitDepth as inputs. - Otherwise (weightedPredFlag is equal to 1 and bcwIdx is equal to 0), the array pbSamples of the prediction samples is derived by invoking the weighted sample prediction process as specified in clause 8.5.6.6.3 with the coding block width nCbW, the coding block height nCbH, two (nCbW)x(nCbH) arrays predSamplesL0 and predSamplesL1, the prediction list utilization flags predFlagL0 and predFlagL1, the reference indices refIdxL0 and refIdxL1, the colour component index cIdx and the bit depth bitDepth as inputs.

Proposal 2: Change the “weightedPredFlag” derivation

The variable weightedPredFlag is derived as follows: - If slice_type is equal to P the following applies: - If one or more following conditions are true, weightedPredFlag is set equal to 1, - luma_weight_l0_flag[ refIdxL0 ] is not equal to 0. - chroma_weight_l0_flag[ refIdxL0 ] is not equal to 0. - Otherwise, weightedPredFlag is set equal to 0. - Otherwise (slice_type is equal to B), the following applies: - If one or more following conditions are true, weightedPredFlag is set equal to 1. - luma_weight_l0_flag[ refIdxL0 ] is not equal to 0. - luma_weight_l1_flag[ refIdxL1 ] is not equal to 0. - chroma_weight_l0_flag[ refIdxL0 ] is not equal to 0. - chroma_weight_l1_flag[ refIdxL1 ] is not equal to 0. - Otherwise, weightedPredFlag is set equal to 0. - If slice_type is equal to P, weightedPredFlag is set equal to pps_weighted_pred_flag. - Otherwise (slice_type is equal to B), weightedPredFlag is set equal to pps_weighted_bipred_flag. The following applies: - If weightedPredFlag is equal to 0, the array pbSamples of the prediction samples is derived by invoking the default weighted sample prediction process as specified in clause 8.5.6.6.2 with the coding block width nCbW, the coding block height nCbH, two (nCbW)x(nCbH) arrays predSamplesL0 and predSamplesL1, the prediction list utilization flags predFlagL0 and predFlagL1, the bi-prediction weight index bcwIdx and the bit depth bitDepth as inputs - Otherwise (weightedPredFlag is equal to 1), the array pbSamples of the prediction samples is derived by invoking the weighted sample prediction process as specified in clause 8.5.6.6.3 with the coding block width nCbW, the coding block height nCbH, two (nCbW)x(nCbH) arrays predSamplesL0 and predSamplesL1, the prediction list utilization flags predFlagL0 and predFlagL1, the reference indices refIdxL0 and refIdxL1, the colour component index cIdx and the bit depth bitDepth as inputs.

Proposal 3: Change the priority of the BCW index

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| …… |  |
| if( sps_bcw_enabled_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && luma_weight_l0_flag[ ref_idx_l0 [ x0 ][ y0 ] ] = = 0 && luma_weight_l1_flag[ ref_idx_l1 [ x0 ][ y0 ] ] = = 0 && chroma_weight_l0_flag[ ref_idx_l0 [ x0 ][ y0 ] ] = = 0 && chroma_weight_l1_flag[ ref_idx_l1 [ x0 ][ y0 ] ] = = 0 && !(slice_type == P && pps_weighted_pred_flag ) && !(slice_type == B && pps_weighted_bipred_flag ) && cbWidth * cbHeight >= 256 ) |  |
| bcw_idx[ x0 ][ y0 ] | ae(v) |
| …… |  |
| } |  |

Also, in the previous meeting, PROF[1] for affine prediction block was adopted to the VTM6.0, and the PROF can be applied when BCW or WP tool is turned on. But, according to the Ticket #459, the behavior between BCW and WP is performed same as Figure 1, which is contrary operating to the philosophy of the BCW. Therefore, the behavior between BCW and WP in PROF process should be aligned to proposed methods.

Conclusion

In this contribution, the methods to fix the behavior between BCW and SW are proposed. The experimental results reportedly show no changes in coding performance and run-time compared to VTM6.0 as anchor in CTC [2]. It is recommended to consider this method in the next VTM.

Reference

- J. Luo, et al., “CE4-2.1: Prediction refinement with optical flow for affine mode”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-O0070, 15th Meeting: Gothenburg, SE, 3–12 July. 2019.

- Frank Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).