| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0297-v2 |
| --- | --- |

| Title: | Non-CE3: On CCLM neighboring sample |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaru Ikeda Yoichi Yagasaki Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81 50 3750 2782 Masaru.Ikeda@sony.com Yoichi.Yagasaki@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution document proposes two aspects on CCLM neighboring sample derivation. The first one is a simplification to harmonize neighboring sample derivation and four sample selection, and the other one is for coding efficiency improvement by increasing sample points to compensate CCLM coding drops between VTM5 and VTM6.

The proposed method 1 reportedly shows 0.01% / 0.06% / 0.06% at AI and 0.01% / 0.10% / 0.10% at RA for Y/U/V over VTM-6.0, respectively. The proposed method 2 on top of the method 1 reported shows

-0.10% / -0.56% / -0.54% at AI and -0.09% / -0.53% / -0.58% at RA for Y/U/V over VTM-6.0, respectively.

Introduction

Neighboring sample derivation

In CCLM, the number of available neighboring chroma samples is derived on the top & top-right and the left & left-below for INTRA_T_CCLM and INTRA_L_CCLM, respectively, while it is done on the top and left in INTRA_LT_CCLM. The flowchart for INTRA_T_CCLM is shown in Figure 1. In the derivation, the number of reference samples on the top-right (numTopRight) is counted for x = nTbW..2 * nTbW - 1 until availTR is equal to FALSE or x is equal to 2 * nTbW - 1. Then the number of reference samples on the top & top-right is derived by adding the smaller one between numTopRight and the transform block height (nTbH) to the transform block width (nTbW).

Figure 1: Flowchart of top-right sample extension for LM-T

An example of transform block size 8x4 and numTopRight = 2 is shown in Figure 2. The startPosN and pickStepN are derived as 1 (=10 >> 3) and 2 (=10>>2), respectively, from following formulations. Thus, available neighboring samples are, in this example, highlighted by light blue and picked samples are marked as X in Figure 2.

    - numIs4N = ( ( availT  &&  availL  &&  predModeIntra  = =  INTRA_LT_CCLM )  ?  0  :  1)

    - startPosN = numSampN >> (2 + numIs4N)

    - pickStepN = Max( 1, numSampN  >>  ( 1 + numIs4N ) )

Figure 2: An example of transform block size 8 x 4 and numTopRight=2

In this case, no sample are picked from top-right neighboring samples although the reference sample range is extended toward top-right. The similar case can be also seen in (nTbW, numTopRight) = (4, 2), (16, 2) and (32, 2) for INTRA_T_CCLM and in (nTbH, numBottomLeft) = (4, 2), (8, 2), (16, 2) and (32, 2) for INTRA_L_CCLM, respectively. It doesn’t seem that the reference sample extension towards top-right or left-below and four sample selection are  harmonized well.

CCLM performance drop between VTM5 and VTM6

It is reported that the coding performance on CCLM in VTM6 dropped by 1.60% for Y, 14.79% for U and 15.90% for V and 0.94% for Y, 11.95% for U and 13.87% for V at AI and RA, respectively, over previous VTM version. Therefore, some simple modification for coding efficiency improvements are preferable to compensate the losses.

|  |  |  | VTM RA |  |  |
| --- | --- | --- | --- | --- | --- |
| Tool | Abbreviation | VTM3 | VTM4 | VTM5 | VTM6 |
| Cross-component linear model | CCLM | 4.09% | 4.20% | 4.00% | 3.33% |

- Proposal

Proposed method 1 for neighboring sample derivation

It is proposed that the top-right/left-below extension is simplified to improve the harmonization between top-right/left-below extension and four sample selection. The flowchart of proposed method for INTRA_T_CCLM is shown in Figure 3. The modifications are yellow highlighted.

Figure 3: Flowchart of proposed method for LM-T

In this proposal, the number of reference samples on the top and top-right is twice as many as transform block width (nTbW) if the number of available neighboring chroma samples on the top-right (numTopRight) is equal to nTbW. Otherwise the number of reference sample is equal to nTbW. It is also the same for INTRA_L_CCLM.

Proposed method 2 for coding efficiency improvement

It is proposed that the sampling points for CCLM parameter derivation are increased from four to eight for larger block size cases together with modified beta which is derived using four/eight averaged samples.[1]

Results

Proposed methods were implemented on top of VTM-6.0 and tested based on CE3 test conditions [2]. The results are presented for AI and RA.

Proposed method 1

The coding results are shown in Table 1 and 2 together with no top-right/left-below extensions which mean top and left samples only are used for INTRA_T_CCLM and INTRA_L_CCLM, respectively. The coding performance for AI reportedly shows 0.01% / 0.06% / 0.06% for Y/U/V while, for no top-right/left-below extensions, 0.03% / 0.38% / 0.39% for Y/U/V, respectively. The coding performance for RA reportedly shows 0.01% / 0.06% / 0.06% for Y/U/V while, for no top-right/left-below extensions, 0.02% / 0.38% / 0.30% for Y/U/V, respectively. Those are minor compared with no top-right/left-below extensions (top or left neighboring samples only).

Table 1: BD-Rate of proposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.13% | 0.10% | #DIV/0! | #DIV/0! |
| Class A2 | 0.01% | 0.11% | 0.01% | 100% | #DIV/0! |
| Class B | 0.02% | 0.04% | 0.06% | 100% | #DIV/0! |
| Class C | 0.03% | -0.03% | 0.08% | 99% | #DIV/0! |
| Class E | 0.00% | 0.07% | 0.06% | 99% | #DIV/0! |
| Overall | 0.01% | 0.06% | 0.06% | #DIV/0! | #DIV/0! |
| Class D | 0.01% | 0.08% | 0.01% | 101% | #DIV/0! |
| Class F (optional) | -0.02% | 0.05% | 0.15% | 100% | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.10% | 100% | #DIV/0! |
| Class A2 | 0.03% | 0.09% | 0.08% | 100% | #DIV/0! |
| Class B | -0.01% | 0.16% | 0.05% | 100% | #DIV/0! |
| Class C | 0.02% | 0.11% | 0.18% | 100% | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.10% | 0.10% | 100% | #DIV/0! |
| Class D | 0.01% | -0.10% | -0.19% | 100% | #DIV/0! |
| Class F (optional) | -0.01% | 0.10% | 0.17% | 101% | #DIV/0! |

Table 2: BD-Rate of no top-right/left-below extensions (top/left only)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.74% | 0.66% | #DIV/0! | #DIV/0! |
| Class A2 | 0.03% | 0.46% | 0.21% | #DIV/0! | #DIV/0! |
| Class B | 0.02% | 0.40% | 0.49% | #DIV/0! | #DIV/0! |
| Class C | 0.03% | 0.21% | 0.35% | #DIV/0! | #DIV/0! |
| Class E | 0.01% | 0.17% | 0.19% | #DIV/0! | #DIV/0! |
| Overall | 0.03% | 0.38% | 0.39% | #DIV/0! | #DIV/0! |
| Class D | 0.02% | 0.28% | 0.14% | #DIV/0! | #DIV/0! |
| Class F (optional) | 0.03% | 0.26% | 0.30% | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.56% | 0.65% | #DIV/0! | #DIV/0! |
| Class A2 | 0.02% | 0.31% | 0.13% | #DIV/0! | #DIV/0! |
| Class B | 0.01% | 0.47% | 0.24% | #DIV/0! | #DIV/0! |
| Class C | -0.01% | 0.18% | 0.24% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.38% | 0.30% | #DIV/0! | #DIV/0! |
| Class D | -0.03% | 0.03% | 0.05% | #DIV/0! | #DIV/0! |
| Class F (optional) | 0.00% | 0.19% | 0.25% | #DIV/0! | #DIV/0! |

Proposed method 2

The results are presented in Table 3, 4 and 5 for the block length threshold = 8, 16 and 32, respectively. The sample points are increased if the number of actual neighboring top or left samples is equal to or larger than the threshold.

Table 3: BD-Rate in case of threshold = 8

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -1.16% | -0.75% | #DIV/0! | #DIV/0! |
| Class A2 | -0.13% | -0.68% | -0.37% | #DIV/0! | #DIV/0! |
| Class B | -0.07% | -0.30% | -0.63% | #DIV/0! | #DIV/0! |
| Class C | -0.07% | -0.48% | -0.37% | #DIV/0! | #DIV/0! |
| Class E | -0.03% | -0.38% | -0.56% | #DIV/0! | #DIV/0! |
| Overall | -0.10% | -0.56% | -0.54% | #DIV/0! | #DIV/0! |
| Class D | -0.06% | -0.42% | -0.47% | #DIV/0! | #DIV/0! |
| Class F (optional) | -0.08% | -0.48% | -0.56% | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | -1.34% | -0.87% | #DIV/0! | #DIV/0! |
| Class A2 | -0.04% | -0.63% | -0.30% | #DIV/0! | #DIV/0! |
| Class B | -0.08% | -0.14% | -0.78% | #DIV/0! | #DIV/0! |
| Class C | -0.03% | -0.33% | -0.32% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.53% | -0.58% | #DIV/0! | #DIV/0! |
| Class D | -0.09% | -0.67% | -0.26% | #DIV/0! | #DIV/0! |
| Class F (optional) | -0.10% | -0.34% | -0.34% | #DIV/0! | #DIV/0! |

Table 4: BD-Rate in case of threshold = 16

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -1.04% | -0.49% | #NUM! | #DIV/0! |
| Class A2 | -0.10% | -0.44% | -0.19% | #NUM! | #DIV/0! |
| Class B | -0.06% | -0.22% | -0.44% | #NUM! | #DIV/0! |
| Class C | -0.03% | -0.26% | -0.19% | #NUM! | #DIV/0! |
| Class E | -0.04% | -0.27% | -0.44% | #NUM! | #DIV/0! |
| Overall | -0.08% | -0.41% | -0.35% | #NUM! | #DIV/0! |
| Class D | -0.05% | -0.17% | -0.17% | #NUM! | #DIV/0! |
| Class F | -0.05% | -0.23% | -0.33% | #NUM! | #DIV/0! |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -1.30% | -0.75% | #NUM! | #DIV/0! |
| Class A2 | -0.01% | -0.28% | -0.15% | #NUM! | #DIV/0! |
| Class B | -0.04% | -0.06% | -0.57% | #NUM! | #DIV/0! |
| Class C | -0.03% | -0.09% | -0.11% | #NUM! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -0.36% | -0.40% | #NUM! | #DIV/0! |
| Class D | -0.02% | -0.37% | -0.34% | #NUM! | #DIV/0! |
| Class F | -0.03% | -0.22% | -0.13% | #NUM! | #DIV/0! |

Table 5: BD-Rate in case of threshold = 32

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.75% | -0.39% | #NUM! | #DIV/0! |
| Class A2 | -0.05% | -0.06% | -0.06% | #NUM! | #DIV/0! |
| Class B | -0.03% | 0.00% | -0.23% | #NUM! | #DIV/0! |
| Class C | -0.01% | -0.06% | 0.16% | #NUM! | #DIV/0! |
| Class E | -0.03% | -0.11% | -0.17% | #NUM! | #DIV/0! |
| Overall | -0.05% | -0.17% | -0.13% | #NUM! | #DIV/0! |
| Class D | -0.02% | 0.04% | 0.04% | #NUM! | #DIV/0! |
| Class F | 0.00% | -0.07% | -0.01% | #NUM! | #DIV/0! |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.93% | -0.63% | #NUM! | #DIV/0! |
| Class A2 | 0.00% | -0.15% | -0.01% | #NUM! | #DIV/0! |
| Class B | -0.01% | 0.02% | -0.28% | #NUM! | #DIV/0! |
| Class C | 0.00% | -0.01% | -0.11% | #NUM! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.21% | -0.25% | #NUM! | #DIV/0! |
| Class D | -0.06% | -0.44% | 0.09% | #NUM! | #DIV/0! |
| Class F | 0.00% | -0.05% | 0.16% | #NUM! | #DIV/0! |

Conclusion

This contribution document proposes two aspects on CCLM neighboring sample derivation. The first one is a simplification to harmonize neighboring sample derivation and four sample selection, and the other one is for coding efficiency improvement by increasing sample points to compensate CCLM coding drops between VTM5 and VTM6. It shows reasonable coding performance for both cases. We recommend considering that the modification is included into next version of test model.

Text changes

The text changes for the proposed method 1 and 2 are light blue and light green highlighted, respectively, below.

- 8.4.5.2.13 Specification of INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM intra prediction mode

Inputs to this process are:

- the intra prediction mode predModeIntra,

- a sample location ( xTbC, yTbC ) of the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable cIdx specifying the colour component of the current block,

- chroma neighbouring samples p[ x ][ y ], with x = −1, y = 0..2 * nTbH − 1 and x = 0.. 2 * nTbW − 1, y = − 1.

Output of this process are predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The current luma location ( xTbY, yTbY ) is derived as follows:

( xTbY, yTbY )  =  ( xTbC << ( SubWidthC − 1 ), yTbC << ( SubHeightC − 1 ) )	(8-165)

The variables availL, availT and availTL are derived as follows:

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC, yTbC ), the neighbouring chroma location ( xTbC − 1, yTbC ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availL.

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC, yTbC ), the neighbouring chroma location ( xTbC, yTbC − 1 ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availT.

- The variable availTL is derived as follows:

availTL  =  availL  &&  availT		(8-166)

- The number of available top-right neighbouring chroma samples numTopRight is derived as follows:

- The variable numTopRight is set equal to 0 and availTR is set equal to TRUE.

- When predModeIntra is equal to INTRA_T_CCLM, the following applies for x = nTbW..2 * nTbW − 1 until availTR is equal to FALSE or x is equal to 2 * nTbW − 1:

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC , yTbC ) the neighbouring chroma location ( xTbC + x, yTbC − 1 ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availableTR

- When availableTR is equal to TRUE, numTopRight is incremented by one.

- numTopRight = ( numTopRight  = =  nTbW )  ?  nTbW  :  0

- The number of available left-below neighbouring chroma samples numLeftBelow is derived as follows:

- The variable numLeftBelow is set equal to 0 and availLB is set equal to TRUE.

- When predModeIntra is equal to INTRA_L_CCLM, the following applies for y = nTbH..2 * nTbH − 1 until availLB is equal to FALSE or y is equal to 2 * nTbH − 1:

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC , yTbC ), the neighbouring chroma location ( xTbC − 1, yTbC + y ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availableLB

- When availableLB is equal to TRUE, numLeftBelow is incremented by one.

- numLeftBelow = ( numLeftBelow  = =  nTbH )  ?  nTbH  :  0

The number of available neighbouring chroma samples on the top and top-right numSampT and the number of available neighbouring chroma samples on the left and left-below numSampL are derived as follows:

- If predModeIntra is equal to INTRA_LT_CCLM, the following applies:

numSampT = availT  ?  nTbW  :  0		(8-167)

numSampL = availL  ?  nTbH  :  0		(8-168)

- Otherwise, the following applies:

numSampT = ( availT  &&  predModeIntra  = =  INTRA_T_CCLM )  ?  ( nTbW + numTopRight )  :  0		(8-169)

numSampL = ( availL  &&  predModeIntra = =  INTRA_L_CCLM )  ?  ( nTbH + numLeftBelow )  :  0		(8-170)

The variable bCTUboundary is derived as follows:

bCTUboundary = ( yTbC & ( 1 << ( CtbLog2SizeY − 1 ) − 1 )  = =  0 )  ?  TRUE  :  FALSE. 	(8-171)

The variable cntN and array pickPosN with N being replaced by L and T, are derived as follows:

- The variable numIs4N is derived as follows:

numIs4N = ( ( availT  &&  availL  &&  predModeIntra  = =  INTRA_LT_CCLM )  ?  0  :  1) 	(8-172)

- The variable numExtNeighSamp is derived as follows:

numExtNeighSamp = predModeInra == ( (INTRA_T_CCLM && numSampT >= 8) || (INTRA_L_CCLM && numSampL >= 8) ) ? 1 : 0

- The variable startPosN is set equal to numSampN  >>  ( 2 + numIs4N + numExtNeighSamp ).

- The variable pickStepN is set equal to Max( 1, numSampN  >>  ( 1 + numIs4N + numExtNeighSamp ) ).

- If availN is equal to TRUE and predModeIntra is equal to INTRA_LT_CCLM or INTRA_N_CCLM, the following assignments are made:

    - cntN is set equal to Min( numSampN, ( 1 + numIs4N + numExtNeighSamp ) << 1 ).

    - pickPosN[ pos ] is set equal to (startPosN + pos * pickStepN), with pos = 0.. cntN − 1.

- Otherwise, cntN is set equal to 0.

The prediction samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If both numSampL and numSampT are equal to 0, the following applies:

predSamples[ x ][ y ] = 1 << ( BitDepthC − 1 )		(8-173)

- Otherwise, the following ordered steps apply:

  - The collocated luma samples pY[ x ][ y ] with x = 0..nTbW * SubWidthC − 1, y= 0..nTbH * SubHeightC − 1 are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY + x, yTbY + y ).

  - The neighbouring luma samples pY[ x ][ y ] are derived as follows:

    - When numSampL is greater than 0, the neighbouring left luma samples pY[ x ][ y ] with x = −1..−3, y = 0..SubHeightC * numSampL − 1, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY + x , yTbY +y ).

    - When numSampT is greater than 0, the neighbouring top luma samples pY[ x ][ y ] with x = 0..SubWidthC * numSampT − 1, y = −1, −2, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY+ x, yTbY + y ).

    - When availTL is equal to TRUE, the neighbouring top-left luma samples pY[ x ][ y ] with x = −1, y = −1, −2, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY+ x, yTbY + y ).

  - The down-sampled collocated luma samples pDsY[ x ][ y ] with x = 0..nTbW − 1,  y = 0..nTbH − 1 are derived as follows:

    - If both SubWidthC and SubHeightC are equal to 1, the following applies:

      - pDsY[ x ][ y ] with x = 1..nTbW − 1, y = 1..nTbH − 1 is derived as follows:

pDstY[ x ][ y ] = pY[ x ][ y ]				(8-174)

    - Otherwise, the following applies:

      - The one-dimensional filter coefficients array F1 and F2, and the 2-dimensional filter coefficients arrays F3 and F4 are specified as follows.

F1[ i ] with i = 0..2				(8-175)

F2[ 0 ] = 1, F2[ 1 ] = 2, F2[ 2 ] = 1	(8-176)

F3[ i ][ j ] = F4[ i ][ j ] = 0, with i = 0..2, j = 0..2	(8-177)

        - If both SubWidthC and SubHeightC are equal to 2, the following applies:

F1[ 0 ] = 1, F1[ 1 ] = 1				(8-178)

F3[ 0 ][ 1 ] = 1, F3[ 1 ][ 1 ] = 4, F2[ 2 ][ 1 ] = 1, F3[ 1 ][ 0 ] = 1, F3[ 1 ][ 2 ] = 1	(8-179)

F4[ 0 ][ 1 ]  = 1, F4[ 1 ][ 1 ] = 2, F4[ 2 ][ 1 ] = 1	(8-180)

F4[ 0 ][ 2 ] = 1,  F4[ 1 ][ 2 ] = 2, F4[ 2 ][ 2 ] = 1	(8-181)

        - Otherwise, the following applies:

F1[ 0 ] = 2, F1[ 1 ] = 0				(8-182)

F3[ 1 ][ 1 ] = 8					(8-183)

F4[ 0 ][ 1 ] = 2, F4[ 1 ][ 1 ] = 4, F4[ 2 ][ 1 ] = 2,	(8-184)

      - If sps_cclm_colocated_chroma_flag is equal to 1, the following applies:

        - pDsY[ x ][ y ] with x = 1..nTbW − 1, y = 1..nTbH − 1 is derived as follows:

pDsY[ x ][ y ] = ( F3[ 1 ][ 0 ] * pY[ SubWidthC * x ][ SubHeightC * y − 1 ] +
		F3[ 0 ][ 1 ] * pY[ SubWidthC * x − 1 ][ SubHeightC * y ] +
		F3[ 1 ][ 1 ] * pY[ SubWidthC * x ][ SubHeightC * y ] +	(8-185)
		F3[ 2 ][ 1 ] * pY[ SubWidthC * x + 1 ][ SubHeightC * y ] +
		F3[ 1 ][ 2 ] * pY[ SubWidthC * x ][ SubHeightC * y + 1 ] + 4 ) >> 3

        - If availL is equal to TRUE, pDsY[ 0 ][ y ] with y = 1..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( F3[ 1 ][ 0 ] * pY[ 0 ][ SubHeightC * y − 1 ] +
		F3[ 0 ][ 1 ] * pY[ − 1 ][ SubHeightC * y ] +
		F3[ 1 ][ 1 ] * pY[ 0 ][ SubHeightC * y ] +	(8-186)
		F3[ 2 ][ 1 ] * pY[ 1 ][ SubHeightC * y ] +
		F3[ 1 ][ 2 ] * pY[ 0 ][ SubHeightC * y + 1 ] + 4 ) >> 3

        - Otherwise (availL is equal to FALSE), pDsY[ 0 ][ y ] with y = 1..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( F2[ 0 ] * pY[ 0 ][ SubHeightC * y − 1 ] +
		F2[ 1 ] * pY[ 0 ][ SubHeightC * y ] +	(8-187)
		F2[ 2 ] * pY[ 0 ][ SubHeightC * y + 1 ] + 2 ) >> 2

        - If availT is equal to TRUE, pDsY[ x ][ 0 ] with x = 1..nTbW − 1 is derived as follows:

pDsY[ x ][ 0 ] = ( F3[ 1 ][ 0 ] * pY[ SubWidthC * x ][ − 1 ] +
		F3[ 0 ][ 1 ] * pY[ SubWidthC * x − 1 ][ 0 ] +
		F3[ 1 ][ 1 ] * pY[ SubWidthC * x ][ 0 ] +	(8-188)
		F3[ 2 ][ 1 ] * pY[ SubWidthC * x + 1 ][ 0 ] +
		F3[ 1 ][ 2 ] * pY[ SubWidthC * x ][ 1 ] + 4 ) >> 3

        - Otherwise (availT is equal to FALSE), pDsY[ x ][ 0 ] with x = 1..nTbW − 1 is derived:

pDsY[ x ][ 0 ] = ( F2[ 0 ] * pY[ SubWidthC * x − 1 ][ 0 ] +
		F2[ 1 ] * pY[ SubWidthC * x ][ 0 ] +	(8-189)
		F2[ 2 ] * pY[ SubWidthC * x + 1 ][ 0 ] + 2 ) >> 2

        - If availL is equal to TRUE and availT is equal to TRUE, pDsY[ 0 ][ 0 ] is derived as follows:

pDsY[ 0 ][ 0 ] = ( F3[ 1 ][ 0 ] * pY[ 0 ][ − 1 ] +
		F3[ 0 ][ 1 ] * pY[ − 1 ][ 0 ] +
		F3[ 1 ][ 1 ] * pY[ 0 ][ 0 ] +	(8-190)
		F3[ 2 ][ 1 ] * pY[ 1 ][ 0 ] +
		F3[ 1 ][ 2 ] * pY[ 0 ][ 1 ] + 4 ) >> 3

        - Otherwise, if availL is equal to TRUE and availT is equal to FALSE, pDsY[ 0 ][ 0 ] is derived as follows:

pDsY[ 0 ][ 0 ] = ( F2[ 0 ] * pY[ − 1 ][ 0 ] + F2[ 1 ] * pY[ 0 ][ 0 ] +
		F2[ 2 ] * pY[ 1 ][ 0 ] + 2 ) >> 2	(8-191)

        - Otherwise, if availL is equal to FALSE and availT is equal to TRUE, pDsY[ 0 ][ 0 ] is derived as follows:

pDsY[ 0 ][ 0 ] = ( F2[ 0 ] * pY[ 0 ][ − 1 ] + F2[ 1 ] * pY[ 0 ][ 0 ] +
		F2[ 2 ] * pY[ 0 ][ 1 ] + 2 ) >> 2	(8-192)

        - Otherwise (availL is equal to FALSE and availT is equal to FALSE), pDsY[ 0 ][ 0 ] is derived as follows:

pDsY[ 0 ][ 0 ] = pY[ 0 ][ 0 ]				(8-193)

      - Otherwise (sps_cclm_colocated_chroma_flag is equal to 0), the following applies:

        - pDsY[ x ][ y ] with x = 1..nTbW − 1, y = 0..nTbH − 1 is derived as follows:

pDsY[ x ][ y ] = ( F4[ 0 ][ 1 ] * pY[ SubWidthC * x − 1 ][ SubHeightC * y ] +
		F4[ 0 ][ 2 ] * pY[ SubWidthC * x − 1 ][ SubHeightC * y + 1 ] +
		F4[ 1 ][ 1 ] * pY[ SubWidthC * x ][ SubHeightC * y ] +	(8-194)
		F4[ 1 ][ 2 ] * pY[ SubWidthC * x ][ SubHeightC * y + 1] +
		F4[ 2 ][ 1 ] * pY[ SubWidthC * x + 1 ][ SubHeightC * y ] +
		F4[ 2 ][ 2 ] * pY[ SubWidthC * x + 1][ SubHeightC * y + 1 ] + 4 ) >> 3

        - If availL is equal to TRUE, pDsY[ 0 ][ y ] with y = 0..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( F4[ 0 ][ 1 ] * pY[ − 1 ][ SubHeightC * y ] +
		F4[ 0 ][ 2 ] * pY[ − 1 ][ SubHeightC * y + 1 ] +
		F4[ 1 ][ 1 ] * pY[ 0 ][ SubHeightC * y ] +	(8-195)
		F4[ 1 ][ 2 ] * pY[ 0 ][ SubHeightC * y + 1] +
		F4[ 2 ][ 1 ] * pY[ 1 ][ SubHeightC * y ] +
		F4[ 2 ][ 2 ] * pY[ 1 ][ SubHeightC * y + 1 ] + 4 ) >> 3

        - Otherwise (availL is equal to FALSE), pDsY[ 0 ][ y ] with y = 0..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( F1[ 0 ] * pY[ 0 ][ SubHeightC * y ]  +
		F1[ 1 ] * pY[ 0 ][ SubHeightC * y + 1] + 1 ) >> 1	(8-196)

  - When numSampL is greater than 0, the selected neighbouring left chroma samples pSelC[ idx ] are set equal to p[ −1 ][ pickPosL[ idx ] ] with idx = 0..cntL − 1, and the selected down-sampled neighbouring left luma samples pSelDsY[ idx ] with idx = 0..cntL − 1 are derived as follows:

    - The variable y is set equal to pickPosL[ idx ].

    - If both SubWidthC and SubHeightC are equal to 1, the following applies:

pSelDsY[ idx ] = pY[ − 1][ y ]			(8-197)

    - Otherwise the following applies:

      - If sps_cclm_colocated_chroma_flag is equal to 1, the following applies:

        - If y is greater than 0 or availTL is equal to TRUE, pSelDsY[ idx ]  is derived as follows:

pSelDsY[ idx ] = ( F3[ 1 ][ 0 ] * pY[ − SubWidthC ][ SubHeightC * y − 1 ] +
		F3[ 0 ][ 1 ] * pY[ −1 − SubWidthC ][ SubHeightC * y ] +
		F3[ 1 ][ 1 ] * pY[ −SubWidthC ][ SubHeightC * y ]  + 	(8-198)
		F3[ 2 ][ 1 ] * pY[ 1 − SubWidthC ][ SubHeightC * y ] +
		F3[ 1 ][ 2 ] * pY[ −SubWidthC ][ SubHeightC * y + 1 ] + 4 ) >> 3

        - Otherwise (y is equal to 0), pSelDsY[ idx ]  is derived as follows:

pSelDsY[ idx ] = ( F2[ 0 ] * pY[ −1 − SubWidthC ][ 0 ] +
		F2[ 1 ] * pY[ −SubWidthC ][ 0 ] +	(8-199)
		F2[ 2 ] * pY[ 1 − SubWidthC ][ 0] + 2 ) >> 2

      - Otherwise (sps_cclm_colocated_chroma_flag is equal to 0), the following applies:

pSelDsY[ idx ] = ( F4[ 0 ][ 1 ] * pY[ −1 − SubWidthC ][ SubHeightC * y ] +
		F4[ 0 ][ 2 ] * pY[ −1 − SubWidthC ][ SubHeightC * y + 1 ] +
		F4[ 1 ][ 1 ] * pY[ −SubWidthC ][ SubHeightC * y ] +	(8-200)
		F4[ 1 ][ 2 ] * pY[ −SubWidthC ][ SubHeightC * y + 1] +
		F4[ 2 ][ 1 ] * pY[ 1 − SubWidthC ][ SubHeightC * y ] +
		F4[ 2 ][ 2 ] * pY[ 1 − SubWidthC][ SubHeightC * y + 1 ] + 4 ) >> 3

  - When numSampT is greater than 0, the selected neighbouring top chroma samples pSelC[ idx ] are set equal to p[ pickPosT[ idx − cntL ] ][ -1 ] with  idx = cntL..cntL + cntT − 1, and the down-sampled neighbouring top luma samples pSelDsY[ idx ] with idx = 0..cntL + cntT − 1 are specified as follows:

    - The variable x is set equal to pickPosT[ idx − cntL ].

    - If both SubWidthC and SubHeightC are equal to 1, the following applies:

pSelDsY[ idx ] = pY[ x ][ − 1]			(8-201)

    - Otherwise, the following applies:

      - If sps_cclm_colocated_chroma_flag is equal to 1, the following applies:

        - If x is greater than 0, the following applies:

          - If bCTUboundary is equal to FALSE, the following applies:

pSelDsY[ idx ] = ( F3[ 1 ][ 0 ] * pY[ SubWidthC * x ][ − 1 − SubHeightC ] +
		F3[ 0 ][ 1 ] * pY[ SubWidthC * x − 1 ][ −SubHeightC ] +
		F3[ 1 ][ 1 ] * pY[ SubWidthC * x ][ −SubHeightC] +	(8-202)
		F3[ 2 ][ 1 ] * pY[ SubWidthC * x + 1 ][ −SubHeightC] +
		F3[ 1 ][ 2 ] * pY[ SubWidthC * x ][ 1 − SubHeightC ] + 4 ) >> 3

          - Otherwise (bCTUboundary is equal to TRUE), the following applies:

pSelDsY[ idx ] = ( F2[ 0 ] * pY[ SubWidthC * x − 1 ][ −1 ] +
		F2[ 1 ] * pY[ SubWidthC * x ][ −1 ]  +	(8-203)
		F2[ 2 ] * pY[ SubWidthC * x + 1 ][ −1 ] + 2 ) >> 2

        - Otherwise (x is equal to 0), the following applies:

          - If availTL is equal to TRUE and bCTUboundary is equal to FALSE, the following applies:

pSelDsY[ idx ] = ( F3[ 1 ][ 0 ] * pY[ −1 ][ − 1 − SubHeightC ] +
		F3[ 0 ][ 1 ] * pY[ −1 ][ −SubHeightC ] +
		F3[ 1 ][ 1 ] * pY[ 0 ][ −SubHeightC ] +	(8-204)
		F3[ 2 ][ 1 ] * pY[ 1 ][ −SubHeightC ] +
		F3[ 1 ][ 2 ] * pY[ −1 ][ 1 − SubHeightC ] + 4 ) >> 3

          - Otherwise, if availTL is equal to TRUE and bCTUboundary is equal to TRUE, the following applies:

pSelDsY[ idx ] = ( F2[ 0 ] * pY[ −1 ][ −1 ] + F2[ 1 ] * pY[ 0 ][ −1 ]  +
		F2[ 2 ] * pY[ 1 ][ −1 ] + 2 ) >> 2	(8-205)

          - Otherwise, if availTL is equal to FALSE and bCTUboundary is equal to FALSE, the following applies:

pSelDsY[ idx ] = ( F2[ 0 ] * pY[ 0 ][ −1 ] + F2[ 1 ] * pY[ 0 ][ −2 ] +
		F2[ 2 ] * pY[ 0 ][ −1 ] + 2 ) >> 2	(8-206)

          - Otherwise (availTL is equal to FALSE and bCTUboundary is equal to TRUE), the following applies:

pSelDsY[ idx ] = pY[ 0 ][ −1 ]		(8-207)

      - Otherwise (sps_cclm_colocated_chroma_flag is equal to 0), the following applies:

        - If x is greater than 0, the following applies:

          - If bCTUboundary is equal to FALSE, the following applies:

pSelDsY[ idx ] = ( F4[ 0 ][ 1 ] * pY[ SubWidthC x − 1 ][ −2 ] +
		F4[ 0 ][ 2 ] * pY[ SubWidthC * x − 1 ][ −1 ] +
		F4[ 1 ][ 1 ] * pY[ SubWidthC * x ][ −2 ] +	(8-208)
		F4[ 1 ][ 2 ] * pY[ SubWidthC * x ][ −1] +
		F4[ 2 ][ 1 ] * pY[ SubWidthC * x + 1 ][ −2 ] +
		F4[ 2 ][ 2 ] * pY[ SubWidthC * x + 1 ][ −1 ] + 4 ) >> 3

          - Otherwise (bCTUboundary is equal to TRUE), the following applies:

pSelDsY[ idx ] = ( F2[ 0 ] * pY[ SubWidthC * x − 1 ][ −1 ] +
		F2[ 1 ] * pY[ SubWidthC * x ][ −1 ] +	(8-209)
		F2[ 2 ] * pY[ SubWidthC * x + 1][ −1 ] + 2 ) >> 2

        - Otherwise (x is equal to 0), the following applies:

          - If availTL is equal to TRUE and bCTUboundary is equal to FALSE, the following applies:

pSelDsY[ idx ] = ( F4[ 0 ][ 1 ] * pY[ −1 ][ −2 ] + F4[ 0 ][ 2 ] * pY[ −1 ][ −1 ] +
		F4[ 1 ][ 1 ] * pY[ 0 ][ −2 ] + F4[ 1 ][ 2 ] * pY[ 0 ][ −1 ] +	(8-210)		F4[ 2 ][ 1 ] * pY[ 1 ][ −2 ] + F4[ 2 ][ 2 ] * pY[ 1 ][ −1 ] + 4 ) >> 3

          - Otherwise, if availTL is equal to TRUE and bCTUboundary is equal to TRUE, the following applies:

pSelDsY[ idx ] = ( F2[ 0 ] * pY[ −1 ][ −1 ] + F2[ 1 ] * pY[ 0 ][ −1 ] +
		F2[ 2 ] * pY[ 1 ][ − 1 ] + 2 ) >> 2	(8-211)

          - Otherwise, if availTL is equal to FALSE and bCTUboundary is equal to FALSE, the following applies:

pSelDsY[ idx ] = ( F1[ 1 ] * pY[ 0 ][ −2 ] + F1[ 0 ] * pY[ 0 ][ −1 ] + 1 ) >> 1	(8-212)

          - Otherwise (availTL is equal to FALSE and bCTUboundary is equal to TRUE), the following applies:

pSelDsY[ idx ] = pY[ 0 ][ −1 ]		(8-213)

  - When cntT+ cntL is not equal to 0, the variables minY, maxY, minC and maxC are derived as follows:

    - When cntT + cntL is equal to 2, pSelComp[ 3 ] is set equal to pSelComp[ 0 ], pSelComp[ 2 ] is set equal to pSelComp[ 1 ], pSelComp[ 0 ] is set equal to pSelComp[ 1 ], and pSelComp[ 1 ] is set equal to pSelComp[ 3 ], with Comp being replaced by DsY and C.

    - The arrays Idx are derived as follows at first:

Idx[ 0 ] = 0		(8-194)

Idx[ 1 ] = 2		(8-195)

Idx[ 0 ] = 1		(8-196)

Idx[ 1 ] = 3		(8-197)

If the number of the arrarys Idx N is larger than 4, the remainig array Idx are derived as follows :

Idx[ i ] = i with i=4,..N-1     Where N = cntL+cntT

    - Then, the arrays Idx are swapped as follows:

      - If the number of the arrays Idx N is equal to 4, the arrays Idx are swapped as following cclm_sort4 process.

        - When pSelDsY[ Idx[ 0 ] ] is greater than pSelDsY[ Idx[ 1 ] ], Idx[ 0 ] and Idx[ 1 ] are swapped as follows::

( Idx[ 0 ], Idx[ 1 ] ) = Swap( Idx[ 0 ], Idx[ 1 ] )	(8-198)

        - When pSelDsY[ Idx[ 2 ] ] is greater than pSelDsY[ Idx[ 3 ] ], Idx[ 2 ] and Idx[ 3 ] are swapped as follows:

( Idx[ 2 ], Idx[ 3 ] ) = Swap( Idx[ 2 ], Idx[ 3 ] )	(8-199)

        - When pSelDsY[ Idx[ 1 ] ] is greater than pSelDsY[ Idx[ 2 ] ], Idx[ 1 ] and Idx[ 2 ] are swapped as follows:

( Idx[ 1 ], Idx[ 2 ] ) = Swap(Idx[ 1 ], Idx[ 2 ] )	(8-200)

        - When pSelDsY[ Idx[ 0 ] ] is greater than pSelDsY[ Idx[ 3 ] ], Idx[ 0 ] and Idx[ 3 ] are swapped as follows:

( Idx[ 0 ], Idx[ 3 ] ) = Swap( Idx[ 0 ], Idx[ 3 ] )	(8-201)

      - Otherwise if the number of the arrays Idx N is equal to 6, the arrays Idx are swapped as following cclm_sort6 process.

        - The following ordered steps applied for cclm_sort6:

          - cclm_sort4 process is applied with input Idx[0], Idx[1], Idx[2], Idx[3]

          - cclm_sort4 process is applied with input Idx[0], Idx[1], Idx[4], Idx[5]

          - cclm_sort4 process is applied with input Idx[2], Idx[3], Idx[4], Idx[5]

      - Otherwise if the number of the arrays Idx N is equal to 8, the arrays Idx are swapped as following cclm_sort8 process.

        - The following ordered steps applied for cclm_sort8:

          - cclm_sort4 process is applied with input Idx[0], Idx[1], Idx[2], Idx[3]

          - cclm_sort4 process is applied with input Idx[4], Idx[5], Idx[6], Idx[7]

          - cclm_sort4 process is applied with input Idx[0], Idx[1], Idx[4], Idx[5]

          - cclm_sort4 process is applied with input Idx[2], Idx[3], Idx[6], Idx[7]

    - The variables maxY, maxC, minY and minC are derived as follows:

maxY = ( pSelDsY[ Idx[ N -2 ] ] + pSelDsY[ Idx[ N -1 ] ] + 1 )  >>  1	(8-202)

maxC = ( pSelC[ Idx[ N -2 ] ] + pSelC[ Idx[ N -1 ] ] + 1 )  >>  1	(8-203)

minY = ( pSelDsY[ Idx[ 0 ] ] + pSelDsY[ Idx[ 1 ] ] + 1 )  >>  1	(8-204)

minC = ( pSelC[ Idx[ 0 ] ] + pSelC[ Idx[ 1 ] ] + 1 )  >>  1	(8-205)

  - The variables a, b, and k are derived as follows:

    - If numSampL is equal to 0, and numSampT is equal to 0, the following applies:

k = 0		(8-226)

a = 0		(8-227)

b = 1 << ( BitDepthC − 1) 		(8-228)

    - Otherwise, the following applies:

diff = maxY − minY		(8-229)

    - If diff is not equal to 0, the following applies:

diffC = maxC − minC	(8-230)

x = Floor( Log2( diff ) )	(8-231)

normDiff = ( ( diff << 4 ) >> x ) & 15	(8-232)

x += ( normDiff  !=  0 )  ?  1  :  0	(8-233)

y = Floor( Log2( Abs ( diffC ) ) ) + 1	(8-234)

a = ( diffC * ( divSigTable[ normDiff ] | 8 ) + 2y − 1 ) >> y	(8-235)

k = ( ( 3 + x − y ) < 1 )  ?  1  :  3 + x − y	(8-236)

a = ( ( 3 + x − y ) < 1 )  ?  Sign( a ) * 15  :  a	(8-237)

where divSigTable[ ] is specified as follows:

divSigTable[ ] = { 0, 7, 6, 5, 5, 4, 4, 3, 3, 2, 2, 1, 1, 1, 1, 0 }	(8-239)

The arrays b_araay are derived as follows :

b_array[i] = pSelC[i] − ( ( a * minY ) >> k )    with i = 0,...N-1

    - b = Σb_array[i]/N   with i = 0,...N-1Otherwise (diff is equal to 0), the following applies:

k = 0	(8-240)

a = 0	(8-241)

b = Σb_array[i]/N   with i = 0,...N-1	(8-242)

  - The prediction samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0.. nTbH − 1 are derived as follows:

predSamples[ x ][ y ] = Clip1C( ( ( pDsY[ x ][ y ] * a ) >> k ) + b )	(8-243)

Reference

[1] M. Ikeda, Y. Yagasaki, T. Suzuki, “CE3-related: Sampling point extension for CCLM”, JVET-O0412, July 2019

[2] G. Van der Auwera, L. Li, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding”, JVET-O02023, July 2019

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).