| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0299 |
| --- | --- |

| Title: | CE5-related: Interaction between Bilateral Filter and Cross-Component Adaptive Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Christopher Hollmann, Jacob Ström Torshamnsgatan 23 16480 Stockholm Sweden | Tel: Email: | +46107146437 {christopher.hollmann, jacob.strom} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document informs about the interactions between the Cross-Component Adaptive Loop Filter (CCALF) and the Bilateral Filter (BIF), both of which are currently tested in CE5. It is reported that the gains provided by the two tools are additive to each other and enabling one does not reduce the performance of the other.

# Introduction

At the 15th JVET meeting in Gothenburg, Sweden, in July 2019 a Core Experiment (CE) was established to investigate the performance of different loop filters. A description of the CE can be found in JVET-O2025 [1]. One of the tested loop filters was an addition to the existing Adaptive Loop Filter (ALF), which takes information from the luminance channel to filter the chroma channels. Therefore, it is referred to as Cross-Component ALF (CCALF). A more detailed description can be found in JVET-P0080 [2]. Another tested loop filter is the Bilateral Filter (BIF), which applies a filter with the goal of removing ringing artifacts in luma, and which is described in JVET-P0073 [3].

This contribution evaluates the interaction between these two filters. Two combinations were tested.

- CCALF as tested in CE5-2.1 combined with BIF as tested in CE5-3.1.1.

- CCALF with a chroma QP offset (supplemental information in CE5-2.1) combined with BIF as tested in CE5-3.1.1

# Evaluation

## Test a) CCALF with BIF

In Table 1 it is shown that the performance of CCALF is not impacted by BIF. The table uses VTM-6.0 with CCALF and BIF as the tested software and VTM-6.0 with BIF as the reference. The reference results are taken from JVET-P0073 [3]. These results can be compared with Table 3.1 from JVET-P0080 [2]. For convenience, the table can also be found in section 6.1 of this document. Detailed results can be found in the Excel sheet test_a1_VTM-6.0_BIF_CCALF.xlsm.

Table 1 Performance of CCALF in presence of BIF

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_BIF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | -3.51% | -4.15% | 107% | 97% |
| Class A2 | 0.13% | -8.68% | -8.98% | 102% | 100% |
| Class B | 0.10% | -10.53% | -8.62% | 101% | 99% |
| Class C | 0.13% | -4.15% | -3.94% | 105% | 104% |
| Class E | 0.19% | -10.22% | -3.47% | 105% | 105% |
| Overall | 0.13% | -7.58% | -6.04% | 104% | 101% |
| Class D | 0.12% | -2.79% | -3.28% | 104% | 109% |
| Class F | 0.18% | -6.62% | -6.59% | 100% | 102% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_BIF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | -5.65% | -5.43% | 103% | 100% |
| Class A2 | 0.16% | -11.65% | -15.69% | 102% | 102% |
| Class B | 0.22% | -17.59% | -13.86% | 102% | 106% |
| Class C | 0.13% | -5.03% | -4.35% | 103% | 105% |
| Class E |  |  |  |  |  |
| Overall | 0.17% | -10.66% | -10.00% | 103% | 104% |
| Class D | 0.11% | -4.54% | -4.53% | 102% | 113% |
| Class F | 0.22% | -4.80% | -4.60% | 110% | 108% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_BIF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.16% | -21.57% | -19.06% | 103% | 103% |
| Class C | 0.20% | -8.42% | -6.96% | 105% | 103% |
| Class E | 0.12% | -16.73% | -6.81% | 103% | 106% |
| Overall | 0.16% | -15.98% | -11.97% | 104% | 104% |
| Class D | 0.15% | -8.06% | -6.40% | 104% | 105% |
| Class F | 0.22% | -10.61% | -8.22% | 104% | 105% |

As we can see in Table 1, the gains compared to JVET-P0080 Table 3.1 (see Section 6.1 below) hardly change at all. As an example, for the Random Access test configuration, Table 1 above shows 0.17% (Y), -10.66% (U) and -10.00% (V), whereas JVET-P0080 Table 3.1 shows 0.16% (Y), -10.69% (U) and -10.07% (V).

Table 2 shows that the statement also is true when evaluated in the other direction, meaning that the performance of BIF is not impacted by the presence of CCALF. The reference used in Table 2 is VTM-6.0 with CCALF, taken from JVET-P0080 [2], whereas the tested software is identical to Table 1. These results can be compared with Table 4.1 from JVET-P0073 [3]. For convenience, the table can also be found in section 6.2 of this document. Detailed results can be found in the Excel sheet test_a2_VTM-6.0_BIF_CCALF.xlsm.

Table 2 Performance of BIF in presence of CCALF

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_CCALF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | 0.25% | 0.22% | 105% | 107% |
| Class A2 | -0.40% | 0.18% | 0.06% | 106% | 107% |
| Class B | -0.34% | 0.26% | 0.19% | 106% | 104% |
| Class C | -0.53% | 0.10% | 0.20% | 105% | 99% |
| Class E | -0.44% | 0.20% | 0.15% | 102% | 103% |
| Overall | -0.40% | 0.20% | 0.17% | 105% | 104% |
| Class D | -0.47% | 0.02% | 0.07% | 109% | 106% |
| Class F | -0.55% | -0.01% | -0.01% | 105% | 105% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_CCALF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | 0.11% | -0.08% | 106% | 105% |
| Class A2 | -0.47% | 0.28% | 0.28% | 104% | 99% |
| Class B | -0.42% | 0.49% | 0.53% | 103% | 101% |
| Class C | -0.49% | 0.39% | 0.35% | 103% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.43% | 0.35% | 0.31% | 104% | 101% |
| Class D | -0.61% | 0.00% | 0.18% | 102% | 104% |
| Class F | -0.48% | -0.01% | 0.08% | 111% | 103% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_CCALF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.21% | 0.55% | 0.67% | 104% | 102% |
| Class C | -0.43% | 0.31% | 0.13% | 105% | 103% |
| Class E | -0.40% | -0.13% | 0.70% | 99% | 99% |
| Overall | -0.33% | 0.30% | 0.50% | 103% | 101% |
| Class D | -0.78% | 0.87% | 0.72% | 103% | 98% |
| Class F | -0.24% | 0.65% | 0.15% | 106% | 112% |

As can be seen when comparing the above table with Table 4.1 in JVET-P0073 (see Section 6.2 below), again the BDR figures hardly change: As an example, for the Random Access configuration, Table 2 above shows -0.43% (Y), 0.35% (U) and 0.31% (V) versus -0.44% (Y), 0.29% (U) and 0.23% (V).

For the sake of completeness, Table 3 shows the results of the tested software compared to VTM-6.0 without CCALF or BIF. Detailed results can be found in both test_a Excel sheets.

Table 3 Performance of combination of CCALF and BIF compared to VTM-6.0

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -3.25% | -3.95% | 115% | 111% |
| Class A2 | -0.26% | -8.52% | -8.87% | 111% | 111% |
| Class B | -0.24% | -10.33% | -8.41% | 112% | 110% |
| Class C | -0.41% | -4.08% | -3.71% | 109% | 105% |
| Class E | -0.25% | -9.97% | -3.31% | 108% | 108% |
| Overall | -0.27% | -7.40% | -5.85% | 111% | 109% |
| Class D | -0.35% | -2.72% | -3.22% | 111% | 107% |
| Class F | -0.37% | -6.61% | -6.67% | 110% | 112% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -5.56% | -5.42% | 107% | 109% |
| Class A2 | -0.26% | -11.50% | -15.51% | 106% | 102% |
| Class B | -0.21% | -17.23% | -13.64% | 107% | 107% |
| Class C | -0.40% | -4.65% | -4.02% | 106% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.27% | -10.40% | -9.80% | 106% | 105% |
| Class D | -0.50% | -4.42% | -4.28% | 105% | 104% |
| Class F | -0.29% | -4.76% | -4.55% | 113% | 105% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -21.11% | -18.66% | 106% | 107% |
| Class C | -0.31% | -7.89% | -6.31% | 108% | 108% |
| Class E | -0.16% | -15.90% | -5.72% | 106% | 102% |
| Overall | -0.16% | -15.40% | -11.31% | 107% | 106% |
| Class D | -0.60% | -7.24% | -5.38% | 106% | 101% |
| Class F | -0.11% | -9.94% | -7.43% | 108% | 112% |

## Test b) CCALF with BIF and chroma QP offset

The structure of tables in test b) is corresponding to test a). Table 4 shows the performance of CCALF and the chroma QP adjustment in presence of the BIF. The reference results are taken from JVET-P0073 [3]. These results can be compared with Table 4.1 from JVET-P0080 [2]. For convenience, the table can also be found in section 6.3 of this document. Detailed results can be found in the Excel sheet test_b1_VTM-6.0_BIF_CCALF_cQPo.xlsm.

Table 4 Performance of CCALF with chroma QP adjustment in presence of BIF

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_BIF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.19% | 5.46% | 3.73% | 104% | 95% |
| Class A2 | -2.03% | -0.33% | -0.69% | 99% | 97% |
| Class B | -0.74% | -1.70% | 1.01% | 101% | 99% |
| Class C | -0.76% | 4.08% | 4.20% | 100% | 103% |
| Class E | -0.46% | -2.84% | 4.65% | 103% | 105% |
| Overall | -0.99% | 0.82% | 2.50% | 101% | 100% |
| Class D | -0.73% | 5.40% | 5.02% | 101% | 104% |
| Class F | -0.73% | -1.02% | -1.12% | 99% | 97% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_BIF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.80% | 5.35% | 4.20% | 100% | 103% |
| Class A2 | -2.05% | -1.04% | -6.12% | 100% | 104% |
| Class B | -0.52% | -6.52% | -2.26% | 102% | 104% |
| Class C | -0.75% | 5.70% | 6.52% | 102% | 106% |
| Class E |  |  |  |  |  |
| Overall | -1.15% | 0.21% | 0.60% | 101% | 104% |
| Class D | -0.65% | 6.13% | 6.28% | 103% | 107% |
| Class F | -0.90% | 2.18% | 2.33% | 107% | 109% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_BIF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.53% | -9.39% | -6.56% | 101% | 102% |
| Class C | -0.70% | 3.70% | 5.51% | 103% | 103% |
| Class E | -0.11% | -4.62% | 6.03% | 104% | 106% |
| Overall | -0.48% | -3.83% | 0.61% | 103% | 103% |
| Class D | -0.53% | 4.41% | 6.84% | 103% | 103% |
| Class F | -0.86% | -3.65% | -1.11% | 100% | 100% |

Again, taking Random Access as an example, Table 4 above shows -1.15% (Y), 0.21% (U) and 0.60% (V), whereas Table 4.1 from JVET-P0080 (see Section 6.3 below) reports -1.16% (Y), 0.24% (U) and 0.59% (V).

Furthermore, Table 5 shows that the BIF is not impacted by the presence of CCALF and the chroma QP offset. The reference results are VTM-6.0 with CCALF and the chroma QP adjustment, taken from JVET-P0080 [2]. These results can be compared with Table 4.1 from JVET-P0073 [3]. For convenience, the table can also be found in section 6.2 of this document. Detailed results can be found in the Excel sheet test_b2_VTM-6.0_BIF_CCALF_cQPo.xlsm. Note that run times are not accurate as the reference was not created on the same cluster.

Table 5 Performance of BIF in presence of CCALF and chroma QP adjustment

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_CCALF_cQPo |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | 0.33% | 0.19% | 78% | 84% |
| Class A2 | -0.39% | 0.14% | 0.16% | 79% | 86% |
| Class B | -0.34% | 0.27% | 0.32% | 81% | 86% |
| Class C | -0.53% | 0.18% | -0.05% | 110% | 104% |
| Class E | -0.43% | 0.26% | 0.10% | 108% | 110% |
| Overall | -0.40% | 0.24% | 0.15% | 90% | 93% |
| Class D | -0.44% | 0.14% | 0.22% | 113% | 88% |
| Class F | -0.55% | 0.01% | 0.05% | 84% | 84% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_CCALF_cQPo |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | 0.17% | 0.05% | 73% | 98% |
| Class A2 | -0.43% | 0.24% | 0.29% | 75% | 95% |
| Class B | -0.41% | 0.25% | 0.25% | 104% | 123% |
| Class C | -0.50% | 0.33% | 0.30% | 107% | 125% |
| Class E |  |  |  |  |  |
| Overall | -0.42% | 0.26% | 0.23% | 91% | 112% |
| Class D | -0.62% | 0.16% | 0.14% | 109% | 122% |
| Class F | -0.50% | 0.37% | 0.08% | 101% | 123% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_CCALF_cQPo |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.18% | 0.47% | 0.65% | 74% | 97% |
| Class C | -0.48% | 0.42% | 0.77% | 77% | 99% |
| Class E | -0.15% | 1.97% | -0.25% | 97% | 115% |
| Overall | -0.27% | 0.83% | 0.47% | 80% | 102% |
| Class D | -0.72% | 0.44% | 0.65% | 104% | 118% |
| Class F | -0.35% | -0.30% | -0.05% | 75% | 94% |

Comparing the above table with Table 4.1 in JVET-P0073 (see Section 6.2 below), again the BDR figures hardly change: As an example, for the Random Access configuration, Table 5 above shows -0.42% (Y), 0.26% (U) and 0.23% (V) versus -0.44% (Y), 0.29% (U) and 0.23% (V).

Also, the results of the test software compared to VTM-6.0 are included in Table 6. Detailed results can be found in both test_b Excel sheets.

Table 6 Performance of CCALF, chroma QP adjustment and BIF comapred to VTM-6.0

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.51% | 5.73% | 3.95% | 111% | 108% |
| Class A2 | -2.42% | -0.16% | -0.57% | 108% | 107% |
| Class B | -1.08% | -1.47% | 1.23% | 111% | 109% |
| Class C | -1.28% | 4.16% | 4.45% | 104% | 104% |
| Class E | -0.89% | -2.57% | 4.83% | 105% | 108% |
| Overall | -1.39% | 1.02% | 2.70% | 108% | 107% |
| Class D | -1.19% | 5.49% | 5.08% | 108% | 102% |
| Class F | -1.27% | -1.04% | -1.21% | 109% | 106% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.13% | 5.45% | 4.20% | 104% | 112% |
| Class A2 | -2.47% | -0.91% | -5.91% | 103% | 103% |
| Class B | -0.94% | -6.11% | -2.01% | 106% | 105% |
| Class C | -1.28% | 6.11% | 6.89% | 105% | 103% |
| Class E |  |  |  |  |  |
| Overall | -1.58% | 0.50% | 0.83% | 105% | 106% |
| Class D | -1.25% | 6.27% | 6.55% | 105% | 98% |
| Class F | -1.41% | 2.21% | 2.39% | 110% | 106% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.72% | -8.84% | -6.12% | 104% | 105% |
| Class C | -1.20% | 4.31% | 6.26% | 107% | 108% |
| Class E | -0.39% | -3.81% | 7.31% | 106% | 102% |
| Overall | -0.80% | -3.20% | 1.37% | 105% | 106% |
| Class D | -1.28% | 5.37% | 8.01% | 105% | 99% |
| Class F | -1.19% | -2.99% | -0.21% | 105% | 107% |

# Conclusion

From the results shown in Table 1 and Table 2 it can be concluded that the two investigated tools, CCALF and BIF, do not interfere with each other and provide, with very small differences, the same gains regardless of whether the other tool is enabled.

We would like to thank Sharp Labs of America for cross-checking these results.

# References

[1] C.-Y. Chen, A. Norkin. Description of Core Experiment 5 (CE5): Loop filtering. JVET-O2025, Gothenburg, Sweden, July 2019

[2] K. Misra et al. CE5-2.1, CE5-2.2: Cross Component Adaptive Loop Filter. JVET-P0080, Geneva, Switzerland, October 2019

[3] J. Ström et al. CE5-3.1: Combination of bilateral filter and SAO. JVET-P0073, Geneva, Switzerland, October 2019

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix – Tables from JVET-P0080 and JVET-P0073

## JVET-P0080 Table 3.1

Table taken from document JVET-P0080 version 1 [2].

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | -3.49% | -4.16% | 105% | 103% |
| Class A2 | 0.13% | -8.67% | -8.93% | 103% | 104% |
| Class B | 0.10% | -10.56% | -8.57% | 102% | 102% |
| Class C | 0.13% | -4.18% | -3.91% | 102% | 101% |
| Class E | 0.19% | -10.16% | -3.46% | 103% | 103% |
| Overall | 0.13% | -7.58% | -6.01% | 103% | 103% |
| Class D | 0.11% | -2.74% | -3.28% | 101% | 103% |
| Class F | 0.18% | -6.58% | -6.64% | 102% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | -5.65% | -5.34% | 100% | 99% |
| Class A2 | 0.21% | -11.72% | -15.74% | 100% | 101% |
| Class B | 0.21% | -17.63% | -14.08% | 102% | 100% |
| Class C | 0.09% | -5.03% | -4.35% | 103% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | -10.69% | -10.07% | 101% | 100% |
| Class D | 0.11% | -4.45% | -4.46% | 100% | 100% |
| Class F | 0.18% | -4.73% | -4.61% | 102% | 98% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.18% | -21.52% | -19.23% | 102% | 100% |
| Class C | 0.12% | -8.18% | -6.44% | 101% | 100% |
| Class E | 0.24% | -15.69% | -6.37% | 100% | 97% |
| Overall | 0.17% | -15.62% | -11.75% | 101% | 99% |
| Class D | 0.18% | -8.10% | -6.04% | 103% | 102% |
| Class F | 0.13% | -10.52% | -7.54% | 104% | 100% |

## JVET-P0073 Table 4.1

Table taken from document JVET-P0073 version 1 [3].

All intra:

|  | Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | 0.27% | 0.22% | 105% | 105% |
| Class A2 | -0.39% | 0.18% | 0.12% | 108% | 103% |
| Class B | -0.34% | 0.24% | 0.23% | 108% | 105% |
| Class C | -0.53% | 0.07% | 0.23% | 104% | 102% |
| Class E | -0.44% | 0.26% | 0.16% | 103% | 103% |
| Overall | -0.40% | 0.20% | 0.20% | 106% | 104% |
| Class D | -0.47% | 0.08% | 0.06% | 104% | 102% |
| Class F | -0.55% | -0.03% | -0.07% | 106% | 104% |

Random access:

|  | Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | 0.11% | 0.00% | 102% | 107% |
| Class A2 | -0.42% | 0.13% | 0.21% | 102% | 99% |
| Class B | -0.43% | 0.43% | 0.28% | 104% | 102% |
| Class C | -0.53% | 0.38% | 0.35% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.44% | 0.29% | 0.23% | 103% | 102% |
| Class D | -0.61% | 0.12% | 0.26% | 103% | 101% |
| Class F | -0.51% | 0.05% | 0.06% | 104% | 101% |

Low-delay B:

|  | Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.19% | 0.62% | 0.46% | 103% | 102% |
| Class C | -0.50% | 0.63% | 0.68% | 101% | 102% |
| Class E | -0.28% | 0.81% | 1.15% | 102% | 102% |
| Overall | -0.32% | 0.67% | 0.71% | 102% | 102% |
| Class D | -0.75% | 0.93% | 1.15% | 103% | 100% |
| Class F | -0.33% | 0.65% | 0.86% | 103% | 100% |

## JVET-P0080 Table 4.1

Table taken from document JVET-P0080 version 1 [2].

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.19% | 5.37% | 3.75% | 102% | 102% |
| Class A2 | -2.04% | -0.29% | -0.74% | 100% | 101% |
| Class B | -0.74% | -1.74% | 0.92% | 101% | 101% |
| Class C | -0.76% | 3.98% | 4.51% | 99% | 101% |
| Class E | -0.46% | -2.81% | 4.71% | 101% | 99% |
| Overall | -0.99% | 0.78% | 2.54% | 101% | 101% |
| Class D | -0.75% | 5.36% | 4.85% | 100% | 102% |
| Class F | -0.73% | -1.03% | -1.29% | 100% | 98% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.82% | 5.25% | 4.13% | 99% | 101% |
| Class A2 | -2.05% | -1.14% | -6.18% | 97% | 102% |
| Class B | -0.54% | -6.34% | -2.25% | 100% | 99% |
| Class C | -0.78% | 5.75% | 6.57% | 101% | 97% |
| Class E |  |  |  |  |  |
| Overall | -1.16% | 0.24% | 0.59% | 99% | 100% |
| Class D | -0.64% | 6.08% | 6.42% | 98% | 97% |
| Class F | -0.92% | 1.86% | 2.34% | 100% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.54% | -9.26% | -6.60% | 102% | 101% |
| Class C | -0.73% | 3.86% | 5.44% | 100% | 99% |
| Class E | -0.24% | -5.62% | 7.60% | 102% | 98% |
| Overall | -0.53% | -3.98% | 0.96% | 101% | 100% |
| Class D | -0.56% | 4.90% | 7.37% | 105% | 104% |
| Class F | -0.84% | -2.82% | -0.28% | 104% | 101% |