| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0301 |
| --- | --- |

| Title: | Non-CE6: Decoding process of implicit TU partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geonjung Ko, Dongcheol Kim, Jason Jung, Ju-Hyung (John) Son, Jin Sam Kwak | Tel: Email: | +82-31-712-0523 greg.ko@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to change the decoding process related to the implicit TU partitioning. The configurable maximum transform size has been adopted and the transform tree syntax related to the implicit TU partitioning has changed accordingly. Meanwhile, the corresponding decoding process has remained unchanged. Therefore, there exists a case that the processing order of partitioned transform blocks is different in the syntax and the decoding process. It is proposed to make a change on the decoding process of the implicit TU partitioning to follow the syntax. The proposed change has no impact on VTM.

# Problem Statement

VVC has adopted the maximum transform size signalling [1]. According to the adoption, there was a syntax change related to the implicit TU partitioning and Table 1 shows the current syntax. When tbWidth or tbHeight of the current block is greater than MaxTbSizeY, it is partitioned into two blocks and the partitioning direction is decided based on verSplitFirst flag. The partitioning is recursively repeated until the block width and height are equal to or less than MaxTbSizeY.

Table 1. Transform tree syntax

| transform_tree( x0, y0, tbWidth, tbHeight , treeType, chType ) { | Descriptor |
| --- | --- |
| InferTuCbfLuma = 1 |  |
| if( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !cu_sbt_flag ) { |  |
| if( tbWidth > MaxTbSizeY ¦ ¦ tbHeight > MaxTbSizeY ) { |  |
| verSplitFirst = ( tbWidth > MaxTbSizeY && tbWidth > tbHeight ) ? 1 : 0 |  |
| trafoWidth = verSplitFirst ? (tbWidth / 2) : tbWidth |  |
| trafoHeight = !verSplitFirst ? (tbHeight / 2) : tbHeight |  |
| transform_tree( x0, y0, trafoWidth, trafoHeight, chType ) |  |
| if( verSplitFirst ) |  |
| transform_tree( x0 + trafoWidth, y0, trafoWidth, trafoHeight, treeType, chType ) |  |
| else |  |
| transform_tree( x0, y0 + trafoHeight, trafoWidth, trafoHeight, treeType, chType ) |  |
| } else { |  |
| transform_unit( x0, y0, tbWidth, tbHeight, treeType, 0, chType ) |  |
| } |  |
| } // … |  |

While the syntax has changed as above, the corresponding decoding process has remained unchanged. Table 2 shows the decoding process for the implicit TU partitioning of intra blocks. In the decoding process when the block width or height is greater than the maximum transform size (maxTbWidth for width and maxTbHeight for height), the block is partitioned vertically and/or horizontally resulting in two or four partitioned blocks. TU partitioning continues recursively until both width and height are equal to or less than the maximum transform size.

Table 2. Decoding process for intra blocks

### 8.4.5 Decoding process for intra blocks #### 8.4.5.1 General decoding process for intra blocks Inputs to this process are: - a sample location ( xTb0, yTb0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, - a variable nTbW specifying the width of the current transform block, - a variable nTbH specifying the height of the current transform block, - a variable predModeIntra specifying the intra prediction mode, - a variable cIdx specifying the colour component of the current block. Output of this process is a modified reconstructed picture before in-loop filtering. The maximum transform block width maxTbWidth and height maxTbHeight are derived as follows: maxTbWidth = ( cIdx = = 0 ) ? MaxTbSizeY : MaxTbSizeY / SubWidthC (8-41) maxTbHeight = ( cIdx = = 0 ) ? MaxTbSizeY : MaxTbSizeY / SubHeightC (8-42) The luma sample location is derived as follows: ( xTbY, yTbY ) = ( cIdx = = 0 ) ? ( xTb0, yTb0 ) : ( xTb0 * SubWidthC, yTb0 * SubHeightC ) (8-43) Depending on maxTbSize, the following applies: - If IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT and nTbW is greater than maxTbWidth or nTbH is greater than maxTbHeight, the following ordered steps apply. - The variables newTbW and newTbH are derived as follows: newTbW = ( nTbW > maxTbWidth ) ? ( nTbW / 2 ) : nTbW (8-44) newTbH = ( nTbH > maxTbHeight ) ? ( nTbH / 2 ) : nTbH (8-45) - The general decoding process for intra blocks as specified in this clause is invoked with the location ( xTb0, yTb0 ), the transform block width nTbW set equal to newTbW and the height nTbH set equal to newTbH, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before in-loop filtering. - If nTbW is greater than maxTbWidth, the general decoding process for intra blocks as specified in this clause is invoked with the location ( xTb0, yTb0 ) set equal to ( xTb0 + newTbW, yTb0 ), the transform block width nTbW set equal to newTbW and the height nTbH set equal to newTbH, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before in-loop filtering. - If nTbH is greater than maxTbHeight, the general decoding process for intra blocks as specified in this clause is invoked with the location ( xTb0, yTb0 ) set equal to ( xTb0, yTb0 + newTbH ), the transform block width nTbW set equal to newTbW and the height nTbH set equal to newTbH, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before in-loop filtering. - If nTbW is greater than maxTbWidth and nTbH is greater than maxTbHeight, the general decoding process for intra blocks as specified in this clause is invoked with the location ( xTb0, yTb0 ) set equal to ( xTb0 + newTbW, yTb0 + newTbH ), the transform block width nTbW set equal to newTbW and the height nTbH set equal to newTbH, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before in-loop filtering.

The difference between the syntax in Table 1 and the decoding process in Table 2 introduces a case of the different partitioning order. For example, if MaxTbSizeY is equal to 32, the block width is equal to 128 and the block height is equal to 64, then based on the syntax, (1) the 128x64 block is vertically partitioned first into two 64x64 blocks, (2) each partitioned block is horizontally partitioned resulting in four 64x32 blocks, and (3) blocks are vertically partitioned as shown in Figure 1 (a). Numbers inside the block in Figure 1 represent the block processing order.

However, following the decoding process, the resulting block processing order is different with that in the syntax as shown in Figure 1 (b). Based on the decoding process, (i) the 128x64 block is partitioned into four 64x32 blocks, since both width and height are greater than the maximum transform size. (ii) Then 64x32 blocks are partitioned again. The first partitioning (step (i)) results in the different processing order.

Figure 1. Implicit TU partitioning when MaxTbSizeY = 32 in (a) syntax and (b) decoding process

# Proposed specification change

It is proposed to change the implicit TU partitioning decoding process to follow the syntax. Table 3 shows the proposed text change for intra blocks. Please note that the same change is required for the decoding process for the residual signal as in the separate document.

Table 3. Proposed decoding process for intra blocks

### 8.4.5 Decoding process for intra blocks #### 8.4.5.1 General decoding process for intra blocks Inputs to this process are: - a sample location ( xTb0, yTb0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, - a variable nTbW specifying the width of the current transform block, - a variable nTbH specifying the height of the current transform block, - a variable predModeIntra specifying the intra prediction mode, - a variable cIdx specifying the colour component of the current block. Output of this process is a modified reconstructed picture before in-loop filtering. The maximum transform block width maxTbWidth and height maxTbHeight are derived as follows: maxTbWidth = ( cIdx = = 0 ) ? MaxTbSizeY : MaxTbSizeY / SubWidthC (8-41) maxTbHeight = ( cIdx = = 0 ) ? MaxTbSizeY : MaxTbSizeY / SubHeightC (8-42) The luma sample location is derived as follows: ( xTbY, yTbY ) = ( cIdx = = 0 ) ? ( xTb0, yTb0 ) : ( xTb0 * SubWidthC, yTb0 * SubHeightC ) (8-43) Depending on maxTbSize, the following applies: - If IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT and nTbW is greater than maxTbWidth or nTbH is greater than maxTbHeight, the following ordered steps apply. - The variables newTbW and newTbH are derived as follows: verSplitFirst = ( nTbW > maxTbWidth && ( cIdx = = 0 ? nTbW : nTbW*SubWidthC ) > ( cIdx = = 0 ? nTbH : nTbH*SubHeightC ) ) ? 1 : 0 newTbW = ( nTbW > maxTbWidth ) verSplitFirst ? ( nTbW / 2 ) : nTbW (8-44) newTbH = ( nTbH > maxTbHeight ) !verSplitFirst ? ( nTbH / 2 ) : nTbH (8-45) - The general decoding process for intra blocks as specified in this clause is invoked with the location ( xTb0, yTb0 ), the transform block width nTbW set equal to newTbW and the height nTbH set equal to newTbH, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before in-loop filtering. - If nTbW is greater than maxTbWidth If verSplitFirst is equal to 1, the general decoding process for intra blocks as specified in this clause is invoked with the location ( xTb0, yTb0 ) set equal to ( xTb0 + newTbW, yTb0 ), the transform block width nTbW set equal to newTbW and the height nTbH set equal to newTbH, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before in-loop filtering. - If nTbH is greater than maxTbHeight Otherwise, the general decoding process for intra blocks as specified in this clause is invoked with the location ( xTb0, yTb0 ) set equal to ( xTb0, yTb0 + newTbH ), the transform block width nTbW set equal to newTbW and the height nTbH set equal to newTbH, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before in-loop filtering. - If nTbW is greater than maxTbWidth and nTbH is greater than maxTbHeight, the general decoding process for intra blocks as specified in this clause is invoked with the location ( xTb0, yTb0 ) set equal to ( xTb0 + newTbW, yTb0 + newTbH ), the transform block width nTbW set equal to newTbW and the height nTbH set equal to newTbH, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before in-loop filtering.

# Conclusion

This contribution proposes to change the decoding process on the implicit TU partitioning to resolve the difference of TU block processing order between the syntax and the decoding process. The proposal has no impact on VTM. It is suggested to adopt the proposed specification change in the next version of VVC draft text.

# Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, 15th meeting, Gothenburg, SE, 3–12 July 2019.