| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0305-v2 |
| --- | --- |

| Title: | CE4-related: Unification of triangle partition mode and geometric merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, Jie Chen, Yan Ye, Jiancong Luo Radiance Jin Hui Tower, Building No.6, The 4 District Wangjing Dong Yuan, Chaoyang District, Beijing, China | Tel: Email: | ruling.lrl@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

In core experiment 4, the complexity and coding performance of geometric merge mode (GEO) are studied. The GEO shares the same concept as that of triangle partition mode (TPM), however, the design of these two modes are different from each other. In this contribution, it is proposed to unify the design of GEO and TPM including the blending process, motion field storage and syntax structure. Firstly, only one flag is signaled at CU level to indicate whether a block is split into two inter-predicted partitions. Secondly, the blending weights and the masks for motion field storage are derived from two sets of pre-defined masks, one set for blending weights and the other set for the motion field storage. It is reported that, under the CTC, the overall performance impact for {Y, U, V} is {-0.25%, -0.36%, -0.43%} for RA and {-0.59%, -0.67%, -0.49%} for LB.

Introduction

In VVC draft 6, triangle partition mode (TPM) is supported for inter prediction. A block is firstly split into two triangle-shape partitions, using either diagonal split or the anti-diagonal split. Then, each triangle partition is inter-predicted using its own uni-prediction motion vector. After predicting each of the triangle partition, the sample values along the diagonal or anti-diagonal edge are adjusted using a blending processing with adaptive weights. This is the prediction signal for the whole block, and transform and quantization process will be applied to the whole block as in other prediction modes.

In CE4, the complexity and coding performance of the geometric merge mode (GEO) are evaluated. The GEO mode shares the same concept as that of TPM while providing more flexible partitioning option for motion compensation. The prediction procedure of GEO is similar to that of TPM. However, the adaptive weights used in the blending process of GEO are calculated using different equations from those used in TPM. Besides, the motion field storage of GEO is different from that of TPM. The differences between these two modes are listed in the table below.

|  | TPM | GEO |
| --- | --- | --- |
| Restrictions on block size | Block size ≥ 64 samples | Block width ≥ 8 and block height ≥ 8 |
| Syntax structure | One flag to separate TPM and GEO mode at CU level |  |
| Interaction with SBT | SBT is not applied to TPM blocks | SBT is applied to GEO blocks |
| Weights for luma samples | Calculated using different equations |  |
| Weights for chroma samples | Calculated using equations | Subsampled from the weights of luma samples |
| Masks for motion field storage | Calculated using equations | Calculated using equations or derived from the weights of luma samples |

Since the designs of these two modes are different from each other, two different logics are required in real implementation. It is asserted that such differences increase the implementation complexity for the decoder design. Moreover, the memory required to store all the blending weights and the masks for motion field storage for GEO and TPM are huge. For TPM, the memory needed to store all this information is calculated as follows:

- For blending weights:4×16+4×32+4×64+8×8+8×16+8×32+8×64+16×4+16×8+16×16+16×32+16×64+32×4+32×8+32×16+32×32+32×64+64×4+64×8+64×16+64×32+64×64+64×128+128×64+128×128×2×4=384,512 bits=48,064 bytes≅48 Kbytes

- For motion field storage:1×4+1×8+1×16+2×2+2×4+2×8+2×16+4×1+4×2+4×4+4×8+4×16+8×1+8×2+8×4+8×8+8×16+16×1+16×2+16×4+16×8+16×16+16×32+32×16+32×32×2×2=12,016 bits=1,502 bytes≅1.5 Kbytes

For GEO, taking the 140-mode variation in CE4-1.1 as an example, the memory needed to store all this information is calculated as follows:

- For blending weights:8×8+8×16+8×32+8×64+16×8+16×16+16×32+16×64+32×8+32×16+32×32+32×64+64×8+64×16+64×32+64×64+64×128+128×64+128×128×140×4=26,414,080 bits=3,301,760 bytes≅3.3 Mbytes

- For motion field storage:2×2+2×4+2×8+2×16+4×2+4×4+4×8+4×16+8×2+8×4+8×8+8×16+16×2+16×4+16×8+16×16+16×32+32×16+32×32×140×2=825,440 bits=103,180 bytes≅103 Kbytes

Memory storage requirement for all 3 variations of GEO in CE4 and TPM is listed in the table below.

| 140 geometric partitioning submodes + triangle mode | 3,454,506 bytes |
| --- | --- |
| 108 geometric partitioning submodes + triangle mode | 2,676,234 bytes |
| 80 geometric partitioning submodes + triangle mode | 1,995,246 bytes |

Proposed methods

In this contribution, it is proposed to unify the design of GEO and TPM. For a block coded using the skip or merge mode, a flag (i.e. ciip_flag[][] equal to 0 in the table below) is signaled to indicate whether the block is split into two geometric partitions.

| merge_data( x0, y0, cbWidth, cbHeight, chType ) { | Descriptor |
| --- | --- |
| … |  |
| } else { |  |
| if( sps_ciip_enabled_flag && sps_triangle_enabled_flag && MaxNumTriangleMergeCand > 1 && slice_type = = B && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if( !ciip_flag[ x0 ][ y0 ] && MaxNumTriangleMergeCand > 1 ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_geo_split_idx[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |

When the flag is equal to false, the block is split into two geometric partitions using a signaled geometric partitioning index (i.e. merge_geo_split_idx[][] in the table above). The value of the geometric partitioning index ranges from 0 to Nmode-1, where Nmode is the total number of supported partitioning modes. Nmode is equal to Nφ × Nρ, where Nφ is the number of supported angles and Nρ is the number of supported steps. In this proposal, Nφ and Nρ are set to 8 and 7, respectively.

Figure 1. An illustration of 8 angles setting

Same as TPM in VVC, each geometric partition is inter-predicted using its own uni-prediction motion vector. After predicting each of the geometric partition, the sample values along the partitioning edge are adjusted using a blending process with adaptive weights. This is the prediction signal for the whole block, and transform and quantization process will be applied to the whole block as in other prediction modes. It is noted that the SBT mode is not applied to the block. Finally, the motion field of the block predicted using the geometric partition mode is stored in 4x4 subblocks.

Adaptive weights for luma samples

To reduce the memory needed for storing the blending weights, it is proposed to derive all the weights from a pre-defined set of masks, g_sampleWeightL[]. Assuming that NA represents the number of pre-defined masks in the set, and NA is set to (Nφ>>1) + 1. M×M represents the size of the pre-defined masks, and M is set to 128 + (ND × (128 >> ND)) << 1, where ND is set to (Nρ >>1). So for the 8 angles with 7 steps setting, NA is set to 5, ND is set to 3 and M is set to 224.

For the partitioning angle ranges from 0 to NA – 1, their masks are directly cropped from the pre-defined masks. On the other hands, the masks for the partitioning angle ranges from NA to Nφ – 1 are cropped from the pre-defined masks and flipped in vertical direction.

For a block of size W×H with geometric partitioning index K, the blending weights for luma samples are derived as follows:

- Variables angle φ and distance ρ are calculated as follows:

φ=K % Nφ

ρ=K / Nφ

- Variables offsetX and offsetY are calculated as follows:

offsetX=(M-W)≫1φ=NA-1 or (φ≠0 and H ≥ W)M-W≫1+ρ≤ND ?-(ρ×W≫ND) :(ρ-ND)×W≫NDotherwise

offsetY=M-H≫1+ρ≤ND ? -(ρ×H≫ND) :(ρ-ND)×H≫NDφ=NA-1 or (φ≠0 and H ≥ W)M-H≫1otherwise

- sampleWeightLxy=g_sampleWeightL[φ][x+offsetX][y+offsetY]φ< NAg_sampleWeightL[Nφ-φ][x+offsetX][H-1-y+offsetY]otherwise

An illustrations of blending weights derivation is shown in Figure 2.

Figure 2. An illustration of the blending weights derivation for a 16x8 block

Adaptive weights for chroma samples

The blending weights for chroma samples are obtained from the weight of top-left luma sample for each corresponding 2x2 luma subblock, which is the same as the method used in CE4-1.1 and CE4-1.2.

Masks for motion field storage

To reduce the memory needed for storing the masks for motion field storage, same as the blending process, all the information are derived from a pre-defined set of masks, g_motionMask[]. The mask for motion field storage is derived as follows:

- Variables offsetXmotion and offsetYmotion are calculated as follows:

offsetXmotion=((M≫2)-(W≫2))≫1φ=NA-1 or (φ≠0 and H ≥ W)((M≫2)-(W≫2))≫1+ρ≤ND ?-(ρ×W≫(ND+2)) :(ρ-ND)×W≫(ND+2)otherwise

offsetYmotion=(M≫2)-(H≫2)≫1+ρ≤ND ? -(ρ×H≫(ND+2)) :(ρ-ND)×H≫(ND+2)φ = NA-1 or (φ≠0 and H ≥ W)(M≫2)-(H≫2)≫1otherwise

- motionMaskxsubblkysubblk=g_motionMask[φ][xsubblk+offsetXmotion][ysubblk+offsetYmotion]φ< NAg_motionMask[Nφ-φ][xsubblk+offsetXmotion][H≫2-1-ysubblk+offsetYmotion]otherwise

Pre-defined masks generation

It is noted that the pre-defined masks are calculated using the similar equations in CE4-1.14. More specifically, the following equations are used to calculate the pre-defined masks g_sampleWeightL[] with φ ranges from 0 to NA − 1.

displacementX=φ

displacementY=displancementX+(16≫2) % 16

rho=DisdisplacementX≪8+DisdisplacementY≪8

offset=1≪8-M≫1

offsetMotion=1≪6-(M≫2)≫1

distFromLine=(x+offset)≪1+1×DisdisplacementX+(y+offset)≪1+1×DisdisplacementY-rho

distScaled=Clip3(0, 26,absdistFromLine+4≫3)

g_sampleWeightL[φ]xy=distFromLine≤0 ?GeoFilterdistScaled:8-GeoFilterdistScaled

rhosubblk=3×DisdisplacementX+3×Dis[displacementY]

distFromLinesubblk=(xsubblk+offsetMotion)≪3+1×DisdisplacementX+(ysubblk+offsetMotion)≪3+1×DisdisplacementY-rho+rhosubblk

g_motionMask[φ]xsubblkysubblk=abs(distFromLinesubblk)<256 ? 2 :(distFromLinesubblk≤0 ?0 :1)

, where Dis[] and GeoFilter[] are two look-up tables as shown in Table 3 and Table 4, respectively.

Table 3. The look-up table for Dis[]

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dis[idx] | 64 | 59 | 45 | 24 | 0 | -24 | -45 | -59 |
| idx | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| Dis[idx] | -64 | -59 | -45 | -24 | 0 | 24 | 45 | 59 |

Table 4. The look-up table for GeoFilter[]

| Idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GeoFilter[idx] | 4 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 6 | 6 | 6 |
| Idx | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |  |
| GeoFilter[idx] | 6 | 6 | 6 | 6 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 8 |  |

Memory usage

With the proposed method, the memory needed for the blending weights and the mask for motion field storage can be reduced by 93-96%. The number of bits needed to store the pre-defined masks are listed as follows:

- For blending weights:M×M×NA×4

- For motion field storage:M×M/16×NA×2

|  | 8 angles + 7 steps |
| --- | --- |
| Memory of the proposed method | 129,360 bytes |
| % of reduction compared to 140 geometric partitioning submodes + triangle mode | 96.3% |
| % of reduction compared to 108 geometric partitioning submodes + triangle mode | 95.2% |
| % of reduction compared to 80 geometric partitioning submodes + triangle mode | 93.5% |

Simulation results

The proposed unification methods are implemented on top of VTM-6.0. In this proposal, the number of angles and the number of steps are set to 8 and 7, respectively.

Table 5. Results of unification of triangle partition mode and geometric merge mode

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.08% | -0.19% | 101% | 100% |
| Class A2 | -0.14% | -0.11% | -0.12% | 101% | 100% |
| Class B | -0.14% | -0.14% | -0.26% | 101% | 99% |
| Class C | -0.58% | -1.03% | -1.06% | 101% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.25% | -0.36% | -0.43% | 101% | 99% |
| Class D | -0.44% | -0.90% | -0.76% | 102% | 100% |
| Class F | -0.30% | -0.41% | -0.36% | 100% | 99% |

|  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.25% | -0.30% | -0.44% | 102% | 100% |
| Class C | -0.70% | -0.97% | -0.94% | 103% | 99% |
| Class E | -1.03% | -0.87% | 0.03% | 100% | 99% |
| Overall | -0.59% | -0.67% | -0.49% | 102% | 99% |
| Class D | -0.66% | -1.23% | -0.58% | 103% | 97% |
| Class F | -0.43% | -0.64% | 0.40% | 102% | 99% |

Comparison between proposals

The differences between CE4-1.14 [1], JVET-P0304 [2] and this proposal are listed in Table 6. It is noted that the design of TPM in CE4-1.14 and JVET-P0304 is not shown in the table since the designs are the same as that in VVC draft 6. As shown in the table, the design of this proposal is unified. Besides, the memory usage of this proposal is the least one among three proposals.

Table 6. Comparison of P0068 (CE4-1.14), P0304 and P0305

|  | CE4-1.14 108 modes | JVET-P0304 108 modes | This proposal |
| --- | --- | --- | --- |
| Are TPM and GEO two different modes? | Yes |  | No, two modes are combined into one mode |
| Is TPM same as VVC draft 6 | Yes |  | No TPM mode |
| Partitioning modes | 2 + 108 (2 for TPM and 108 for GEO) |  | 56 |
| Supported angles and steps in GEO | 16 angles + 7 steps (equivalent to 32 angles + 4 steps) |  | 8 angles + 7 steps (equivalent to 16 angles + 4 steps) |
| Syntax structure | Two flags (one flag to indicate whether TPM or GEO is used or not; the other flag to indicate which one of TPM and GEO is used) |  | One flag (one flag to indicate whether the combined mode is used or not) |
| Restrictions on block size | TPM: block size ≥ 64 samples GEO: block width ≥ 8 and block height ≥ 8 |  | Block size ≥ 64 samples |
| Is SBT applied? | TPM: No GEO: Yes |  | No |
| Weights for luma samples in GEO | Calculated by equations | Cropped from the pre-defined masks | Cropped and flipped from the pre-defined masks |
| Number of pre-defined masks | N/A | 16 | 5 |
| Weights for chroma samples in GEO | Subsampled from the weights of luma samples |  |  |
| Masks for motion field storage in GEO | Calculated by equations | Cropped from the pre-defined masks | Cropped and flipped from the pre-defined masks |
| Overall memory usage for GEO and TPM | 2,676,234 bytes | 463,518 bytes | 129,360 bytes |
| Y coding gain for {RA, LB} | {-0.34%, -0.73%} | {-0.33%, -0.63%} | {-0.25, -0.59%} |

Conclusion

In this contribution, it is proposed to unify the design of GEO and TPM including syntax structure, blending process and motion field storage. With the proposed method, 0.25% and 0.59% gain can be achieved in random access and low delay B conditions. Besides, the memory needed to store the blending weights and the masks for motion field storage can be reduced by 93-96%. Considering the trade-off between complexity and coding performance, we recommend to adopt this contribution to VVC.

References

H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, M. Bläser and J. Sauer, “CE4: CE4-1.1, CE4-1.2 and CE4-1.14: Geometric Merge Mode (GEO)”, JVET-P0068, Oct. 2019.

R.-L. Liao, J. Chen, Y. Ye and J. Luo, “CE4-related: Simplification of blending weights and motion field storage in geometric merge mode”, JVET-P0304, Oct. 2019.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).