| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0320-v1 |
| --- | --- |

| Title: | Non-CE4/AHG17: On slice-level syntax for BDOF and DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Filippov, V. Rufitskiy | Email: | alexey.filippov@huawei.com vasily.rufitskiy@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

In this proposal, a slice level syntax for BDOF and DMVR is defined to avoid signaling the relevant slice-level for slices that are not of type B.

# Introduction

From [1], it is known that bi-directional optical flow (BDOF) and decoder side motion vector refinement (DVMR) could operate on a bi-predicted coding blocks, i.e. belonging to a PUs for which two reference blocks are specified. When a PU belongs to a slice of a type that disallows bi-predictive coding (i.e. slice_type == B), a slice header flag that enables or disables BDOF or DMVR could be skipped and inferred to zero, since it is guaranteed by the slice type that within the slice there will be no bi-predicted blocks.

According to [1], slice_disable_bdof_dmvr_flag equal to 1 specifies that neither of bi-directional optical flow inter prediction and decoder motion vector refinement based inter bi-prediction is enabled in the current slice. slice_disable_bdof_dmvr_flag equal to 0 specifies that bi-directional optical flow inter prediction or decoder motion vector refinement based inter bi-prediction may or may not be enabled in the current slice. When slice_disable_bdof_dmvr_flag is not present, the value of slice_disable_bdof_dmvr_flag is inferred to be 0.

# Proposed method

Hence, it is proposed to skip signaling of the syntax element slice_disable_bdof_dmvr_flag for slices that are not of type B (see Table 1).

Table 1. Proposed modification to slice-level signaling.

| if(slice_type = = B && sps_bdof_dmvr_slice_present_flag ) |  |
| --- | --- |
| slice_disable_bdof_dmvr_flag | u(1) |

Taking into account the proposal described in JVET-P0275, where the syntax element max_num_merge_cand_minus_max_num_triangle_cand is also skipped for slices that are not of type B, the combined modifications with those changes will be as shown in Table 2.

Table 2. Proposed modification to slice-level signaling.

| if( slice_type = = B ) { |  |
| --- | --- |
| if( sps_bdof_dmvr_slice_present_flag ) |  |
| slice_disable_bdof_dmvr_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 ) |  |
| max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| } | ue(v) |

# Experimental results

## The simulation results over VTM-6.0 are reportedly presented in Table 3 for LDP configuration. In the simulations as compared to the JVET common test conditions [2], SPS flags for BDOF and DMVR are on in cfg-files for LDP configuration.

Table 3. Simulation results for the proposed modification

|  | Low delay P |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 with BDOF and DMVR sps flags |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.00% | 0.00% | 0.00% | 97% | 98% |
| Class E | -0.01% | -0.01% | -0.01% | 98% | 98% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.01% | -0.01% | -0.01% | 98% | 98% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

# Conclusion

It is proposed to adopt the suggested modification to the VVC specification draft and to the VTM Software.

# References

- B. Bross, J. Chen, S. Liu., Versatile Video Coding (Draft 6), “Versatile Video Coding (Draft 6)”, JVET-O2001, 15th JVET Meeting, Gothenburg, Sweden, July 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th JVET Meeting, Geneva, Switzerland, March 2019.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).